3GPP TSG-RAN5 Meeting #97	R5-226617

# Toulouse, France, 14 – 18 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3288 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC 12.1.1.2- PC5-only operation / Sidelink communication / Reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The synchronisation source should be clarified for UE and NR-SS-UE 2. Update the conditions as per the update of TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarified the synchronisation source for UE and NR-SS-UE 2. Added Tx/Rx condition as per the update of TS 38.508-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### 12.1.1.2	PC5-only operation / Sidelink communication / Reception

12.1.1.2.1	Test Purpose (TP)

(1)

with { UE being authorized for performing NR sidelink Communication }

ensure that {

when { UE is configured by upper layer to perform NR sidelink reception }

then { UE is able to monitor NR sidelink transmission using sl-RxPool included in pre-configuration }

}

12.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.7. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.7]

A UE capable of NR sidelink communication that is configured by upper layers to receive NR sidelink communication shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in RRCReconfiguration message or sl-FreqInfoList included in SIB12:

3>	if the UE is configured with sl-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated by sl-RxPool;

3>	else if the cell chosen for NR sidelink communication provides SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated by sl-RxPool in SIB12;

2>	else:

3>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources that were preconfigured by sl-RxPool in SL-PreconfigurationNR, as defined in sub-clause 9.3;

12.1.1.2.3	Test description

12.1.1.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

- 	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-  	The UE uses GNSS as the synchronization reference source.

-	The UE is equipped with below information in UE or in a USIM containing default values (as per TS 38.508-1 [4] clause 4.8.3.3.3).

Table 12.1.1.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [4], table 4.10.1-1, except SL-BWP-PoolConfigCommon field as defined in table 12.1.1.2.3.3-1 |  |

Preamble:

-	The UE is in state 0-A as defined in TS 38.508-1 [4].

12.1.1.2.3.2	Test procedure sequence

Table 12.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). | - | - | - | - |
| 3 | The NR-SS-UE1 transmits a DIRECT LINK ESTABLISHMENT REQUEST message using the pool of resources that were preconfigured by sl-RxPool in SL-PreconfigurationNR. | <-- | PC5-S: DIRECT LINK ESTABLISHMENT REQUEST | - | - |
| 4 | Check: Does the UE transmit a DIRECT LINK SECURITY MODE COMMAND message? | --> | PC5-S: DIRECT LINK SECURITY MODE COMMAND | 1 | P |
| 5 | The NR-SS-UE1 transmits a DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: DIRECT LINK SECURITY MODE COMPLETE | - | - |
| 6 | The UE transmits a DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: DIRECT LINK ESTABLISHMENT ACCEPT | - | - |
| 7 | The NR-SS-UE1 transmits an RRCReconfigurationSidelink message. | <-- | PC5-RRC: RRCReconfigurationSidelink | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationSidelinkComplete message? | --> | PC5-RRC: RRCReconfigurationSidelinkComplete | 1 | P |

12.1.1.2.3.3	Specific message contents

Table 12.1.1.2.3.3-1: SL-BWP-PoolConfigCommon (Preamble)

Derivation Path: TS 38.508-1 [4], Table 4.6.6-4 with condition RXPOOL and SELECTED

<End of Changed Section>