3GPP TSG-RAN5 Meeting #97                                                  	                    R5-226643

# Toulouse, France, 14th - 18th November 2022

Agenda item:	5.4.17

Source: 	Qualcomm Inc

Title: 	PC5 device assumptions

Document for:	Discussion and Endorsement

## 1.	Introduction

TS 38.101-2 [1] has defined FWA UE type for Power class 5 and associated core requirements. This contribution proposes device and antenna assumptions for FWA devices supporting PC5.

## 2.	Discussion

Below are the UE type assumption along with Power class assumption as defined in table 6.2.1.0-1 of [1].

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

A FWA device is expected to support PC1 and/or PC5. The device and antenna assumptions for FWA supporting PC1 is already captured in [2].

In this paper we propose to keep the PC5 FWA device and antenna assumption same as PC1 FWA. Proposals are captured in Table 2.1

Table 2.1: PC5 FWA device and antenna assumptions

| Number of antenna arrays | 1 |
| --- | --- |
| Number of elements in array | 12 x 12 |
| Max DUT size | 40cm |
| Max DUT radiating aperture | 10.6 cm (diagonal), from l/2 separated elements at 24G, 12 elements per dimension |
| Power supply | AC power supply |
| Max weight of UE | 10 lbs (4.5 Kg) |
| Beam steering range and granularity in xz plane | 4o beam steering granularity (from 30o to 150o) |
| Beam steering range and granularity in xy plane | 4o beam steering granularity (from 30o to 150o) |

## 3.	Conclusion

Proposal1:  Endorse PC5 FWA device and antenna assumptions as listed in Table 2.1.

## 4.		References

[1] TS 38.101-2 v17.7.0

[2] R5-198203, MU Assumptions for PC1 Fixed Wireless Access (FWA) UE, Qualcomm