3GPP TSG-RAN5 Meeting #97	R5-226658

# Electronic Meeting, 14th Nov 2022 – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2624 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to connection diagram for FR2 1x2 channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Israel Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The connection diagrams present in 38.508-1 incorrectly assume that a 1x2 channel without fading is equivalent to a 1x1 conducted channel from test equipment perspective. RAN4 test requirements have been defined for a 1x2 channel in static AWGN scenario. The RAN4 assumption is that AWGN is applied independently per branch with the Es/Noc values assumed per UE receiver antenna port. While coming up with RAN4 RRM requirements, FR2 UE is assumed to make use of diversity combining. For the UE to make use of diversity combining in these FR2 RRM test cases, TE needs to ensure signal is split into 2 branches and AWGN independently added to each of them before being transmitted via H and V polarizations, resepectively. In the case where AWGN level is similar or close to signal power, the uncorrelated AWGN will provide sufficient diversity gain for the UE. In the case where AWGN is not added or Signal power is quite high compared to AWGN power, uncorrelated AWGN will no longer provide any meaningful diversity gain. One option to address this is to use a phase rotator to distinguish the 2 signal paths. This approach is similar to what is currently done for FR1 tests wherein a phase rotator of 5*n Hz is introduced to decorrelate the signals and allow diversity combining. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated section A.3.3.3.1 - Updated connection diagram A.3.3.3.1-1 - Added connection diagram A.3.3.3.1-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## A.3.3	Test Equipment Parts for Radiated Measurements

### A.3.3.1	Transmitter/Receiver tests

The Test Equipment part is focused on logical representation of TE measurement and link antenna(s) and positioner controller. The Test Equipment connection diagram below is applicable for NR radiated RX and TX tests, including CA and UL MIMO tests.

Figure A.3.3.1.1: Basic TE diagram for radiated RX and TX tests

For NR radiated RX tests requiring to simulate a modulated interference, connection diagram defined in figure A.3.3.1.2 will apply.

Figure A.3.3.1.2: TE diagram for radiated RX tests with Modulated Interference

### A.3.3.2	Demodulation and CSI tests

Figure A.3.3.2.1: Demodulation and CSI tests

Figures A.3.3.2.1-1 and A.3.3.2.1-1 show the connection diagram inside SS NR of Figure A.3.3.2.1 for downlink signal path.

Figure A.3.3.2.1-1: TE diagram for Demodulation and CSI tests (1x2)

Figure A.3.3.2.1-2: TE diagram for Demodulation and CSI tests (2x2)

### A.3.3.3	RRM tests

The Test Equipment part is focused on logical representation of TE antenna(s) and positioner. The Test Equipment connection diagram below is applicable for NR radiated RRM tests. SS NR uses several antennas to cover all required AoA offsets. The actual number of antennas is not determined and depends on the TE implementation. Positioner in the TE part is optional.

Figure A.3.3.3.1: TE diagram for radiated RRM tests

Figure A.3.3.3.1-1 shows the connection diagram inside the SS NR of Figure A.3.3.3.1 for downlink signal path when only AWGN is configured. Figure A.3.3.3.1-2 shows the connection diagram inside the SS NR of Figure A.3.3.3.1 for downlink signal path when fading is also configured. In the connection diagrams, the parameter n determines the number of NR cells present and shall be determined by the test cases. The value of the phase rotator is φn = 5*n Hz, which is a small offset to ensure there is a linearly independent version of the signal on each of the split branches.

Figure A.3.3.3.1-1: TE diagram for 1x2 radiated RRM tests without fading

Figure A.3.3.3.1-2: TE diagram for 1x2 radiated RRM tests with fading

<< End of Changes >>