3GPP TSG-RAN5 Meeting #97	R5-226673

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0602 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR-U PDSCH Demod test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Israel Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR-U WP requires addition of PDSCH Demod test case on band with shared spectrum access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case contents for 2Rx and 4Rx FR1 TDD PDSCH on band with shared spectrum access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.9 (new), 5.2.2.2.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.1.8	Applicability of different requirements with Multi-TRxP

The applicability rules for requirements with multi-TRxP transmission schemes in section 5 are specified in Table 5.1.1.8-1.

Table 5.1.1.8-1: Applicability of requirements with Multi-TRxP Transmission

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.2.1.6 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.11 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.13 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.2.2.6 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.2.2.14 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.11 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.12 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.13 (Test 1-1) |  |
| FR1 FDD | PDSCH | Clause 5.2.3.1.6 (Test 1-1) | FR1 FDD | PDSCH | Clause 5.2.3.1.14 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.11 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.12 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.13 (Test 1-1) |  |
| FR1 TDD | PDSCH | Clause 5.2.3.2.6 (Test 1-1) | FR1 TDD | PDSCH | Clause 5.2.3.2.14 (Test 1-1) |  |

#### 5.1.1.9	Applicability of requirements for PDSCH on bands with shared spectrum access

| Tests | Applicability notes |
| --- | --- |
| All tests in Clause 5.2.2.2.15 and 5.2.3.2.15 | Only test the supported largest channel bandwidth. |
| All tests in Clause 5.2A.2.3 and 5.2A.3.3 | Only test the supported largest channel bandwidth on SCell. |

<several sections skipped>

5.2.2.2.14_1.4	Test requirement

Table 5.2.2.2.14.0-3 defines the primary level settings.

The residual BLER specified in Note 3 of Table 5.2.2.2.14_1.4-1 test shall meet or be lower than the specified value in Table 5.2.2.2.14_1.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.14_1.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and antenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR (dB) (Note 4) |
| 1-1 | R.PDSCH.2-16.2 TDD | 40 / 30 | 16QAM, 0.54 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 1 (Note 3) | 3.8 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent. Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2. Note 3: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. Note 4: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |

##### 5.2.2.2.15	2Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access

5.2.2.2.15.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.15-3, with the addition of test parameters in Table 5.2.2.2.15-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.15-1.

Table 5.2.2.2.15-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance for UE supporting operations in shared spectrum access | 1-1, 1-2, 1-3, 1-4 |

Table 5.2.2.2.15-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| DL transmission model |  |  | As specified in B.5 |
| Downlink Model Parameters | SSB Q factor |  | 8 |
|  | Downlink transmission duration values | Slots | {2,4,6,7} |
|  | Occupied OFDM symbols in slot other than the last slot of the downlink duration | Symbols | 14 |
|  | Occupied OFDM symbols in the last slot of the downlink duration | Symbols | {6,9,12,14} (Note 1) |
|  | Downlink period | ms | 5 |
|  | LBT failure probability (pLBT) |  | 0.25 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: If DL Transmission duration is 2 Slot, the occupied OFDM symbols in the last slot of the downlink duration is 14. |  |  |  |

Table 5.2.2.2.15-3: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 |
| 1-2 | R.PDSCH.2-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.1 |
| 1-3 | R.PDSCH.2-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.2 |
| 1-4 | R.PDSCH.2-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 |