3GPP TSG-RAN5 Meeting #97	R5-226683

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0284 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.3A.1 - MCPTT CO session establishment/modification without provisional responses other than 100 Trying |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Up to now the procedure considers only the cases for INVITE containing a media description for floor control but not the case when there is no media description for floor control at all. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Option c added to NOTE 1 covering the case of no media description for floor control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3A.1	MCPTT CO session establishment/modification without provisional responses other than 100 Trying

5.3A.1.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3A.1.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3A.1.3	Procedure

Table 5.3A.1.3-1: MCPTT CO session establishment/modification without provisional responses other than 100 Trying

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 1a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 2 | Check: Does the UE (MCPTT Client) send a SIP INVITE requesting the establishment/modification of an MCPTT call? | --> | SIP INVITE | - | P |
| 3 | The SS sends SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCPTT Client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| - | EXCEPTION: Steps 6a1 describes behaviour that depends on the test case requirements; the "lower case letter" identifies a step sequence that takes place if the UE requests implicit floor control in step 2 (i.e. the "mc_implicit_request" fmtp attribute included in the SDP offer and the SS responded with the "mc_implicit_request" fmtp attribute included and the “mc_granted” fmtp attribute not present in the SDP answer (NOTE1) | - | - | - | - |
| 6a1 | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| NOTE1: Possibilities in SDP-offer/answer depend on the test case requirements a. UE sends SDP offer with media description for floor control but without implicit floor request b. UE sends SDP offer with media description for floor control and with implicit floor request i. SDP answer from SS contains “mc_implicit_request” and “mc_granted” (Floor is implicitly granted) ii. SDP answer from SS contains “mc_implicit request” and but no “mc_granted” (Floor needs to be explicitly granted at step 6a1) iii. SDP answer from SS contains no “mc_implicit_request”and no “mc_granted” (the UE needs to explicitly request the floor) c. UE sends SDP offer without media description for floor control |  |  |  |  |  |

5.3A.1.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3A.1.4-1: SIP INVITE (step 2, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.5.1-1 with condition MCPTT

Table 5.3A.1.4-2: SIP 200 (OK) (step 4, Table 5.3A.1.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP and MCPTT