3GPP TSG-RAN WG5 Meeting # 97	R5-226716

# Toulouse, France, 14th – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2636 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE DownlinkConfigCommon for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update IE DownlinkConfigCommon forRedCap |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added content “initialDownlinkBWP-RedCap-r17” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 4.6.3	Radio resource control information elements

#### –	AdditionalSpectrumEmission

Table 4.6.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 |  |  |

<< Unchanged sections skipped>>

#### –	DownlinkConfigCommon

Table 4.6.3-52: DownlinkConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
|  | BWP-DownlinkCommon with condition SCell_add |  | SCell_Add |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon |  |  |
| } |  |  |  |

#### –	DownlinkConfigCommonSIB

Table 4.6.3-53: DownlinkConfigCommonSIB

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| frequencyInfoDL | FrequencyInfoDL-SIB |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon with condition InitialBWP_SIB |  |  |
| bcch-Config SEQUENCE { |  |  |  |
| modificationPeriodCoeff | n4 |  |  |
| } |  |  |  |
| pcch-Config SEQUENCE { |  |  |  |
| defaultPagingCycle | rf128 |  |  |
| nAndPagingFrameOffset CHOICE { |  |  |  |
| halfT | 0 |  |  |
| } |  |  |  |
| ns | one |  |  |
| firstPDCCH-MonitoringOccasionOfPO | Not present |  |  |
| nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<< Unchanged sections skipped>>

<< End of changes >>