3GPP TSG-RAN5 Meeting #97	R5-226791

# Toulouse, France, 14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3304 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of V2X TC 12.2.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Pre-test conditions and Specific message contents still FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Pre-test conditions and Specific message contents. Corrected editorial errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

12.2.7.1	Inter-carrier concurrent operation / Sidelink CSI reporting / Configuration

12.2.7.1.1	Test Purpose (TP)

(1)

With { UE in NR RRC_CONNECTED state and having established PC5 RRC connection with peer UE }

Ensure that {

When { UE is configured by upper layer to configure SL CSI-RS resource to peer UE }

Then { UE sends an rrcreconfigurationsidelink message including sl-CSI-RS-Config and sl-latencyboundcsi-Report to peer UE and starts to transmit SL CSI-RS }

}

(2)

With { UE in NR RRC_CONNECTED and having established PC5 RRC connection with peer UE }

Ensure that {

When { UE is configured by upper layer to trigger SL CSI report }

Then { UE sends an SCI format 2-A to trigger SL CSI report and the 'CSI request' field in the corresponding SCI format 2-A is set to 1 }

}

12.2.7.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clause 5.8.9.1.1, 5.8.9.1.2, TS 38.214 clause 8.5.2.2, 8.2.1, TS 38.212 clause 8.3.1.1 and 8.4.1.1. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.9.1.1]

The purpose of this procedure is to modify a PC5-RRC connection, e.g. To establish/modify/release sidelink drbs, to (re-)configure NR sidelink measurement and reporting, to (re-)configure sidelink CSI reference signal resources and CSI reporting latency bound.

The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in sub-clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases:

…

-	the (re-)configuration of the sidelink CSI reference signal resources and CSI reporting latency bound.

In RRC_CONNECTED, the UE applies the NR sidelink communications parameters provided in rrcreconfiguration (if any). In RRC_IDLE or RRC_INACTIVE, the UE applies the NR sidelink communications parameters provided in system information (if any). For other cases, ues apply the NR sidelink communications parameters provided in sidelinkpreconfignr (if any). When UE performs state transition between above three cases, the UE applies the NR sidelink communications parameters provided in the new state, after acquisition of the new configurations. Before acquisition of the new configurations, UE continues applying the NR sidelink communications parameters provided in the old state.

[TS 38.331, clause 5.8.9.1.2]

The UE shall set the contents of rrcreconfigurationsidelink message as follows:

…

1>	set the sl-CSI-RS-Config;

1>	set the sl-latencyboundcsi-Report,

NOTE 1:	How to set the parameters included in sl-CSI-RS-Config and sl-latencyboundcsi-Report is up to UE implementation.

The UE shall submit the rrcreconfigurationsidelink message to lower layers for transmission.

[TS 38.214, clause 8.5.2.2]

The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-freqallocation, sl-oneantennaport, sl-CSI-RS-firstsymbol in SL-CSI-RS-Config.

Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1.

A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element. A UE is not expected to receive sidelink CSI-RS and PSSCH DM-RS, nor CSI-RS and 2nd-stage SCI, on the same symbol.

Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A triggering a report.

[TS 38.214, clause 8.2.1]

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and

-	the 'CSI request' field in the corresponding SCI format 2-A is set to 1.

The following parameters for CSI-RS transmission are configured for each CSI-RS configuration:

-	sl-CSI-RS-firstsymbol indicates the first OFDM symbol in a PRB used for SL CSI-RS

-	sl-CSI-RS-freqallocation indicates the number of antenna ports and the frequency domain allocation for SL CSI-RS.

When the UE is configured with Qp={1,2} CSI-RS port(s) in sidelink and the number of scheduled layers is ,

-	The CSI-RS scaling factor  specified in clause 8.4.1.5.3 of [4, TS 38.211] is given by  where  is the scaling factor for the corresponding PSSCH specified in clause 8.3.1.5 of [4, TS 38.211].

[TS 38.212, clause 8.3.1.1]

SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 1-A:

-	Priority – 3 bits as specified in clause 5.4.3.3 of [12, TS 23.287] and clause 5.22.1.3.1 of [8, TS 38.321].

-	Frequency resource assignment – bits when the value of the higher layer parameter sl-maxnumperreserve is configured to 2; otherwise  bits when the value of the higher layer parameter sl-maxnumperreserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter sl-maxnumperreserve is configured to 2; otherwise 9 bits when the value of the higher layer parameter sl-maxnumperreserve is configured to 3, as defined in clause 8.1.5 of [6, TS 38.214].

-	Resource reservation period – bits as defined in clause 16.4 of [5, TS 38.213], where  is the number of entries in the higher layer parameter sl-resourcereserveperiodlist, if higher layer parameter sl-multireserveresource is configured; 0 bit otherwise.

-	DMRS pattern – bits as defined in clause 8.4.1.1.2 of [4, TS 38.211], where  is the number of DMRS patterns configured by higher layer parameter sl-PSSCH-DMRS-timepatternlist.

-	2nd-stage SCI format – 2 bits as defined in Table 8.3.1.1-1.

-	Beta_offset indicator – 2 bits as provided by higher layer parameter sl-betaoffsets2ndsci and Table 8.3.1.1-2.

-	Number of DMRS port – 1 bit as defined in Table 8.3.1.1-3.

-	Modulation and coding scheme – 5 bits as defined in clause 8.1.3 of [6, TS 38.214].

-	Additional MCS table indicator – as defined in clause 8.1.3.1 of [6, TS 38.214]: 1 bit if one MCS table is configured by higher layer parameter sl-Additional-MCS-Table; 2 bits if two MCS tables are configured by higher layer parameter sl- Additional-MCS-Table; 0 bit otherwise.

-	PSFCH overhead indication – 1 bit as defined clause 8.1.3.2 of [6, TS 38.214] if higher layer parameter sl-PSFCH-Period = 2 or 4; 0 bit otherwise.

-	Reserved – a number of bits as determined by higher layer parameter sl-numreservedbits, with value set to zero.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B |
| 10 | Reserved |
| 11 | Reserved |

Table 8.3.1.1-2: Mapping of Beta_offset indicator values to indexes in Table 9.3-2 of [5, TS38.213]

| Value of Beta_offset indicator | Beta_offset index in Table 9.3-2 of [5, TS38.213] |
| --- | --- |
| 00 | 1st index provided by higher layer parameter sl-betaoffsets2ndsci |
| 01 | 2nd index provided by higher layer parameter sl-betaoffsets2ndsci |
| 10 | 3rd index provided by higher layer parameter sl-betaoffsets2ndsci |
| 11 | 4th index provided by higher layer parameter sl-betaoffsets2ndsci |

Table 8.3.1.1-3: Number of DMRS port(s)

| Value of the Number of DMRS port field | Antenna ports |
| --- | --- |
| 0 | 1000 |
| 1 | 1000 and 1001 |

[TS 38.212, clause 8.4.1.1]

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, when HARQ-ACK information includes only NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ process number –  bits.

-	New data indicator – 1 bit.

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2.

-	Source ID – 8 bits as defined in clause 8.1 of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause 8.1 of [6, TS 38.214].

-	HARQ feedback enabled/disabled indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Cast type indicator – 2 bits as defined in Table 8.4.1.1-1 and in clause 8.1 of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214] and in clause 8.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast When HARQ-ACK information includes ACK or NACK |
| 10 | Unicast |
| 11 | Groupcast When HARQ-ACK information includes only NACK |

12.2.7.1.3	Test description

12.2.7.1.3.1	Pre-test conditions

System Simulator:

-	SS-NW

-	NR Cell 1

-	System information combination 14 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used in NR Cell 1.

-	NR-SS-UE

-	NR-SS-UE1: Operating as NR sidelink communication transmitting and receiving device on the resources that UE is expected to use for transmission and reception via PC5 interface

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

-	The UE uses GNSS as the synchronization reference source.

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL as defined in TS 38.508-1 [4] subclause 4.4A on NR Cell 1with parameters Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

12.2.7.1.3.2	Test procedure sequence

Table 12.2.7.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to request a Sidelink CSI Report. Note: This step is triggered by MMI or AT command | - | - | - | - |
| 2 | Check: Does the UE transmit a RRCReconfigurationSidelink message including IEs sl-CSI-RS-Config and sl-latencyboundcsi-Report on SL-SRB3? | --> | PC5 RRC: RRCReconfigurationSidelink | 1 | P |
| 3 | The NR-SS-UE1 transmits a RRCReconfigurationCompleteSidelink message on SL-SRB3. | <-- | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 4 | Check: Does the UE transmit an SCI format 2-A to trigger SL CSI report and the 'CSI request' field in the corresponding SCI format 2-A is set to 1. | --> | PSSCH (SCI format 2-A) | 2 | P |
| 5 | The NR-SS-UE1 sends a Sidelink CSI Reporting MAC CE. | <-- | MAC CE (Sidelink CSI Reporting) | - | - |

12.2.7.1.3.3	Specific message contents

Table 12.2.7.1.3.3-1: SIB12-IEs-r16 (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-CSI-Acquisition-r16 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.7.1.3.3-2: RRCReconfigurationSidelink (step 2, Table 12.2.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-3 with condition SL_CSI and TX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| sl-CSI-RS-Config-r16 CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| sl-CSI-RS-FreqAllocation-r16 | Any value |  |  |
| sl-CSI-RS-FirstSymbol-r16 | (3..12) |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-LatencyBoundCSI-Report-r16 | (3..160) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.7.1.3.3-3: RRCReconfigurationCompleteSidelink (step 3, Table 12.2.7.1.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-4 with condition RX

<End of modified section>