3GPP TSG-RAN5 Meeting #97	R5-226832

# Toulouse, France,  14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0708 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test point analysis for reference sensitivity exception case 7.3A.1_1 test analysis for band CA_n70A-n71A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has changed minimum requirements format for reference sensitivity UL Harmonic and Cross band isolation exceptions, which now includes specific test points for each, starting from 38.101-1 17.6.0. The selected test points are not the same as the ones selected by RAN5 for CA_n29A-n71A and CA_n70A-n71A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The band combination CA_n70A-n71A reference sensitivity test points are aligned with 38.101-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band combination CA_n70A-n71A reference sensitivity test points will remain not aligned with 38.101-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.1.3.1-6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 4.1.3.1-6: Test frequency selection per band pair for UL harmonics exception

| Band pair (Note 2) |  | Frequency |  | Channel BW [MHz] |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1A / n77A |  | Mid / Low |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid / 3900 |  | 20/ Highest |  | Exception Note 2 of TS 38.101 table 7.3A.4-1 |  |  |
|  |  | Mid / 3870 |  | 20/20 |  | Exception Note 3 of TS 38.101 table 7.3A.4-1 |  |  |
| n2A/n48A |  | UL 1860 MHz / DL 3700 MHz |  | 20/20 |  | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3400 MHz |  | 20/100 |  | HD avoid |  |  |
| n2A/n77A |  | UL 1860 MHz / DL 3720 MHz |  | 20/100 |  | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3700 MHz |  | 20/20 |  | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3800 MHz |  | 20/100 |  | HD avoid |  |  |
| n3A / n77A |  | TBD/TBD |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | TBD/TBD |  | Highest/Highest |  | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |  |
|  |  | TBD/TBD |  | 20/20 |  | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |  |
| n3A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid/3495 MHz |  | Highest/Highest |  | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |  |
|  |  | Mid/3465 MHz |  | 20/20 |  | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |  |
| n5A/n77A |  | UL 834 MHz / DL 3336 MHz |  | 20/100 |  | Exception Note 4 of TS 38.101-1 table 7.3A.4-1 |  |  |
|  |  | UL 834 MHz / DL 4170 MHz |  | 20/100 |  | Exception Note 5 TS 38.101-1 table 7.3A.4-1 |  |  |
|  |  | UL 834 MHz/DL 3800 MHz |  | 20/100 |  | HD Avoid |  |  |
| n5A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 in TS 38.521-1 [2] |  |  |
|  |  | Mid/3346 MHz |  | Highest/Highest |  | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |  |  |
|  |  | Mid/3316 MHz |  | 20/20 |  | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |  |  |
| n8A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid/3590 MHz |  | Highest/Highest |  | Exception Note 5 of TS38.101 table 7.3A.4-1 |  |  |
| n70A / n71A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Low/665MHz (UL) |  | Highest / 5 |  | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@0 |  |  |
|  |  | Low/665MHz (UL) |  | 5 / 5 |  | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@0 |  |  |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. |  |  |  |  |  |  |  |