3GPP TSG-RAN5 Meeting #97	R5-226854

# Toulouse , France , 14th – 18th Nov. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1998 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new FR1 test case REFSENS for SUL with UL MIMO and DL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE which can transmit UL MIMO on SUL bands together with DL CA, test case REFSENS need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 7.3D.2_2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For UE which can transmit UL MIMO on SUL band and DL CA,test case are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3D.2_2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New test case 7.3D.2_2 is added after new test case 7.3D.2_1 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

7.3D.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1 and parameters specified Tables 7.3D.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3.

7.3D.2_2	Reference sensitivity power level for SUL with UL MIMO (3CC)

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for UL MIMO on SUL with DL CA. This test case is covered by 7.3D.2 and 7.3D.2_1.

7.3D.2_2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 2 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3D.2_2.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support SUL and UL MIMO operating on the SUL bands..

7.3D.2_2.3	Minimum conformance requirements

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.3C shall be met with the UL MIMO configurations described in clause 6.2D.1. For UL MIMO, the parameter PUMAX is the total transmitter power over the two transmits power over the two transmit antenna connectors

The normative reference for this requirement is TS 38.101-1 [2] clauses 7.3D and 7.3C.

7.3D.2_2.4	Test description

NOTE:	No testing needs to be performed since the testing has been covered in test case 7.3D.2 and 7.3D.2_1.

For band combination CA_nX_SUL_nY-nZ, test the REFSENS of SUL configuration or NR band as listed in table 7.3C.3.4-1.

Table 7.3D.2_2.4-1: Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78A-n80A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
|  | n1 | 7.3D.2 | Table 7.3D.2.4.1-1 |
| CA_n1A_SUL_n78A-n84A | SUL_n78A-n84A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
|  | n1 | 7.3D.2 | Table 7.3D.2.4.1-1 |
| CA_n3A_SUL_n78A-n80A | SUL_n78A-n80A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
|  | n3 | 7.3D.2 | Table 7.3D.2.4.1-1 |
| CA_n28A_SUL_n41A-n83A | SUL_n41A-n83A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
|  | n28 | 7.3D.2 | Table 7.3D.2.4.1-1 |
| SUL_n79C-n83A | SUL_n79A-n83A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | 7.3D.2_1 | Table 7.3D.2_1.4.1 |
|  | n28 | 7.3D.2 | Table 7.3D.2.4.1-1 |

7.3D.2_2.5	Test requirement

Same test requirement as clause 7.3D.2 and 7.3D.2_1 for each band or band combinations listed in table 7.3D.2_2.4-1.

<End of Changes>