3GPP TSG-RAN5 Meeting #97-e	R5-226903

# Toulouse, France, 14th – 18th Nov. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2013 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating 6.3, 6.3D and 6.3G to coordinate the output power dynamic test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Coordinating the test cases based on discussion paper R5-226898. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the test applicability of test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicability across test cases are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.3D, 6.3G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3	Output power dynamics

6.3.1	Minimum output power

6.3.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3.1.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.1.3	Minimum conformance requirements

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

The minimum output power is defined as the mean power in at least one sub-frame 1 ms. The minimum output power shall not exceed the values specified in Table 6.3.1.3-1.

Table 6.3.1.3-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40 | 4.515 |
| 10 | -40 | 9.375 |
| 15 | -40 | 14.235 |
| 20 | -40 | 19.095 |
| 25 | -39 | 23.955 |
| 30 | -38.2 | 28.815 |
| 40 | -37 | 38.895 |
| 45 | -36.5 | 43.575 |
| 50 | -36 | 48.615 |
| 60 | -35.2 | 58.35 |
| 70 | -34.6 | 68.07 |
| 80 | -34 | 78.15 |
| 90 | -33.5 | 88.23 |
| 100 | -33 | 98.31 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.1.

6.3.1.4	Test description

6.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | DFT-s-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.1.4.3.

6.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "down" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power.

3.	Measure the mean power of the UE in the associated measurement channel bandwidth specified in Table 6.3.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exception.

Table 6.3.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.3.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3.1.5-1.

Table 6.3.1.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.1.5-2 |  |  |

Table 6.3.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

6.3.2	Transmit OFF power

6.3.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3.2.2	Test applicability

The requirements of this test apply in test cases 6.3.3 Transmit ON/OFF time mask to all types of NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test cases 6.3.3 Transmit ON/OFF time mask to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.2.3	Minimum conformance requirements

Transmit OFF power is defined as the mean power in the channel bandwidth when the transmitter is OFF. The transmitter is considered OFF when the UE is not allowed to transmit on any of its ports.

The Transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1ms) excluding any transient periods. The Transmit OFF power shall not exceed the values specified in Table 6.3.2.3-1.

Table 6.3.2.3-1: Transmit OFF power

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.2.

6.3.2.4	Test description

This test is covered by clause 6.3.3 Transmit ON/OFF time mask.

6.3.2.5	Test requirement

The requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3.2.5-1.

Table 6.3.2.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.2.5-2 |  |  |

Table 6.3.2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3.3	Transmit ON/OFF time mask

6.3.3.1	General

The transmit power time mask defines the transient period(s) allowed

-	between transmit OFF power as defined in sub-clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

-	between continuous ON-power transmissions with power change or RB hopping is applied.

When a UE signals the transient period capability, the transient period value (tp) can be 2, 4, or 7 s.  If no capability is signalled, the default transient period value of 10 s applies.

In case of RB hopping, and in following figures where tpstart is specified, the transient period is shared symmetrically when the transient period is 10 s. If the UE signals a transient period (tp) of 2, 4 or 7 s, the transient period start position is given by tpstart in Table 6.3.3.1-1.

Table 6.3.3.1-1: tpstart  values

| tp (s) | tpstart (s) |
| --- | --- |
| 2 | -0.5 |
| 4 | -1 |
| 7 | -2.7 |
| NOTE 1: Negative values mean that the transient period starts before the symbol boundary |  |

Unless otherwise stated the minimum requirements in clause 6.5 apply also in transient periods.

In the following sub-clauses, following definitions apply:

-	A slot or long subslot transmission is a transmission with more than 2 symbols.

-	A short subslot transmission is a transmission with 1 or 2 symbols.

6.3.3.2	General ON/OFF time mask

6.3.3.2.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3.3.2.5.

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power as defined in sub-clause 6.3.2 and transmit ON power symbols (transmit ON/OFF)

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3.3.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.3.2.3	Minimum conformance requirements

The general ON/OFF time mask defines the observation period between transmit OFF and ON power and between transmit ON and OFF power for each SCS. ON/OFF scenarios include: the beginning or end of DTX, measurement gap, contiguous, and non-contiguous transmission, etc.

The OFF power measurement period is defined in a duration of at least one slot excluding any transient periods. The ON power is defined as the mean power over one slot excluding any transient period.

Figure 6.3.3.2.3-1: General ON/OFF time mask for NR UL transmission in FR1

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.3.2.

6.3.3.2.4	Test description

6.3.3.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range and High range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.3.2.4.3.

6.3.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.3.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is  such that the UE transmits on slots 8 for15kHz SCS, on slots 8 and 18 for 30kHz SCS and on slots 17 and 37 for 60kHz SCS.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	ON power sub test:

3.1.	Measure the output power of the UE PUSCH transmission during one slot.

4.	OFF power sub test:

4.1.	Measure the UE transmission OFF power during the slot prior to the PUSCH transmission, excluding a transient period of 10 µs in the end of the slot.

4.2.	Measure the UE transmission OFF power during the slot following the PUSCH transmission, excluding a transient period of 10 µs at the beginning of the slot.

6.3.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3.3.2.4.3-1: Void

Table 6.3.3.2.4.3-2: Void

Table 6.3.3.2.4.3-3: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms10 |  | FR1_15kHz |
| nrofDownlinkSlots | 6 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 14 |  | FR1_60kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSlots | 3 |  | FR1_15kHz,FR1_30kHz |
|  | 4 |  | FR1_60kHz |
| nrofUplinkSymbols | 4 |  | FR1_30kHz |
|  | 2 |  | FR1_15kHz, |
|  | 8 |  | FR1_60kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3.3.2.4.3-4: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | FR1_15kHz,FR1_30kHz |
|  | 6 |  | FR1_60kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |

Table 6.3.3.2.4.3-5: Void

Table 6.3.3.2.4.3-6: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.3.3.2.4.3-7: P-Max

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  |  |

6.3.3.2.5	Test requirement

The requirement for the power measured in steps 2, 3 and 4 of the test procedure shall not exceed the values specified in Table 6.3.3.2.5-1.

Table 6.3.3.2.5-1: General ON/OFF time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |  |
| Transmit ON power | Same as Table 6.2.1.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth of OFF power is specified in Table 6.3.3.2.5-2 NOTE 2: TT for each frequency and channel bandwidth of ON power is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.3.2.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3.3.2.5-3: Void

6.3.3.3	Transmit power time mask for slot and short or subslot boundaries

No test case details are specified. Current test procedures for time masks are based on power measurement in relatively long period compared with transient period. For time masks between 2 active time slots with different power level, the test procedure can’t provide enough resolution to identify non-conformant UEs. Therefore, the minimum requirement is not testable.

6.3.3.4	PRACH time mask

6.3.3.4.1	Test purpose

To verify that the PRACH time mask meets the requirements given in 6.3.3.4.5.

The time mask for PRACH time mask defines the transient period(s)allowed between transmit OFF power and transmit ON power when transmitting the PRACH.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3.3.4.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.3.4.3	Minimum conformance requirements

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 6.3.3.4.3-1. The measurement period for different PRACH preamble format is specified in Table 6.3.3.4.3-1.

Table 6.3.3.4.3-1: PRACH ON power measurement period

| PRACH preamble format | SCS (kHz) | Measurement period (ms) |
| --- | --- | --- |
| 0 | 1.25 | 0.903125 |
| 1 | 1.25 | 2.284375 |
| 2 | 1.25 | 3.352604 |
| 3 | 5 | 0.903125 |
| A1 | 15 | 0.142708 |
|  | 30 | 0.071354 |
| A2 | 15 | 0.285417 |
|  | 30 | 0.142708 |
| A3 | 15 | 0.428125 |
|  | 30 | 0.2140625 |
| B1 | 15 | 0.140365 |
|  | 30 | 0.070182 |
| B4 | 15 | 0.83046875 |
|  | 30 | 0.415234375 |
| A1/B1 | 15 | 0.142708 ms for first six occasion 0.140365 ms for the last occasion |
|  | 30 | 0.071354 ms for first six occasion 0.070182 ms for the last occasion |
| A2/B2 | 15 | 0.285417 ms for first two occasion 0.278385 ms for the third occasion |
|  | 30 | 0.142708 ms for first two occasion 0.1391925 ms for the third occasion |
| A3/B3 | 15 | 0.428125 ms for the first occasion 0.41640625 ms for the second occasion |
|  | 30 | 0.2140625 ms for the first occasion 0.208203125 ms for the second occasion |
| C0 | 15 | 0.10703125 |
|  | 30 | 0.053515625 |
| C2 | 15 | 0.333333 |
|  | 30 | 0.166667 |
| NOTE: For PRACH on PRACH occasion start from the beginning of 0.5ms or span the boundary of 0.5ms of the subframe, the measurement period will plus 0.032552μs |  |  |

Figure 6.3.3.4.3-1: PRACH ON/OFF time mask

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.3.4.

6.3.3.4.4	Test description

6.3.3.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes [TBD]. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 | Mid range (NOTE 1) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 | SCS defined in TS 38.211 [8] subclause 6.3.3.2 determined by PRACH Configuration Index for long sequence Lowest, Highest for short sequence |  |
| PRACH preamble format |  |  |
|  | Paired Spectrum | Unpaired Spectrum |
| PRACH Configuration Index for test point 1 | 4 (long sequence) | 12 (long sequence) |
| PRACH Configuration Index for test point 2 | 160 (short sequence) | 123 (short sequence) |
| NOTE 1: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.3.4.4.3.

6.3.3.4.4.2	Test procedure

1.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

2.	The UE shall send the signalled preamble to the SS.

3.	The SS measure the UE transmission OFF power during the slot preceding the PRACH preamble excluding a transient period of 10 µs according to Figure 6.3.3.4.3-1.

4.	Measure the output power of the transmitted PRACH preamble according to Figure 6.3.3.4.3-1.

5.	Measure the UE transmission OFF power, starting 10 µs after the PRACH preamble ends for a measurement period.

6.3.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.3 with the following exceptions:

Table 6.3.3.4.4.3-1: RACH-ConfigCommon: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  | PRACH Format A3 |
| l839 | 0 | NR Cell 1 | PRACH Format 0 |
|  | TBD | Other than NR Cell 1 | PRACH Format 0 |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.4.4.3-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 4 | Paired Spectrum | PRACH Format 0 |
|  | 160 | Paired Spectrum | PRACH Format A3 |
|  | 12 | Unpaired Spectrum | PRACH Format 0 |
|  | 123 | Unpaired Spectrum | PRACH Format A3 |
| msg1-FDM | four |  | FR1 |
|  | one |  | FR1 5MHz PRACH Format A3 for SCS 15 kHz OR FR1 10MHz PRACH Format A3 for SCS 30 kHz OR FR1 10MHz PRACH Format A3 for SCS 60 kHz |
| preambleReceivedTargetPower | -118 |  | PRACH Format 0 |
|  | -122 |  | PRACH Format A3 for SCS 15 kHz |
|  | -124 |  | PRACH Format A3 for SCS 30 kHz |
|  | -128 |  | PRACH Format A3 for SCS 60kHz |
| powerRampingStep | dB0 |  |  |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |

Table 6.3.3.4.4.3-3: ServingCellConfigCommonSIB: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 32 |  | SCS_15kHz |
|  | 35 |  | SCS_30kHz |
| } |  |  |  |

Table 6.3.3.4.4.3-4: PUSCH-TimeDomainResourceAllocationList: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 6 | K2+ Δ=8 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 | Unpaired Spectrum for SCS15kHz and PRACH Format 0 |
| } |  |  |  |
| } |  |  |  |

6.3.3.4.5	Test requirement

The requirement for the power measured in steps (3), (4) and (5) of the test procedure shall not exceed the values specified in Table 6.3.3.4.5-1.

Table 6.3.3.4.5-1: PRACH time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Expected PRACH Transmission ON Measured Power for PRACH Format 0 and PRACH Format A3 for SCS 30kHz | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm |
| Expected PRACH Transmission ON Measured Power for PRACH Format A3 for SCS 15kHz and SCS 60kHz | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm |
| ON Power Tolerance | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.3.4.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.3.4.5-2: Test Tolerance (Transmit OFF power and PRACH time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3.3.5	Void

6.3.3.6	SRS time mask

6.3.3.6.1	Test purpose

To verify that the SRS time mask meets the requirements given in 6.3.3.6.5.

The time mask for SRS time mask defines the transient period(s) allowed between transmit OFF power and transmit ON power when transmitting the SRS.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3.3.6.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.3.6.3	Minimum conformance requirements

For SRS transmission mapped to one OFDM symbol, the ON power is defined as the mean power over the symbol duration excluding any transient period; See Figure 6.3.3.6.3-1

Figure 6.3.3.6.3-1: Single SRS time mask for NR UL transmission

For SRS transmission mapped to two or more OFDM symbols the ON power is defined as the mean power for each symbol duration excluding any transient period. For consecutive SRS transmissions without power change, Figure 6.3.3.6.3-2 applies.

Figure 6.3.3.6.3-2: Consecutive SRS time mask for the case when no power change is required with SRS usage other than antenna switching

When power change between consecutive SRS transmissions is required, then Figure 6.3.3.6.3-3 and Figure 6.3.3.6.3-4 apply.

Figure 6.3.3.6.3-3: Consecutive SRS time mask for the case when power change is required and when 15kHz and 30kHz SCS is used in FR1 with SRS usage other than antenna switching

Figure 6.3.3.6.3-4: Consecutive SRS time mask for the case when power change is required and when 60kHz SCS is used in FR1

Figure 6.3.3.6.3-5: FR1 Time mask for 15 kHz and 30 kHz SCS for the case when consecutive SRS switching usage is between antenna switching & other sets

where “other sets” belongs to a “usage set” other than the set for antenna switching. The usage sets for SRS switching are defined in section 6.2.1 of TS 38.214 [12].

NOTE:	Guard period of one symbol is defined between two SRS resources of an SRS resource set for antenna switching for 15kHz, 30kHz and 60kHz SCS in Table 6.2.1.2-1 of TS 38.214 [10].

The above transient period applies to all the transmit CCs in CA with the CC sounding SRS. UE RF requirements do not apply during this transient period.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.3.6.

6.3.3.6.4	Test description

6.3.3.6.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.3.3.6.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.3.6.4.1-1: Test Configuration Table

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 | Mid range (NOTE 1) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 | Lowest, Highest |  |
| SRS configuration | Paired Spectrum | Unpaired Spectrum |
| c-SRS (SRS bandwidth configuration) | 7 (24 RB for BW 5 MHz) 14 (52 RB for BW 10 MHz) 20 (76 RB for BW 15 MHz) 25 (104 RB for BW 20 MHz) 33 (132 RB for BW 25 MHz) 42 (160 RB for BW 30 MHz) 51 (216 RB for BW 40MHz) 58 (242 RB for BW 45MHz) 60 (264 RB for BW 50MHz) for SCS 15 KHz | 7 (for BW 5 MHz) 13 (for BW 10 MHz) 19 (for BW 15 MHz) 25 (for BW 20 MHz) 30 (for BW 25 MHz) 40 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) for SCS 15 KHz |
|  | 1 (8 RB for BW 5 MHz) 7 (24 RB for BW 10 MHz) 10 (36 RB for BW 15 MHz) 13 (48 RB for BW 20 MHz) 17 (64 RB for BW 25 MHz) 20 (76 RB for BW 30 MHz) 25 (104 RB for BW 40MHz) 30 (117 RB for BW 45MHz) 33 (132 RB for BW 50MHz) 42 (160 RB for BW 60MHz) 45 (184 RB for BW 70MHz) 51 (216 RB for BW 80MHz) 56 (240 RB for BW 90MHz) 63 (272 RB for BW 100MHz) for SCS 30 KHz | 1 (for BW 5 MHz) 6 (for BW 10 MHz) 10 (for BW 15 MHz) 12 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 50MHz) 40 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 53 (for BW 90MHz) 61 (for BW 100MHz) for SCS 30 KHz |
|  | 1 (8 RB for BW 10 MHz) 4 (16 RB for BW 15 MHz) 7 (24 RB for BW 20 MHz) 8 (28 RB for BW 25 MHz) 10 (36 RBfor BW 30 MHz) 13 (48 RB for BW 40MHz) 15 (56 RB for BW 45MHz) 17 (64 RB for BW 50MHz) 20 (76 RB for BW 60MHz) 22 (88 RB for BW 70MHz) 25 (104 RB for BW 80MHz) 29 (120 RB for BW 90MHz) 33 (132 RB for BW 100MHz) for SCS 60 KHz | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20, 25 MHz) 9 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 19 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 27 (for BW 90MHz) 30 (for BW 100MHz) for SCS 60 KHz |
| b-SRS | 0 | 0 |
| b-hop | 3 | 0 |
| freqDomainPosition | 0 | 0 |
| SRS-PeriodicityAndOffset | sl10 for SCS 15 KHz | sl10 for SCS 15 KHz |
|  | sl20 for SCS 30 KHz | sl20 for SCS 30 KHz |
|  | sl40 for SCS 60 KHz | sl40 for SCS 60 KHz |
| transmissionComb | n2 | n2 |
| CombOffset | 0 | 0 |
| cyclicShift | 0 | 0 |
| startPosition | 0 | 0 |
| nrofSymbols | n1 | n1 |
| NOTE 1: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.3.6.4.3.

6.3.3.6.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.3.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slot 4 for15kHz SCS, on slot 8 for 30kHz SCS and on slot 16 for 60kHz SCS. PUSCH is transmitted in the first half of the frame.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	ON power sub test:

3.1.	Measure the output power of the transmitted SRS transmission during 1 OFDM symbol. The SRS transmission in the second half of the frame is used for measurement since there is no PUSCH transmission before and after.

4.	OFF power sub test:

4.1.	The SS measure the UE transmission OFF power during the 13 OFDM symbols, preceding the SRS symbol excluding a transient period of 10 μs.

4.2.	Measure the UE transmission OFF power during the slot following the SRS under test, excluding a transient period of 10 µs

6.3.3.6.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.3 with the following exceptions:

Table 6.3.3.6.4.3-0: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1[5], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| srs-Config | SRS-Config in Table 6.3.3.6.4.3-1 |  |  |
| } |  |  |  |
| Note: This message exception is only valid for the initial BWP and not for an additional BWP inside BWP-Uplink. |  |  |  |

Table 6.3.3.6.4.3-1: SRS-Config: SRS time mask measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { | 1 entry |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| alpha | alpha08 |  |  |
| p0 | 0 |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { | 1 entry |  |  |
| resourceMapping SEQUENCE { |  |  |  |
| startPosition | 0 |  |  |
| nrofSymbols | n1 |  |  |
| repetitionFactor | n1 |  |  |
| } |  |  |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS |  |  |  |
|  | 7 (for BW 5 MHz) 14 (for BW 10 MHz) 20 (for BW 15 MHz) 25 (for BW 20 MHz) 33 (for BW 25 MHz) 42 (for BW 30 MHz) 51 (for BW 40MHz) 58 (for BW 45MHz) 60 (for BW 50MHz) |  | Paired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 7 (for BW 10 MHz) 10 (for BW 15 MHz) 13 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 45MHz) 33 (for BW 50MHz) 42 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 56 (for BW 90MHz) |  | Paired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20 MHz) 8 (for BW 25 MHz) 10 (for BW 30 MHz) 13 (for BW 40MHz) 15 (for BW 45MHz) 17 (for BW 50MHz) 20 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 29 (for BW 90MHz) 33 (for BW 100MHz) |  | Paired Spectrum for SCS 60 KHz |
|  | 7 (for BW 5 MHz) 13 (for BW 10 MHz) 19 (for BW 15 MHz) 25 (for BW 20 MHz) 30 (for BW 25 MHz) 40 (for BW 30 MHz) 51 (for BW 40MHz) 60 (for BW 50MHz) |  | Unpaired Spectrum for SCS 15 KHz |
|  | 1 (for BW 5 MHz) 6 (for BW 10 MHz) 10 (for BW 15 MHz) 12 (for BW 20 MHz) 17 (for BW 25 MHz) 20 (for BW 30 MHz) 25 (for BW 40MHz) 30 (for BW 50MHz) 40 (for BW 60MHz) 45 (for BW 70MHz) 51 (for BW 80MHz) 53 (for BW 90MHz) 61 (for BW 100MHz) |  | Unpaired Spectrum for SCS 30 KHz |
|  | 1 (for BW 10 MHz) 4 (for BW 15 MHz) 7 (for BW 20, 25 MHz) 9 (for BW 30 MHz) 13 (for BW 40MHz) 17 (for BW 50MHz) 19 (for BW 60MHz) 22 (for BW 70MHz) 25 (for BW 80MHz) 27 (for BW 90MHz) 30 (for BW 100MHz) |  | Unpaired Spectrum for SCS 60 KHz |
| b-SRS | 0 |  |  |
| b-hop | 3 |  | Paired Spectrum |
|  | 0 |  | Unpaired Spectrum |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE{ |  |  |  |
| sl10 | 9 |  | SCS 15 KHz |
| sl20 | 18 |  | SCS 30KHz |
| sl40 | 36 |  | SCS 60 KHz |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.3.6.4.3-2: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

6.3.3.6.5	Test requirement

The requirement for the power measured in steps (1), (2) and (3) of the test procedure shall not exceed the values specified in Table 6.3.3.6.5-1.

Table 6.3.3.6.5-1: SRS time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Table 6.2.1.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3.3.6.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.3.6.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3.4	Power control

6.3.4.1	General

The requirements on power control accuracy apply under normal conditions.

6.3.4.2	Absolute power tolerance

6.3.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20ms.

6.3.4.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.4.2.3	Minimum conformance requirements

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame(1ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20ms. The tolerance includes the channel estimation error.

The minimum requirement specified in Table 6.3.4.2.3-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 and the maximum output power as specified in sub-clause 6.2.1.

Table 6.3.4.2.3-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.4.2

6.3.4.2.4	Test description

6.3.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in clause 6.2D.3.4.1 for NS_03, NS_03U, NS_04 and NS_35. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Absolute power tolerance test case |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.4.2.4.3. Note that PDCCH DCI format 0_1 sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3.4.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3.4.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Measure the initial output power of the first sub-frame (1ms) of UE PUSCH first transmission.

3.	Repeat for the two test points as indicated in section 6.3.4.2.4.3. The timing of the execution between the two test points shall be larger than 20ms.

6.3.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.3.4.2.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -114 | Test point 1 to verify a UE relative low initial power transmission |  |
| } |  |  |  |

Table 6.3.4.2.4.3-2: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 | Test point 2 to verify a UE relative high initial power transmission |  |
| } |  |  |  |

Table 6.3.4.2.4.3-3: Void

Table 6.3.4.2.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

6.3.4.2.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3.4.2.5-1 and 6.3.4.2.5-2.

Table 6.3.4.2.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -8.3 | -7.9 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 |  | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -8.5 | -8 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.4.2.5-2: Absolute power tolerance: test point 2

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -3.6 | 0.4 | 1.4 | 2.7 | 3.6 | 4.4 | 5.7 | 6.2 | 6.7 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -4.2 | -0.8 | 1.2 | 2.5 | 3.5 | 4.3 | 5.7 | 6.2 | 6.6 | 7.5 | 8.2 | 8.8 | 9.3 | 9.8 |
|  | SCS60 | N/A | -1.2 | 1 | 2.2 | 3.3 | 4.2 | 5.5 | 6 | 6.5 | 7.4 | 8.1 | 8.7 | 9.2 | 9.7 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The higher power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2.1.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3.4.2.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB | 1.4 dB |
| 40MHz < BW ≤ 100MHz | 1.4 dB | 1.4 dB | 1.4 dB |

6.3.4.3	Relative power tolerance

6.3.4.3.1	Test purpose

To verify the ability of the UE transmitter to set its output power in a target sub-frame(1ms) relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is less than or equal to 20ms.

6.3.4.3.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.4.3.3	Minimum conformance requirement

The UE shall meet the requirements specified in Table 6.3.4.3.3-1.

The minimum requirements specified in Table 6.3.4.3.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the minimum output power as defined in sub-clause 6.3.1 and the measured  PUMAX as defined in sub-clause 6.2.1.

To account for RF Power amplifier mode changes, 2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep over a range bounded by the requirements of minimum power and maximum power specified in subclauses 6.3.1 and 6.2.1, respectively. For those exceptions, the power tolerance limit is a maximum of ± 6.0 dB in Table 6.3.4.3.3-1.

Table 6.3.4.3.3-1: Relative Power Tolerance

| Power step P (Up or down) (dB) | All combinations of PUSCH and PUCCH transitions (dB) | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames (dB) | PRACH (dB) |
| --- | --- | --- | --- |
| ΔP < 2 | ± 2.0 (NOTE) | ± 2.5 | ± 2.0 |
| 2 ≤ ΔP < 3 | ± 2.5 | ± 3.5 | ± 2.5 |
| 3 ≤ ΔP < 4 | ± 3.0 | ± 4.5 | ± 3.0 |
| 4 ≤ ΔP ≤ 10 | ± 3.5 | ± 5.5 | ± 3.5 |
| 10 ≤ ΔP < 15 | ± 4.0 | ± 7.0 | ± 4.0 |
| 15 ≤ ΔP | ± 5.0 | ± 8.0 | ± 5.0 |
| NOTE: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods: for a power step ΔP ≤ 1 dB, the relative power tolerance for transmission is ± 0.7 dB. |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3.4.3.

6.3.4.3.4	Test description

6.3.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.4.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation (NOTE 1) |
| 5MHz | N/A for Relative power tolerance test case |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-1 See Table 6.3.4.3.5-2 See Table 6.3.4.3.5-7 |
| 10MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 15MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 20MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 25MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 30MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 40MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 45MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 50MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-3 See Table 6.3.4.3.5-4 See Table 6.3.4.3.5-7 |
| 60MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 70MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 80MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 90MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 100MHz |  |  | DFT-s-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| Note 1: The starting resource block shall be RB# 0 |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3.4.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5 Message contents are defined in clause 6.3.4.3.4.3.

6.3.4.3.4.2	Test procedure

The procedure is separated in various subtests to verify different aspects of relative power control. The power patterns of the subtests are described in figure 6.3.4.3.4.2-1 thru figure 6.3.4.3.4.2-5.

Figure 6.3.4.3.4.2-1: FDD ramping up test power patterns

Figure 6.3.4.3.4.2-2: FDD ramping down test power patterns

Figure 6.3.4.3.4.2-3: TDD ramping up test power patterns

Figure 6.3.4.3.4.2-4: TDD ramping down test power patterns

Figure 6.3.4.3.4.2-5: Alternating Test Power patterns

1.	Sub test: ramping up pattern

1.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -33 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

1.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3.4.3.4.2-1 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame) and Figure 6.3.4.3.4.2-3 (TDD pattern A: sub-test is divided in 20 arbitrary radio frames with 2 active uplink sub-frames per radio frame). Uplink RB allocation as defined in table 6.3.4.3.5-1/6.3.4.3.5-3/ 6.3.4.3.5-5 depending on channel bandwidth. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit a +1dB TPC command for every first slot in a sub-frame. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

1.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3.4.3.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

1.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.4.3.5-1/6.3.4.3.5-3/ 6.3.4.3.5-5 to force bigger UE power steps at various points in the power range.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.2.

2.	Sub test: ramping down pattern

2.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1.  Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -(MU + Uplink power control window size) to -MU dB of the target power level 20.7 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size is same as defined in step 1.1.

2.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3.4.3.4.2-2 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink slots per radio frame) and Figure 6.3.4.3.4.2-4 (TDD pattern A: sub-test is divided in 20 arbitrary radio frames with 2 active uplink sub-frames per radio frame). Uplink RB allocation as defined in table 6.3.4.3.5-2/6.3.4.3.5-4/ 6.3.4.3.5-6 depending on channel bandwidth. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit a -1dB TPC command for every first slot in a sub-frame. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

2.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3.4.4.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

2.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3.4.3.5-2/6.3.4.3.5-4/ 6.3.4.3.5-6 to force bigger UE power steps at various points in the power range.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.3.

3.	Sub test: alternating pattern

3.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.  Send uplink power control commands for PUSCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as – (Uplink power control window size / 2) dB to + (Uplink power control window size / 2) dB of the target power level -10 dBm, where:

-	Uplink power control window size is same as defined in step 1.1.

3.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3.5.2.4.2-5 for 10 sub-frames an uplink RB allocation alternating pattern as defined in table 6.3.4.3.5-7 while transmitting 0dB TPC command for PUSCH via the PDCCH.

3.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements specified in clause 6.3.4.3.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.4.

6.3.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with the following exceptions:

Table 6.3.4.3.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.3.4.3.5	Test requirement

Each UE power step measured in the test procedure 6.3.4.3.4.2 should satisfy the test requirements specified in Table 6.3.4.3.5-1 thru 6.3.4.3.5-7.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3.4.3.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80+/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-5: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 81 RBs | TPC=+1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 81 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 75 RBs | TPC=+1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 75 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-6: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 81 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 81 RBs to 1 RB | TPC=-1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 75 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 75 RBs to 1 RB | TPC=-1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3.4.3.5-7: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
| 5 |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40,45, 50 | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 24 | TPC=0dB | 13.80 | 10dB ≤ ΔP < 15dB | 13.80 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 60 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 81 | TPC=0dB | 19.08 | 15dB < ΔP | 19.08 +/- (5 + TT) |
| 60, 70,80,90,100 |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 60 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 75 | TPC=0dB | 18.75 | 15dB < ΔP | 18.75 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

6.3.4.4	Aggregate power tolerance

6.3.4.4.1	Test purpose

To verify the ability of the UE transmitter to maintain its power during non-contiguous transmissions within 21ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in 38.213 kept constant.

6.3.4.4.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply to all types of NR Power Class 3 and Power Class 2 UE release 15 and forward that don't support txDiversity-r16.

6.3.4.4.3	Minimum conformance requirements

The aggregate power control tolerance is the ability of the UE transmitter to maintain its power in a sub-frame(1ms) during non-contiguous transmissions within 21ms  in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in 38.213 kept constant.

The minimum requirement specified in Table 6.3.4.4.3-1 apply in the power range bounded by the minimum output power as specified in sub-clause 6.3.1 and the maximum output power as specified in sub-clause 6.2.2.

Table 6.3.4.4.3-1: Aggregate power tolerance

| TPC command | UL channel | Aggregate power tolerance within 21ms |
| --- | --- | --- |
| 0 dB | PUCCH | ± 2.5 dB |
| 0 dB | PUSCH | ± 3.5 dB |

The normative reference for this requirement is TS 38.01-1 [2] clause 6.3.4.4

6.3.4.4.4	Test description

6.3.4.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3.4.4.4.1-1 and table 6.3.4.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3.4.4.4.1-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 1) |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |
|  | N/A for aggregate power tolerance testcase | PUCCH format = Format 1 Length in OFDM symbols = 14 |
| 1 |  |  |
| NOTE 1: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |

Table 6.3.4.4.4.1-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range (NOTE 2) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for aggregate power tolerance testcase | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to G.0, G.1, G.2, G.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.3.4.4.4.1-1 (PUCCH sub-test) and Table 6.3.4.4.4.1-2 (PUSCH sub-test)

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3.4.4.4.3.

6.3.4.4.4.2	Test procedure

The procedure is separated in two subtests to verify PUCCH and PUSCH aggregate power control tolerance respectively. The uplink transmission patterns are described in figure 6.3.4.4.4.2-1.

Figure 6.3.4.4.4.2-1 Test uplink transmission

1.	PUCCH sub test:

1.1.	The SS transmits PDSCH via PDCCH DCI format 0_1 for C_RNTI to transmit the DL RMC according to Table 6.3.4.4.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. Send uplink power control commands for PUCCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as – (Uplink power control window size / 2) dB to + (Uplink power control window size / 2) dB of the target power level + 0 dBm, where:

-	Uplink power control window size = 1dB (UE power step size) + 2.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 2.0dB for PUCCH with 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

1.2.	Every 5 sub-frames (5ms) transmit to the UE downlink PDSCH MAC padding bits as well as 0 dB TPC command for PUCCH via the PDCCH to make the UE transmit ACK/NACK on the PUCCH  for 1 sub-frame(1ms). The downlink transmission is scheduled in the appropriate slots to make the UE transmit PUCCH as described in figure 6.3.4.4.4.2-1

1.3.	Measure the power of 5 consecutive PUCCH transmissions to verify the UE transmitted PUCCH power is maintained within 21ms.

2.	PUSCH sub test:

2.1.	The SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send uplink power control commands for PUSCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as – (Uplink power control window size / 2) dB to + (Uplink power control window size / 2) dB of the target power level + 0 dBm, where:

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for PUSCH with 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.4.

2.2.	Every 5 sub-frames (5ms) schedule the UE's PUSCH data transmission for 1 sub-frame(1ms), and transmit 0 dB TPC command for PUSCH via the PDCCH to make the UE transmit PUSCH. The uplink transmission patterns are described in figure 6.3.4.4.4.2-1,

2.3.	Measure the power of 5 consecutive PUSCH transmissions to verify the UE transmitted PUSCH power is maintained within 21ms transmissions.

6.3.4.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.3.4.4.5	Test requirement

The requirement for the power measurements made in step (1.3) and (2.3) of the test procedure shall not exceed the values specified in Table 6.3.4.4.5-1. The power measurement period shall be 1 sub-frame(1ms).

Table 6.3.4.4.5-1: Power control tolerance

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5power measurements in the pattern, the 2nd, and later measurements shall be within ± (2.5 + TT) dB of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (3.5 + TT) dB of the 1st measurement. |
| Note 1: For SCS 30kHz 1 sub-frame corresponds to 2 slots and for SCS 60kHz 1 sub-frame corresponds to 4 slots, so 2 TPC commands will be sent for a single measurement period. Note 2: TT=0.7dB. |  |  |

<Unchanged Text Skipped>

6.3D	Output power dynamics for UL MIMO

6.3D.1	Minimum output power for UL MIMO

6.3D.1.1	Test purpose

To verify the UE's ability to transmit with a UL MIMO broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each UE antenna connector in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3D.1.3-1.

Table 6.3D.1.3-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40 | 4.515 |
| 10 | -40 | 9.375 |
| 15 | -40 | 14.235 |
| 20 | -40 | 19.095 |
| 25 | -39 | 23.955 |
| 30 | -38.2 | 28.815 |
| 40 | -37 | 38.895 |
| 45 | -36.5 | 43.575 |
| 50 | -36 | 48.615 |
| 60 | -35.2 | 58.35 |
| 70 | -34.6 | 68.07 |
| 80 | -34 | 78.15 |
| 90 | -33.5 | 88.23 |
| 100 | -33 | 98.31 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.1.

6.3D.1.4	Test description

6.3D.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.1.4.1-1.  The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3D.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.1.4.3.

6.3D.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3D.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "down" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step to ensure that the UE transmits at its minimum output power.

3.	Measure the sum of mean power of the UE at each UE antenna connector in the associated measurement channel bandwidth specified in Table 6.3D.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of 1ms over all active uplink slots and in the uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

6.3D.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.3D.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3D.1.5-1.

Table 6.3D.1.5-1: Minimum output power

| Channel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07+TT |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3D.1.5-2 |  |  |

Table 6.3D.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

6.3D.2	Transmit OFF power for UL MIMO

6.3D.2.1	Test purpose

To verify that the UE transmit OFF power for UL MIMO is lower than the value specified in the test requirement.

An excess Transmit OFF power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3D.2.2	Test applicability

The requirements of this test apply in test cases 6.3D.3 Transmit ON/OFF time mask for UL MIMO to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.2.3	Minimum conformance requirements

The transmit OFF power is defined as the mean power at each transmit connector in a duration of at least one sub-frame (1ms) excluding any transient periods.

The transmit OFF power at each transmit connector shall not exceed the values specified in Table 6.3D.2.3-1.

Table 6.3D.2.3-1: Transmit OFF power

| Channel bandwidth | (MHz) | 5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| Transmit OFF power | (dBm) | -50 |  |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.2.

6.3D.2.4	Test description

This test is covered by clause 6.3D.3 Transmit ON/OFF time mask for UL MIMO.

6.3D.2.5	Test requirement

The requirement for the transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3D.2.5-1.

Table 6.3D.2.5-1: Transmit OFF power

| Channel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3D.2.5-2 |  |  |

Table 6.3D.2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3D.3	Transmit ON/OFF time mask for UL MIMO

6.3D.3.1	Test purpose

To verify that the general ON/OFF time mask for UL MIMO meets the requirements given in 6.3D.3.5

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power as defined in sub-clause 6.3D.2 and transmit ON power symbols (transmit ON/OFF)

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3D.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.3.3	Minimum conformance requirements

For UE supporting UL MIMO, the ON/OFF time mask requirements in subclause 6.3.3.2.3 apply to each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in subclause 6.3.3.2.3 apply to each transmit antenna connector with the UL MIMO configurations specified in Table 6.3D.3.3-1.

Table 6.3D.3.3-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.3

6.3D.3.4	Test description

6.3D.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3D.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.3.4.3.

6.3D.3.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3D.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 8 for15kHz SCS, on slots 8 and 18 for 30kHz SCS and on slots 17 and 37 for 60kHz SCS. The PDCCH DCI format 0_1 is specified with the condition 2TX_UL_MIMO in 38.508-1 [5] subclause 4.3.6.1.1.2.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	On power sub test:

3.1.	Measure the sum output power at two transmit antenna connectors of the UE PUSCH transmission during one slot.

4.	OFF power sub test:

4.1.	Measure the UE transmission OFF power at each antenna connectors during the slot prior to the PUSCH transmission, excluding a transient period of 10 µs at the end of the slot.

4.2.	Measure the UE transmission OFF power at each antenna connectors during the slot following the PUSCH transmission, excluding a transient period of 10 µs at the beginning of the slot.

6.3D.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and following exceptions.

Table 6.3D.3.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[5], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 |  |  |
| } |  |  |  |

Table 6.3D.3.4.3-2: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms10 |  | FR1_15kHz |
| nrofDownlinkSlots | 6 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 14 |  | FR1_60kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSlots | 3 |  | FR1_15kHz,FR1_30kHz |
|  | 4 |  | FR1_60kHz |
| nrofUplinkSymbols | 4 |  | FR1_30kHz |
|  | 2 |  | FR1_15kHz, |
|  | 8 |  | FR1_60kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3D.3.4.3-3: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | FR1_15kHz,FR1_30kHz |
|  | 6 |  | FR1_60kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |

Table 6.3D.3.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

Table 6.3D.3.4.3-5: P-Max

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | Power class 2 or power class 1.5 |

6.3D.3.5	Test requirement

The requirement for the power measured in steps 2, 3 and 4 of the test procedure shall not exceed the values specified in Table 6.3D.3.5-1.

Table 6.3D.3.5-1: General ON/OFF time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power | Same as test ID 1 in Table 6.2D.2.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3D.3.2.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3D.3.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3D.3.5-3: Void

6.3D.4	Power control for UL MIMO

6.3D.4.1	Absolute power tolerance for UL MIMO

6.3D.4.1.1	Test purpose

To verify the ability of the UE transmitter for UL MIMO to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20ms.

6.3D.4.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.4.1.3	Minimum conformance requirements

For UE supporting UL MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in subclause 6.3.4.2 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in Table 6.3D.4.1.3-1

Table 6.3D.4.1.3-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.4.

6.3D.4.1.4	Test description

6.3D.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.4.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation (NOTE 1) |
| 1 | N/A for Absolute power tolerance test case |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3D.4.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.1.4.3. Note that PDCCH DCI format 0_1 sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3D.4.1.4.2	Test procedure

Same test procedure as clause 6.3.4.2.4.2 with following exceptions.

The power of UE PUSCH first transmissions should be measured as the sum power at each antenna connector.

6.3D.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO and following exceptions.

Table 6.3D.4.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -114 | Test point 1 to verify a UE relative low initial power transmission |  |
| } |  |  |  |

Table 6.3D.4.1.4.3-2: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-119 PUSCH-ConfigCommon |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 | Test point 2 to verify a UE relative high initial power transmission |  |
| } |  |  |  |

Table 6.3D.4.1.4.3-3: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

6.3D.4.1.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3D.4.1.5-1 and 6.3D.4.1.5-2.

Table 6.3D.4.1.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -8.3 | -7.9 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 |  | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -8.5 | -8 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3.2.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3D.4.1.5-2: Absolute power tolerance: test point 2

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -3.6 | 0.4 | 1.4 | 2.7 | 3.6 | 4.4 | 5.7 | 6.2 | 6.7 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -4.2 | -0.8 | 1.2 | 2.5 | 3.5 | 4.3 | 5.7 | 6.2 | 6.6 | 7.5 | 8.2 | 8.8 | 9.3 | 9.8 |
|  | SCS60 | N/A | -1.2 | 1 | 2.2 | 3.3 | 4.2 | 5.5 | 6 | 6.5 | 7.4 | 8.1 | 8.7 | 9.2 | 9.7 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The higher power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2.1.3 Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3.4.2.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3D.4.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.4 dB |
| 40MHz < BW ≤ 100MHz | 1.4 dB | 1.4 dB |

6.3D.4.2	Relative power tolerance for UL MIMO

6.3D.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its output power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20ms.

6.3D.4.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.4.2.3	Minimum conformance requirements

For UE supporting UL MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in subclause 6.3.4.3 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in Table 6.3D.4.2.3-1

Table 6.3D.4.2.3-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.4

6.3D.4.2.4	Test description

6.3D.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.4.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation (NOTE 1) |
| 5MHz | N/A for Relative power tolerance test case |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-1 See Table 6.3D.4.2.5-2 See Table 6.3D.4.2.5-7 |
| 10MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 15MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 20MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 25MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 30MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 40MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 45MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 50MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-3 See Table 6.3D.4.2.5-4 See Table 6.3D.4.2.5-7 |
| 60MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-5 See Table 6.3D.4.2.5-6 See Table 6.3D.4.2.5-7 |
| 70MHz |  |  | CP-OFDM QPSK | See Table 6.3.4.3.5-5 See Table 6.3.4.3.5-6 See Table 6.3.4.3.5-7 |
| 80MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-5 See Table 6.3D.4.2.5-6 See Table 6.3D.4.2.5-7 |
| 90MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-5 See Table 6.3D.4.2.5-6 See Table 6.3D.4.2.5-7 |
| 100MHz |  |  | CP-OFDM QPSK | See Table 6.3D.4.2.5-5 See Table 6.3D.4.2.5-6 See Table 6.3D.4.2.5-7 |
| Note 1: The starting resource block shall be RB# 0 |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3D.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.  Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.2.4.3.

6.3D.4.2.4.2	Test procedure

Same test procedure as clause 6.3.4.3.4.2 with following exceptions.

The power of PUSCH transmissions should be measured as the sum power at each antenna connector.

Step 1.1 in ramping up pattern sub test should be changed into following description:

1.1	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at -31.8 dBm +/- 2.7 dB.

6.3D.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.3D.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3D.4.2.4.2 should satisfy the test requirements specified in Table 6.3D.4.2.5-1 thru 6.3D.4.2.5-7.

Table 6.3D.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80+/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-5: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 81 RBs | TPC=+1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 81 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 75 RBs | TPC=+1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 75 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-6: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 81 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 81 RBs to 1 RB | TPC=-1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 75 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 75 RBs to 1 RB | TPC=-1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

Table 6.3D.4.2.5-7: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
| 5 |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40,45,50 | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 24 | TPC=0dB | 13.80 | 10dB ≤ ΔP < 15dB | 13.80 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 60 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 81 | TPC=0dB | 19.08 | 15dB < ΔP | 19.08 +/- (5 + TT) |
| 60,70,80,90,100 |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 60 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 75 | TPC=0dB | 18.75 | 15dB < ΔP | 18.75 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3.1. |  |  |  |  |  |  |  |

6.3D.4.3	Aggregate power tolerance for UL MIMO

6.3D.4.3.1	Test purpose

To verify the ability of the UE with UL MIMO to maintain its power during non-contiguous transmissions within 21ms in response to 0 dB commands with respect to the first UE transmission and all other power control parameters as specified in 38.213 kept constant.

6.3D.4.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

6.3D.4.3.3	Minimum conformance requirements

For UE supporting UL MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in subclause 6.3.4.4 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL MIMO configurations described in Table 6.3D.4.3.3-1

Table 6.3D.4.3.3-1: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Codebook Index |
| --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | Codebook index 0 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3D.4

6.3D.4.3.4	Test description

6.3D.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.4.3.4.1-1 and table 6.3D.4.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.4.3.4.1-1: Test Configuration Table: PUCCH sub-test

| Initial Conditions |  |  |
| --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |
| Test Parameters for Channel Bandwidths |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |
|  | N/A for aggregate power tolerance testcase | PUCCH format = Format 1 Length in OFDM symbols = 14 |
| 1 |  |  |

Table 6.3D.4.3.4.1-2: Test Configuration Table: PUSCH sub-test

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for aggregate power tolerance testcase | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.3D.4.3.4.1-1 (PUCCH sub-test) and Table 6.3D.4.3.4.1-2 (PUSCH sub-test)

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3D.4.3.4.3.

6.3D.4.3.4.2	Test procedure

Same test procedure as clause 6.3.4.4.4.2 with following exceptions.

The power of PDCCH /PUSCH transmissions should be measured as the sum power at each antenna connector.

6.3D.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 ensuring Table 4.6.3-182 with condition 2TX_UL_MIMO.

6.3D.4.3.5	Test requirement

The requirement for the power measurements made in step (1.3) and (2.3) of the test procedure shall not exceed the values specified in Table 6.3D.4.3.5-1. The power measurement period shall be 1 sub-frame.

Table 6.3D.4.3.5-1: Power control tolerance

| TPC commands | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (2.5dB + TT) of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (3.5dB + TT) of the 1st measurement. |
| Note 1: For SCS 30kHz 1 sub-frame corresponds to 2 slots, so 2 TPC commands will be sent for a single measurement period. For SCS 60kHz 1 sub-frame corresponds to 4 slot, so 4 TPC commands will be sent for a single measurement period. |  |  |

<Unchanged Text Skipped>

6.3G	Output power dynamics for Tx Diversity

6.3G.1	Minimum output power for Tx Diversity

6.3G.1.1	Test purpose

Same test purpose as in 6.3.1.1.

6.3G.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

The requirements of this test apply to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.1.

6.3G.1.4	Test description

Same test description as specified in clause 6.3.1.4 with following exceptions:

Step 3 of Test procedure as in 6.3.1.4.2 is replaced by:

3.	Measure the sum of mean power at each antenna connector in the associated measurement channel bandwidth specified in Table 6.3G.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3G.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3G.1.5-1.

| Table 6.3G.1.5-1: Minimum output powerChannel bandwidth (MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.1.5-2 |  |  |

Table 6.3G.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

6.3G.2	Transmit OFF power for Tx Diversity

6.3G.2.1	Test purpose

Same test purpose as in 6.3.2.1.

6.3G.2.2	Test applicability

The requirements of this test apply in test cases 6.3G.3 Transmit ON/OFF time mask for Tx Diversity to all types of NR Power Class 2 and Power Class 3 UE  release 15 and forward that support txDiversity-r16.

The requirements of this test apply in test cases 6.3G.3 Transmit ON/OFF time mask for Tx Diversity to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1 ms) excluding any transient periods.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.2.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.2.

6.3G.2.4	Test description

This test is covered by clause 6.3G.3 Transmit ON/OFF time mask.

6.3G.2.5	Test requirement

For each transmit antenna connector, the requirement for the Transmit OFF power shall not exceed the values specified in Table 6.3G.2.5-1.

| Table 6.3G.2.5-1: Transmit OFF powerChannel bandwidth (MHz) | Transmit OFF power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -50+TT | 4.515 |
| 10 | -50+TT | 9.375 |
| 15 | -50+TT | 14.235 |
| 20 | -50+TT | 19.095 |
| 25 | -50+TT | 23.955 |
| 30 | -50+TT | 28.815 |
| 40 | -50+TT | 38.895 |
| 45 | -50+TT | 43.575 |
| 50 | -50+TT | 48.615 |
| 60 | -50+TT | 58.35 |
| 70 | -50+TT | 68.07 |
| 80 | -50+TT | 78.15 |
| 90 | -50+TT | 88.23 |
| 100 | -50+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.2.5-2 |  |  |

Table 6.3G.2.5-2: Test Tolerance (Transmit OFF power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.3	Transmit ON/OFF time mask for Tx Diversity

6.3G.3.1	General ON/OFF time mask for Tx Diversity

6.3G.3.1.1	Test purpose

Same test purpose as in 6.3.3.2.1.

6.3G.3.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

This test case applies to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.3.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the general ON/OFF time mask requirements in clause 6.3.3.2.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.1.4	Test description

Same test description as in clause 6.3.3.2.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.1.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.1.5-1.

| Table 6.3G.3.1.5-1: General ON/OFF time mask | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |  |
| Transmit ON power | Same as Table 6.2G.2.5-1 and Table 6.2G.2.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.1.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in Table 6.3G.3.1.5-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3G.3.1.5-3: Test Tolerance for ON power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.3.2	PRACH time mask for Tx Diversity

6.3G.3.2.1	Test purpose

Same test purpose as in 6.3.3.4.1.

6.3G.3.2.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

This test case applies to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the PRACH time mask requirements in clause 6.3.3.4.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.2.4	Test description

Same test description as in clause 6.3.3.4.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.2.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.2.5-1.

Table 6.3G.3.2.5-1: PRACH time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Expected PRACH Transmission ON Measured Power for PRACH Format 0 and PRACH Format A3 for SCS 30kHz | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm |
| Expected PRACH Transmission ON Measured Power for PRACH Format A3 for SCS 15kHz and SCS 60kHz | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm |
| ON Power Tolerance | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.2.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in Table 6.3.3.4.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.2.5-2: Test Tolerance (Transmit OFF power and PRACH time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.3.3	SRS time mask for Tx Diversity

6.3G.3.3.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

This test case applies to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the SRS time mask requirements in clause 6.3.3.6.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.3.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.3.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.3.5-1.

Table 6.3G.3.3.5-1: SRS time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Table 6.2G.1.5-1 and Table 6.2G.1.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.3.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.3.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.4	Power control for Tx Diversity

6.3G.4.1	Absolute power tolerance for Tx Diversity

FFS

6.3G.4.2	Relative power tolerance for Tx Diversity

6.3G.4.2.1	Test purpose

Same test purpose as in 6.3.4.3.1.

6.3G.4.2.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

This test case applies to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.4.2.3	Minimum conformance requirement

For UE supporting Tx diversity, the relative power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.2.4	Test description

Same test description as in clause 6.3.4.3.4 with the output power is measured as the sum of both antenna connectors.

6.3G.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3G.4.2.4 should satisfy the test requirements specified in Table 6.3G.4.2.5-1 through 6.3G.4.2.5-7.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3G.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 50MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80+/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 50MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  |  |  |  |  |  |  |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-5: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 81 RBs | TPC=+1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 81 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 75 RBs | TPC=+1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 75 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-6: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 81 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 81 RBs to 1 RB | TPC=-1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 75 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 75 RBs to 1 RB | TPC=-1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-7: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
| 5 |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40,50 | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 24 | TPC=0dB | 13.80 | 10dB ≤ ΔP < 15dB | 13.80 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 60 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 81 | TPC=0dB | 19.08 | 15dB < ΔP | 19.08 +/- (5 + TT) |
| 60, 70,80,90,100 |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 60 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 75 | TPC=0dB | 18.75 | 15dB < ΔP | 18.75 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

6.3G.4.3	Aggregate power tolerance for Tx Diversity

6.3G.4.3.1	Test purpose

Same test purpose as in 6.3.4.4.1.

6.3G.4.3.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that support txDiversity-r16.

This test case applies to all types of NR Power Class 1.5 UE release 15 and forward.

6.3G.4.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the absolute power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.3.4	Test description

Same test description as in clause 6.3.4.4.4 with the output power is measured as the sum of both antenna connectors.

6.3G.4.3.5	Test requirement

The requirement for the power measurements made in step 1.3 and 2.3 of the test procedure shall not exceed the values specified in Table 6.3G.4.3.5-1. The power measurement period shall be 1 sub-frame(1ms).

Table 6.3G.4.3.5-1: Power control tolerance

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5power measurements in the pattern, the 2nd, and later measurements shall be within ± (2.5 + TT) dB of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (3.5 + TT) dB of the 1st measurement. |
| Note 1: For SCS 30kHz 1 sub-frame corresponds to 2 slots and for SCS 60kHz 1 sub-frame corresponds to 4 slots, so 2 TPC commands will be sent for a single measurement period. Note 2: TT=0.7dB. |  |  |

<End of Changes>