3GPP TSG-RAN5 Meeting #97	R5-226935

# Toulouse, France, 14th – 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2648 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default 5GSM messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Information Elements of some of the 5GSM messages have been updated in the core specifications. The default message definitions have to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of the PDU SESSION AUTHENTICATION COMMAND, PDU SESSION AUTHENTICATION COMPLETE and REMOTE UE REPORT RESPONSE 5GSM messages have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### –	PDU session authentication command

Table 4.7.2-4: PDU SESSION AUTHENTICATION COMMAND

| Derivation Path: 24.501 clause 8.3.4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMMAND message identity |  | ‘1100 0101’B |  |  |
| EAP message |  | Set according to TS 38.508 Table 4.7.3.2-1 | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | Not Present |  |  |

#### –	PDU session authentication complete

Table 4.7.2-5: PDU SESSION AUTHENTICATION COMPLETE

| Derivation Path: 24.501 clause 8.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | The value indicated in PDU SESSION AUTHENTICATION COMMAND message |  |  |
| PTI |  | '0000 0000'B | No procedure transaction identity assigned |  |
| PDU SESSION AUTHENTICATION COMPLETE message identity |  | ‘1100 0110’B |  |  |
| EAP message |  | Set according to TS 38.508 Table 4.7.3.2-2 | See TS 24.501 [25] subclause 9.11.2.2 |  |
| Extended protocol configuration options |  | If present: contents not checked |  |  |

##### <<< TEXT SKIPPED >>>

#### –	Remote UE report response

Table 4.7.2-20: REMOTE UE REPORT RESPONSE

| Derivation Path: 24.501 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Set according to specific message content. |  |  |
| PTI |  | Set according to specific message content. |  |  |
| REMOTE UE REPORT RESPONSE message identity |  | ‘1101 1011’B |  |  |