3GPP TSG-RAN5 Meeting #97	R5-226951

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0407 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RedCap UE capability for Legacy test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, SN length Capability is not applicable to RedCap UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Update SN length Capability for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE can not verify several L2 testcases nornally. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.3, A.4.3.4, A.4.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.4.3.3	PDCP Implementation Capabilities

Table A.4.3.3-1: UE PDCP Implementation Capabilities

| Item |  | UE PDCP Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support 12 bit length of PDCP sequence number |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_shortSN |  | Yes |  | Yes (for RedCap UE) |  | This PICS shall always be true for RedCap UE. |  |
| 1A |  | Support of 18 bit length of PDCP sequence number |  | 38.822, 4.2, 38.306, 4.2.21.3 |  | Rel-15 |  | pc_longSN |  | No |  | Yes (for non-RedCap UE) |  | This PICS shall always be true for non-RedCap UE. |  |
| 2 |  | Supports Out of order delivery of data to upper layers by PDCP |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_outOfOrderDelivery |  | No |  |  |  |  |  |
| 3 |  | Support CA-based PDCP duplication over MCG or SCG DRB |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |  | No |  |  |  |  |  |
| 4 |  | Support PDCP duplication over split DRB |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationSplitDRB |  | No |  |  |  |  |  |
| 5 |  | Support PDCP duplication with more than two RLC entities |  | 38.306, 4.2.4 |  | Rel-16 |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |
| 6 |  | Support PDCP duplication over split SRB1/2 |  | 38.306, 4.2.4 |  | Rel-15 |  | pc_pdcp_DuplicationSplitSRB |  | No |  |  |  |  |  |
| 7 |  | Support EHC (Ethernet header compression) |  | 38.306, 4.2.4 |  | Rel-16 |  | pc_NR_ehc_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |

A.4.3.4	RLC Implementation Capabilities

Table A.4.3.4-1: UE RLC Implementation Capabilities

| Item | UE RLC Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support RLC AM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_am_WithShortSN | Yes | Yes (for RedCap UE) | This PICS shall always be true for RedCap UE. |
| 1A | Support RLC AM with 18 bit length of RLC sequence number | 38.306, 4.2.21.4 | Rel-15 | pc_am_WithLongSN | No | Yes (for non-RedCap UE) | This PICS shall always be true for non-RedCap UE. |
| 2 | Support RLC UM with 12 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WIthLongSN | Yes |  |  |
| 3 | Support RLC UM with 6 bit length of RLC sequence number | 38.306, 4.2.5 | Rel-15 | pc_um_WithShortSN | Yes |  |  |

A.4.3.12	RedCap Capabilities

According to TS 38.306 [17] clause 4.2.21:

-	For a RedCap UE, PICS associated to the following features shall not be set to 'true':

-	CA: PICS defined in clause A.4.3.2A;

-	MR-DC: PICS defined in clause A.4.3.2B;

-	DAPS: PICS include pc_intraFreqDAPS_r16, interFreqDAPS_r16 and other DAPS related PICS;

-	CPAC: PICS include pc_condPSCellChange_r16 and other CPAC related PICS.

-	Features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches or more than 2 UL MIMO layers.

Table A.4.3.12-1: RedCap Capabilities

| Item | UE RedCapCapabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support of 16 DRBs for RedCap UEs. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOf16DRB_RedCap_r17 | No |  |  |
| 2 | Support of RedCap. | 38.306, 4.2.21.2 | Rel-17 | pc_supportOfRedCap_r17 | No |  | This PICS shall always be true for RedCap UE. |
| 3 | Void |  |  |  |  |  |  |
| 4 | Void |  |  |  |  |  |  |
| 5 | Support of Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE. | 38.306 4.2.21.6.1 | Rel-17 | pc_halfDuplexFDD_TypeA_RedCap_r17 | No |  |  |