3GPP TSG-RAN5 Meeting #97	R5-226995

Toulouse, France, 14th November – 18th November 2022

Title: 	TRP coverage and side conditions review for beam correspondence

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.13.8

Document for:	Discussion and Endorsement

# 1.	Introduction

In RAN5#95-e, enhanced beam correspondence core requirements part of FR2 RF Enhancement work item was added into TS 38.521-2 ([1]) via CR in [2].

In RAN5#96-e, [3] and [4] proposed (among other proposals) that TRP should be removed from 6.2.1.1_1 test title as TRP measurements were not covered. However, this proposal was not agreeable as it would leave TRP uncovered for all types of NR UEs release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence. AP#96e.24 was defined to fix this in the test specification [1].

This document makes proposals on a possible way forward to solve this issue.

# 2.	Discussion

During RAN5#96e offline discussions, a preference was shown to keep TRP coverage for all types of NR UEs release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence under test case 6.2.1.1_1 in such a way that 6.2.1.1 in [1] will cover EIRP and TRP measurements for UEs not supporting enhanced beam correspondence while 6.2.1.1-1 in [1] will cover those measurements for UEs supporting beam correspondence.

Proposal 1: Keep TRP coverage for all types of NR UEs release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence under test case 6.2.1.1_1 in TS38.521-2 [1].

As TRP measurement in test 6.2.1.1_1 requires Rel-15 side conditions as in 6.2.1.1 in [1], additional concerns were raised during offline discussions related to:

- Whether configuration of side conditions should be made explicit

- When side conditions need to be activated

Observation 1: Side conditions are not explicitly referred to in the test procedure for (enhanced) beam correspondence tests.

Side conditions have an impact in the UE performance when the device is setting the Tx beam peak direction since it will be receiving SSB/CSI-RS to complete the beam selection (before locking the beam).

In Annex K.1.1 in [1] (TX beam peak direction search), explicit configuration of side conditions is specified as:

Although test procedure does not include any step for this:

Observation 2: Side conditions in Tx beam peak direction search are specified to be configured before setting TX beam peak direction (although this configuration is not explicitly listed in the test procedure).

In test case 6.2.1.1 in [1], appropriate side conditions are applied through indirect reference to annex K.1.1:

Proposal 2: Clarify configuration of side conditions in test case 6.2.1.1 in [1] adding the following step 1a:

1a.If the UE does not support beamCorrespondenceWithoutULBeamSweeping, the side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2 respectively.

In test case 6.6.1 in [1], appropriate side conditions are applied through indirect reference to annex K.1.1:

Proposal 3: Clarify configuration of side conditions in test case 6.6.1 in [1] adding the following step 2.1a:

2.1a. The side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2 respectively.

In order to add TRP coverage to test 6.2.1.1_1 in [1] making explicit mention to side conditions configuration the following changes to test procedure are proposed:

6.2.1.1_1.4.2 Test procedure The following cases are tested depending on UE capability: - Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported and beamCorrespondenceSSB-based-r16 is supported: 1.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1 1.2 Skip to Step 7. 2. Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, and beamCorrespondenceCSI-RS-based-r16 is supported 2.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1 2.2 Skip to Step 7. 3. Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is NOT supported, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 are supported 3.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1. 3.2 Repeat 6.2.1.1.4.2 with step 6 skipped with Tx Beam Peak direction determined using the side conditions in Table 6.6.2.3.3-1 Record the verdict (as this result will not be compared to test requirements in this test case but in a different one). 3.3 Skip to Step 7. 4. Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceSSB- based-r16 is supported: 4.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1. 4.2 Skip to Step 7. 5. Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported and beamCorrespondenceCSI-RS-based-r16 is supported: 5.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.2.3.3-1 5.2 Skip to Step 7 6. Test procedure if beamCorrespondenceWithoutUL-BeamSweeping is supported, beamCorrespondenceCSI-RS-based-r16 and beamCorrespondenceSSB-based-r16 is supported 6.1 Same as 6.2.1.1.4.2 with the exception that step 6 is skipped and measurements shall be carried out using only side conditions defined in Table 6.6.1.3.3.1.1-1. 6.2 Repeat 6.2.1.1.4.2 with step 6 skipped with Tx Beam Peak direction determined using the side conditions in Table 6.6.2.3.3-1. Record the verdict (as this result will not be compared to test requirements in this test case but in a different one). 7. Set side conditions for SSB-based and CSI-RS based L1-RSRP measurements as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2. 8. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete. 9. SS activates the UE BeamlockFunction (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only. 10. Measure TRP of the transmitted signal for the assigned NR channel with a rectangular measurement filter with bandwidths according to Table 6.5.2.3.5-1. Total radiated power is measured according to TRP measurement procedure defined in Annex K.1.7 and measurement grid specified in Annex M.4. TRP is calculated considering both polarizations, theta and phi. 11. SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3. NOTE 1: The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

Proposal 4: Add TRP measurement to test case 6.2.1.1_1 making explicit mention to side conditions configuration as described above.

# 3. 	Conclusion

This document makes proposals on a possible way forward to add TRP measurements to test case 6.2.1.1_1 in TS 38.521-2 [1] and clarify side conditions for SSB-based and CSI-RS based L1-RSRP measurement wherever required.

The following proposals and observations are made:

Proposal 1: Keep TRP coverage for all types of NR UEs release 16 and forward supporting either SSB-based or CSI-RS based enhanced beam correspondence under test case 6.2.1.1_1 in TS38.521-2 [1].

Observation 1: Side conditions are not explicitly referred to in the test procedure for (enhanced) beam correspondence tests.

Observation 2: Side conditions in Tx beam peak direction search are specified to be configured before setting TX beam peak direction (although this configuration is not explicitly listed in the test procedure).

Proposal 2: Clarify configuration of side conditions in test case 6.2.1.1 in [1] adding the following step 1a:

1a.If the UE does not support beamCorrespondenceWithoutULBeamSweeping, the side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2 respectively.

Proposal 3: Clarify configuration of side conditions in test case 6.6.1 in [1] adding the following step 2.1a:

2.1a. The side conditions for SSB-based and CSI-RS based L1-RSRP measurements are applied as per Table 6.6.1.3.3.1.1-1 and Table 6.6.1.3.3.1.1-2 respectively.

Proposal 4: Add TRP measurement to test case 6.2.1.1_1 making explicit mention to side conditions configuration as described above.

These proposals are implemented in [5], [6], [7].

# 4.	References

- 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-223750, “FR2 Enhanced Beam Correspondence test updates”, Apple Portugal, RAN5#95-e

- R5-224109, “Discussion on measurement uncertainties for enhanced beam correspondence”, Keysight Technologies UK Ltd, RAN5#96-e

- R5-224110, “Enhanced Beam correspondence Measurement Uncertainties and test tolerances”, Keysight Technologies UK Ltd, RAN5#96-e

- R5-226999, “SSB-based and CSI-RS based L1-RSRP measurements side conditions clarifications in test 6.2.1.1”, Keysight Technologies UK Ltd, RAN5#97

- R5-227000, “SSB-based and CSI-RS based L1-RSRP measurements side conditions clarifications in test 6.6.1”, Keysight Technologies UK Ltd, RAN5#97

- R5-227001, “TRP measurement addition in test 6.2.1.1_1”, Keysight Technologies UK Ltd, RAN5#97