3GPP TSG-RAN5 Meeting #97 	R5-227091

# Toulouse,  France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0615 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Additional PDSCH Reference Channel TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the message exception of 7.3.2.2.3 in TS38.521-4, DCI format 2_6 transmissions affect slot 40 instead of slot 5. The muted slots must be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the muted slots in Table A.3.3.2.5-3, accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be correctly performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

A.3.3.2.5	Reference measurement channels for SCS 120 kHz FR2

Table A.3.3.2.5-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-1.1 TDD | R.PDCCH.5-1.2 TDD | R.PDCCH.5-1.3 TDD | R.PDCCH.5-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |  |  |
| CORESET frequency domain allocation |  | 60 | 60 | 60 | 60 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 40 | 56 | 56 | 12 |  |  |

Table A.3.3.2.5-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

Table A.3.3.2.5-3: Additional PDSCH Reference Channel TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| DCI format |  | DCI 1_0 | DCI 1_1 |
| TDD UL/DL pattern |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 |
| Number of consecutive PDSCH symbols |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 13 | 13 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 13 | 13 |
| Allocated slots per 2 frames |  | 126 | 126 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 12 | 12 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 12 | 12 |
| Overhead for TBS determination |  | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3104 | 3624 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 4480 | 5504 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 4480 | 5504 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 | 16 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 24 | 24 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 | 1 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | CBs | 1 | 1 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0, 40 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 10296 | 11880 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 15048 | 18216 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 15048 | 18216 |
| Max. Throughput averaged over 2 frames | Mbps | 26,022 | 31.667 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames |  |  |  |

<<End of change>>