3GPP TSG-RAN5 Meeting #97	R5-227133

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3381 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Broadcast TC 14.1.2.3-MBS Interest Indication-intrafreq |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.1.2.3 is not completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.1.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MBS TC 14.1.2.3 can’t be executed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.2.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2,TS 38.508-1 CR 0287,2660 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 'NR-XX2'and 'NR-XX4' shall be resolved according to R5-227138 Table 4.4.3.1.2-1 of TS 38.508-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.2.3	MBS Broadcast/ Service Continuity/ Handover/ MBS Interest Indication/ intra-frequency

14.1.2.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and is receiving MBS broadcast service and has not transmitted a MBSInterestIndication message }

ensure that {

when { SIB21 provided in the serving cell indicates that the MBS service is available on a frequency of intra-frequency neighbour cell and the FSAI of this frequency is also indicated in the USD for this session }

then { UE transmits a MBSInterestIndication message indicating interest in MBS reception on the frequency }

}

(2)

with { UE in NR RRC_CONNECTED state and having transmitted a MBSInterestIndication message indicating interest in MBS reception on a frequency of intra-frequency neighbour cell }

ensure that {

when { 1s after the UE has transmitted the MBSInterestIndication message the UE receives RRCReconfiguration message including a reconfigurationWithSync indicating a the NR frequency of the intra-frequency neighbour cell }

then { UE performs intra-frequency handover and continues to receive MBS reception }

}

(3)

with { UE in NR RRC_CONNECTED state and having transmitted a MBSInterestIndication message }

ensure that {

when { Since the last time the UE transmitted an MBS Interest Indication, UE handovers from a cell not providing SIB21 to a cell providing SIB21 }

then { UE transmits a MBSInterestIndication message }

}

(4)

with { UE in NR RRC_CONNECTED state and having transmitted a MBSInterestIndication message }

ensure that {

when { The set of MBS broadcast services of interest is different from mbs-ServiceList included in the last transmission of the MBS Interest Indication }

then { UE transmits a MBSInterestIndication message indicating new mbs-ServiceList }

}

14.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.6.5.2; TS 38.331, clauses 5.9.4.2, 5.9.4.3, 5.9.4.4 and 5.9.4.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.6.5.2]

To ensure service continuity of MBS broadcast, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, consisting of the following information:

-	List of MBS frequencies UE is interested to receive, sorted in decreasing order of interest;

-	Priority between the reception of all listed MBS frequencies and the reception of any unicast bearer;

-	List of MBS broadcast services the UE is interested to receive, in case SIB20 is scheduled by the UE's PCell.

MBS Interest Indication information reporting can be implicitly enabled/disabled by the presence of SIB21.

The gNB may use this information, together with the information about the UE's capabilities (e.g., supported band combinations), when providing an RRC configuration and/or downlink assignments to the UE, to allow the UE to receive the MBS services the UE is interested in. MBS Interest Indication information can be exchanged between source gNB and target gNB during handover.

[TS 38.331, clause 5.9.4.2]

Upon initiating the procedure, the UE shall:

1>	if SIB21 is provided by the PCell:

2>	ensure having a valid version of SIB21 for the PCell;

2>	if the UE did not transmit MBS Interest Indication since last entering RRC_CONNECTED state; or

2>	if since the last time the UE transmitted an MBS Interest Indication, the UE connected to a PCell not providing SIB21:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

2>	else:

3>	if the set of MBS broadcast frequencies of interest, determined in accordance with 5.9.4.3, is different from mbs-FreqList included in the last transmission of the MBS Interest Indication; or

…

4>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate transmission of the MBSInterestIndication message;

NOTE:	The UE may send MBS Interest Indication even when it is able to receive the MBS services it is interested in i.e. to avoid that the network allocates a configuration inhibiting MBS broadcast reception.

3>	else if SIB20 is provided for the PCell or for the SCell:

4>	if since the last time the UE transmitted the MBS Interest Indication, the UE connected to a PCell not providing SIB20 and the UE was not provided with SIB20 for an SCell; or

4>	if the set of MBS broadcast services of interest determined in accordance with 5.9.4.4 is different from mbs-ServiceList included in the last transmission of the MBS Interest Indication:

5>	set the contents of MBS Interest Indication according to 5.9.4.5 and initiate the transmission of MBSInterestIndication message.

[TS 38.331, clause 5.9.4.3]

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.

[TS 38.331, clause 5.9.4.4]

The UE shall:

1>	consider an MBS service to be part of the MBS services of interest if the following conditions are met:

2>	the UE is receiving or interested to receive this service via a broadcast MRB; and

2>	the session of this service is ongoing or about to start; and

2>	one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service but that frequency is included in the USD of this service.

NOTE:	The UE may determine whether the session is ongoing from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

[TS 38.331, clause 5.9.4.5]

The UE shall set the contents of the MBS Interest Indication as follows:

1>	if the set of MBS frequencies of interest, determined in accordance with 5.9.4.3, is not empty:

2>	include mbs-FreqList and set it to include the MBS frequencies of interest sorted by decreasing order of interest, using the absoluteFrequencySSB for serving frequency, if applicable, and the ARFCN-ValueNR(s) as included in SIB21 or in USD (for neighbouring frequencies);

…

2>	if SIB20 is provided for the PCell or for the SCell:

3>	include mbs-ServiceList and set it to indicate the set of MBS services of interest sorted by decreasing order of interest determined in accordance with 5.9.4.4.

14.1.2.3.3	Test description

14.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) and NR Cell 2 (TAI-1).

-	The SS configures the NR Cell 1 as the "Serving cell" and NR Cell 2 as "Non-suitable "Off" cell".

-	System information combination NR-XX2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

-	System information combination NR-XX2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 2.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-3 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.1.2.3.3.2	Test procedure sequence

Table 14.1.2.3.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T0", and "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 14.1.2.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 2 (measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 1(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 2 (measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell 1(measId 1): Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |

Table 14.1.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is made interested in receiving a MBS service with MBS Service ID=1 associated with the MBS FSAI 1 (Note 1). | - | - | - | - |
| 2 | The UE is made aware that the MBS Service ID=1 is ongoing (Note 1). | - | - | - | - |
| 3 | The SS starts to broadcast SIB21 (according to System information combination NR-XX4) as defined in TS 38.508-1 [4] clause 4.4.3.1.2) on NR Cell 1 including mbs-FSAI-IntraFreqList-r17 indicating MBS-FSAI-r17=1. | - | - | - | - |
| 4 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification on NR Cell 1. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 5 | Check: Does the UE transmit MBSInterestIndication message. | --> | MBSInterestIndication | 1 | P |
| 6 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 7 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| 8a1-8a2 | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 9 is repeated 5 times | - | - | - | - |
| 9 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 10 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 11 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 12 | Check: Is the number of reported MBS Packets received on the MTCH in step 11 greater than zero? | - | - | 2 | P |
| 13 | The SS waits for 1s. | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message to setup intra frequency measurement on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1 to confirm the setup of intra- frequency measurement. | --> | RRCReconfigurationComplete | - | - |
| 16 | The SS changes NR Cell 1 and NR Cell 2 level according to the row "T1" in table 14.1.2.3.3.2-1/2. | - | - | - | - |
| 17 | The UE transmits a MeasurementReport message to report event A3 on NR Cell 1 with the measured RSRP, RSRQ value for NR Cell 2. | --> | MeasurementReport | - | - |
| 18 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform intra-frequency handover to NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 19 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2? | --> | RRCReconfigurationComplete | - | - |
| 20 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 21 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. | - | - | - | - |
| - | Exception: Step 22 is repeated 5 times | - | - | - | - |
| 22 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 23 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 24 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 25 | Check: Is the number of reported MBS Packets received on the MTCH in step 24 greater than the number of reported in step 11? | - | - | 2 | P |
| 26 | The SS transmits an RRCReconfiguration message to setup intra-frequency measurement on NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 27 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2 to confirm the setup of intra-frequency measurement. | --> | RRCReconfigurationComplete | - | - |
| 28 | The SS changes NR Cell 1 and NR Cell 2 level according to the row "T2" in table 14.1.2.3.3.2-1/2. | - | - | - | - |
| 29 | The UE transmits a MeasurementReport message to report event A3 on NR Cell 2 with the measured RSRP, RSRQ value for NR Cell 1. | --> | MeasurementReport | - | - |
| 30 | The SS transmits an RRCReconfiguration message on NR Cell 2 to order the UE to perform intra-frequency handover to NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 31 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1? | --> | RRCReconfigurationComplete | - | - |
| 32 | UE transmits an MBSInterestIndication message on NR Cell 1 | --> | MBSInterestIndication | 3 | P |
| 33 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 34 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. | - | - | - | - |
| - | Exception: Step 35 is repeated 5 times | - | - | - | - |
| 35 | The SS transmits a MBS Packet on the MTCH with LCID=1. | <-- | MBS Packet | - | - |
| 36 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 37 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 38 | Check: Is the number of reported MBS Packets received on the MTCH in step 37 greater than the number of reported in step 25? | - | - | 2 | P |
| 39 | The UE is made secondly interested in receiving a MBS service with MBS Service ID=2 associated with the MBS FSAI 1 (Note 1). | - | - | - | - |
| 40 | The UE is made aware that the MBS Service ID=2 is ongoing (Note 1). | - | - | - | - |
| 41 | UE transmits an MBSInterestIndication message on NR Cell 1 to update the mbs-ServiceList | --> | MBSInterestIndication | 4 | P |
| Note 1: The request may be performed by MMI or AT command. |  |  |  |  |  |

14.1.2.3.3.3	Specific message contents

Table 14.1.2.3.3.3-1: SIB1 of NR Cell 1 and NR Cell 2 (preamble and all steps, Table 14.1.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.2.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  | 2 entries |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  | NR Cell 2 OR (preamble AND NR Cell 1) |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| SchedulingInfo2-r17 [2] |  | Not present |  | NR Cell 2 |
| SchedulingInfo2-r17 [2] SEQUENCE { |  |  | entry 2 | Step3 AND NR Cell 1 |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 3 | entry number for si-SchedulingInfo in SIB1 +2 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType21 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.3.3.3-2: ServingCellConfigCommonSIB (Table 14.1.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.2.3.3.3-3 |  |
| } |  |  |  |

Table 14.1.2.3.3.3-3: DownlinkConfigCommonSIB (Table 14.1.2.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.2.3.3.3-4 |  |
| } |  |  |  |

Table 14.1.2.3.3.3-4: BWP-DownlinkCommon (Table 14.1.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditioni MBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.3.3.3-5: SIB21 of NR Cell 1 (step 3, Table 14.1.2.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB21-r17 ::= SEQUENCE { |  |  |  |
| mbs-FSAI-IntraFreq-r17 SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 { | 1 entry |  |  |
| MBS-FSAI-r17[1] | ‘000001’H | entry 1 OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| mbs-FSAI-InterFreqList-r17 | Not present |  |  |
| } |  |  |  |

Table 14.1.2.3.3.3-6: ACTIVATE TEST MODE (preamble, Table 14.1.2.3.3.2-3)

Derivation Path: 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.2.3.3.3-7: RRCReconfiguration (step 14 and step26, Table 14.1.2.3.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition MEAS

Table 14.1.2.3.3.3-8: MeasConfig (Table 14.1.2.3.3.3-7)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 14.1.2.3.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 14.1.2.3.3.3-9: ReportConfigNR-EventA3 (Table 14.1.2.3.3.3-8)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  | EVENT_A3 |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 6 | 3dB | FR1 |
|  | FFS |  | FR2 |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms640 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.3.3.3-10: RRCReconfiguration (step 18 and step 30, Table 14.1.2.3.3.2-3)

Derivation path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange

Table 14.1.2.3.3.3-11: CLOSE UE TEST LOOP (step 8a1, Table 14.1.2.3.3.2-3)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Broadcast MRB

Table 14.1.2.3.3.3-12: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 10, step 23 and step 36, Table 14.1.2.3.3.2-3)

Derivation Path: 36.508 [6], Table 4.7A-9

<End of modified section 1>