3GPP TSG-RAN5 Meeting #97	R5-227146

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2665 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of DNN configuration for PDU session associated with MBS Multicast session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 23.247, DNN/S-NSSAI for PDU session associated with Multicast MBS session can be pre-configured by UE. Therefore, DNN is added for MBS multicast. [23.247 7.2.1.2] The PDU Session associated with Multicast MBS session(s) (i.e. the associated PDU Session) is established using the procedures as specified in TS 23.502 [6] clause 4.3.2.2 with the following differences: NOTE 1: The DNN and S-NSSAI are used to establish the PDU session which can carry the operations related to Multicast MBS session(s), i.e. session join/leave, and can be associated with multicast MBS session(s). - In step 2, the AMF selects an SMF capable of handling Multicast MBS sessions based on DNN and S-NSSAI, locally configured data or a corresponding SMF profile stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling Multicast MBS sessions. [23.247 6.11] If the MBS Session is multicast, the Service Announcement may include the DNN and S-NSSAI of the PDU Session to indicate which PDU Session is associated with the MBS Session. NOTE 2: For multicast, AF or MBSF provides Service Announcement only after the MBS information is available to 5GC or the start time need be included, to avoid potential rejection sent by SMF of the MBS session join request. NOTE 3: The MBS Service related information, e.g. default PLMN ID, DNN and S-NSSAI can be pre-configured in the UE. NOTE 4: If DNN and S-NSSAI information is not provided in the service announcement or pre-configured, how UE determines the PDU session to join the MBS Session is implementation specific. NOTE 5: How the UE can get the Service Announcement from other entities is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add DNN configuration for MBS multicast - Add Qos rule#9 and QoS flow #8 for MBS DNN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DNN configuration for MBS multicast is not specified in the 38.508-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.508-2 CR 0411 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.8.4	DNN/APN configurations

The present subclause provides DNN/APN configurations required for flexible PDU/PDN handling. Table 4.8.4-1 provides configurations for the types on DNN/APN handled in the present version of the test specification. If in the future new PDU types need to be handled, then new DNN/APN configuration(s) may be added.

Table 4.8.4-1: DNN/APN configurations, first set

| Configurations | Config #1 | Config #2 | Config #3 |
| --- | --- | --- | --- |
| DNN/APN type | internet | ims | urllc |
| DNN/APN ID | pc_APN_ID_Internet | pc_APN_ID_IMS | pc_APN_ID_URLLC |
| 5GC QoS rule | Reference QoS rule #1 as specified in subclause 4.8.2.1. | Reference QoS rule #2 as specified in subclause 4.8.2.1. | Reference QoS rule #8 as specified in subclause 4.8.2.1. |
| EPC default bearer context | Reference default EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.1-1. | Reference default EPS bearer context #2 as specified in TS 36.508 [10], Table 6.6.1-1. | N/A |
| EPC dedicated bearer context | Reference dedicated EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.2-1. | Reference dedicated EPS bearer context #4 as specified in TS 36.508 [10], Table 6.6.2-1. | N/A |
| IP address allocation | Yes | Yes | Yes |
| IMS registration | No | Yes NOTE 1 | No |
| SST condition (NOTE 2) | SST_eMBB | SST_eMBB | SST_URLLC |
| NOTE 1: For PDN establishment the Procedure for IMS signalling according to TS 36.508 [2], subclause 4.5A.3 applies; for PDU establishment the Procedure for IMS signalling according to TS 34.229-5 [47], Annex A.2 applies. NOTE 2: The possible values of the SST condition are defined in Table 4.6.0.1-5. |  |  |  |

Table 4.8.4-2: DNN/APN configurations, second set

| Configurations | Config #4 | Config #5 | Config #6 |
| --- | --- | --- | --- |
| DNN/APN type | miot | v2x | mbs |
| DNN/APN ID | pc_APN_ID_MIoT | pc_APN_ID_V2X | pc_APN_ID_MBS |
| 5GC QoS rule | FFS | FFS | Reference QoS rule #9 as specified in subclause 4.8.2.1. |
| EPC default bearer context | N/A | N/A | N/A |
| EPC dedicated bearer context | N/A | N/A | N/A |
| IP address allocation | Yes | Yes | Yes |
| IMS registration | No | No | No |
| SST condition (NOTE 1) | SST_MIoT | SST_V2X | SST_eMBB |
| NOTE 1: The possible values of the SST condition are defined in Table 4.6.0.1-5. |  |  |  |

<End of modified section 1>

<Start of modified section 2>

Table 4.8.2.1-XX: Reference QoS rule #9

| Derivation Path: TS 24.501, table 9.11.4.13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| QoS rule identifier | ‘0000 1000’B | 8 (unique per PDU session) |  |
| Rule operation code | ‘001’B | Create new QoS rule |  |
| DQR bit | ‘1’B | The QoS rule is the default QoS rule. |  |
| Number of packet filters | ‘0001’B | 1 packet filter |  |
| Packet filter list | See table 4.8.2.2-1 | Packet filter list #1 |  |
| QoS rule precedence | ‘1111 1111’B | 255 (unique per PDU session; If the default QoS rule contains a match-all packet filter, then the highest precedence value shall be used for the default QoS rule.) |  |
| Spare bit | ‘0’B |  |  |
| Segregation | ‘0’B | Spare |  |
| QoS flow identifier (QFI) | ’00 1100’B | QFI 10 (Table 4.8.2.3-YY) |  |

<End of modified section 2>

<Start of modified section 3>

Table 4.8.2.3-YY: Reference QoS flow #8

| Derivation Path: TS 24.501, table 9.11.4.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 1001’B | QFI 10 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0101 0010’B | 5QI 9 |  |

<Start of modified section 3>