3GPP TSG-RAN5 Meeting #97	R5-227158

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3392 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RedCap TC 7.1.1.1.17-Msg1-based UE identification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - featureCombinationPreamblesList-r17 could directly configured in rach-ConfigCommon. It is not needed to configure it in additionalRACH-ConfigList-r17-> rach-ConfigCommon for this test case because other rach parameter does not need to change. - ssb-perRACH-OccasionAndCB-PreamblesPerSSB in Table 7.1.1.1.17.3.3-5 does not need a specific value for this test case.The Common value in 38.508-1 is good. - RACH resource for RedCap UE Msg1-based UE identification could be configuered in legacy initialUplinkBWP or initialUplinkBWP-RedCap. Test case does not cover the second configuration - According to 38.300, “a RedCap UE is identified by the dedicated LCID(s) indicated for CCCH identification (CCCH or CCCH1) regardless whether RedCap specific Random Access configuration is configured by the network.” Therefore, the LCID in step7 should also be set to 35. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Directly configured featureCombinationPreamblesList-r17 in rach-ConfigCommon and delete the redundent configuration in 7.1.1.1.17.3.3-4. - Delete the ssb-perRACH-OccasionAndCB-PreamblesPerSSB in Table 7.1.1.1.17.3.3-5 and directly use the common value in 38.508-1. - Add the step12-20 to test that RACH resource for RedCap UE configured in initialUplinkBWP-RedCap and not configured in initialUplinkBWP, and RedCap UE could use the RACH resource for RedCap UE in initialUplinkBWP-RedCap to perform random access. - Add the discription in step7 to clarify that RedCap UE should use LCID 35 in CCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration is not correct and does not cover the senario of RACH resource for RedCap UE in initialUplinkBWP-RedCap |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.1.17	Random access procedure / RedCap UE identification / Msg1-based

7.1.1.1.17.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { UE starts Random Access procedure  }

then { the UE uses the RedCap specific random access resource in Msg1}

}

7.1.1.1.17.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.1.1, 5.1.1b, 5.1.1c and 5.1.1d, TS38.300, clauses 16.13.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1]

…

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

…

-	featurePriorities: priorities for features, such as REDCAP, Slice group(s), etc. (see clause 5.1.1d);

…

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

…

-	numberOfPreamblesForThisPartition: the number of consequtive preambles associated with the set of Random Access Resources applicable to the Random Access procedure;

…

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

…

1>	perform the BWP operation as specified in clause 5.15;

1>	select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

…

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

…

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including REDCAP and/or a specific slice group(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

Editor's Note: FFS if some clarification is needed on how feature applicability is known (e.g. from RRC etc)

2>	if none of the sets of Random Access resources are available for the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there are one or more set(s) of Random Access resources available (as specified in clause 5.1.1c) and one of these set(s) of Random Access resources can be used for indicating all features triggering this Random Access procedure:

3>	select the available set of Random Access resources for this Random Access procedure.

…

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if REDCAP indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a RACH procedure for which REDCAP indication is not applicable.

…

[TS 38.321, clause 5.1.1d]

The MAC entity shall:

1>	among the available sets of Random Access resources, identify those configured with a feature which has the highest priority assigned in featurePriorities among all the features applicable to this Random Access procedure as specified in TS 38.331 [5].

1>	if a single set of Random Access resources is identified:

2>	select this set of Random Access resources.

…

[TS 38.300, clause 16.13.3]

A RedCap UE can be identified by the network during Random Access procedure via MSG3/MSGA from a RedCap specific LCID(s) and optionally via MSG1/MSGA (PRACH occasion or PRACH preamble). For RedCap UE identification via MSG1/MSGA, RedCap specific Random Access configuration may be configured by the network. For MSG3/MSGA, a RedCap UE is identified by the dedicated LCID(s) indicated for CCCH identification (CCCH or CCCH1) regardless whether RedCap specific Random Access configuration is configured by the network.

7.1.1.1.17.3	Test description

7.1.1.1.17.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

7.1.1.1.17.3.2	Test procedure sequence

Table 7.1.1.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP. | - | - | - | - |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message including a matched identity. | <-- | - | - | - |
| 5 | Check: Does the UE transmits Preamble on PRACH with ra-PreambleIndex range from 8 to 11 on NR Cell 1? | --> | PRACH Preamble | 1 | P |
| 6 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 5. | <-- | Random Access Response | - |  |
| 7 | The UE transmits a MAC PDU containing an RRCSetupRequest message by setting LCID to 35? | --> | MAC PDU (RRCSetupRequest) | - | - |
| 8 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing an RRCSetup message and ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (RRCSetup and UE Contention Resolution Identity MAC CE) | - | - |
| 9 | UE transmit a MAC PDU containing an RRCSetupComplete message indicating acceptance of RRCSetup message | --> | MAC PDU (RRCSetupComplete) | - | - |
| 10 | The test steps 5 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 11 | The SS transmits RRCRelease. | <-- | RRCRelease | - |  |
| 12 | The SS changes the SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP-RedCap. | - | - | - | - |
| 13 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 14 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 15 | The SS transmits a Paging message including a matched identity. | <-- | - | - | - |
| 16 | Check: Does the UE transmits Preamble on PRACH with ra-PreambleIndex range from 12 to 15 on NR Cell 1? | --> | PRACH Preamble | 1 | P |
| 17 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16. | <-- | Random Access Response | - |  |
| 18 | The UE transmits a MAC PDU containing an RRCSetupRequest message by setting LCID to 35? | --> | MAC PDU (RRCSetupRequest) | - | - |
| 19 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing an RRCSetup message and ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (RRCSetup and UE Contention Resolution Identity MAC CE) | - | - |
| 20 | UE transmit a MAC PDU containing an RRCSetupComplete message indicating acceptance of RRCSetup message | --> | MAC PDU (RRCSetupComplete) | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.1.17.3.3	Specific message contents

Table 7.1.1.1.17.3.3-1: SIB1 (step 1 and step13, Table 7.1.1.1.17.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.1.17.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| featurePriorities-r17 SEQUENCE { |  |  |  |  |
| redCapPriority-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.17.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.1.17.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 7.1.1.1.17.3.3-3 | Step1 |
| uplinkConfigCommon-v1700 SEQUENCE { |  |  | Step13 |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon | Table 7.1.1.1.17.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-3: UplinkConfigCommonSIB (Table 7.1.1.1.17.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.1.17.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-4: BWP-UplinkCommon (Table 7.1.1.1.17.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.1.17.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-5: RACH-ConfigCommon (Table 7.1.1.1.17.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] SEQUENCE { |  | entry 1 |  |
| featureCombination-r17 SEQUENCE { |  |  |  |
| redCap-r17 | True |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 |  | Step 1 |
|  | 12 |  | Step 13 |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>