3GPP TSG-RAN5 Meeting #97	 R5-227174

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2671 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PC5 message for LINK IDENTIFIER UPDATE procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.Source layer-2 ID/target layer-2 ID/source user info/ target user info for the DIRECT LINK IDENTIFIER UPDATE REQUESET/ACCEPT/ACK message does not consider Tx and Rx condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Tx and Rx condition for the Source layer-2 ID/target layer-2 ID/source user info/ target user info for the DIRECT LINK IDENTIFIER UPDATE REQUESET/ACCEPT/ACK message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

–	DIRECT LINK IDENTIFIER UPDATE REQUEST

Table 4.7.4-23: DIRECT LINK IDENTIFIER UPDATE REQUEST

| Derivation Path: TS 24.587 [54] Table 7.3.18.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE REQUEST message identity |  | '0001 0011'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| MSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Source layer-2 ID |  | Not checked |  | Tx |
|  |  | '00 00 10'H | New Layer-2 ID in initiating UE side | Rx |
| Source user info |  | Not checked |  | Tx |
| Source user info |  |  |  | Rx |
| Application layer ID IEI |  | '57'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H | New application Layer ID in initiating UE side |  |
| Source link local IPv6 address |  | Not present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

–	DIRECT LINK IDENTIFIER UPDATE ACCEPT

Table 4.7.4-24: DIRECT LINK IDENTIFIER UPDATE ACCEPT

| Derivation Path: TS 24.587 [54] Table 7.3.19.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE ACCEPT message identity |  | '0001 0100'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| MSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Source layer-2 ID |  | Not checked |  | Tx |
|  |  | '00 00 20'H |  | Rx |
| Target layer-2 ID |  | '00 00 10'H |  | Tx |
|  |  | Same as the source layer-2 ID in DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition |  | Rx |
| Target user info |  |  |  | Tx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 03 00'H | New application Layer ID in initiating UE side |  |
| Target user info |  | Not Present |  | Rx AND (NOT Update_user_info) |
| Target user info |  |  |  | Rx AND Update_user_info |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | Same as the source user info in DIRECT LINK IDENTIFIER UPDATE REQUEST message with Tx condition | New application Layer ID in initiating UE side |  |
| Target link local IPv6 address |  | Not Present |  |  |
| Source user info |  | Not checked |  | Tx |
| Source user info |  |  |  | Rx |
| Application layer ID IEI |  | '57'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H | New application Layer ID in target UE side |  |
| Source link local IPv6 address |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the DIRECT LINK IDENTIFIER UPDATE REQUEST. |

–	DIRECT LINK IDENTIFIER UPDATE ACK

Table 4.7.4-25: DIRECT LINK IDENTIFIER UPDATE ACK

| Derivation Path: TS 24.587 [54] Table 7.3.20.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE ACK message identity |  | '0001 0101'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| LSB of KNRP-sess ID |  | Not checked |  | Tx |
|  |  | '00'H |  | Rx |
| Target layer-2 ID |  | '00 00 20'H |  | Tx |
|  |  | Same as the source layer-2 ID in DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition |  | Rx |
| Target user info |  |  |  | Tx |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | '00 00 04 00'H | New application Layer ID in target UE side |  |
| Target user info |  | Not Present |  | Rx AND (NOT Update_user_info) |
| Target user info |  |  |  | Rx AND Update_user_info |
| Application layer ID IEI |  | '28'H |  |  |
| Length of Application layer ID contents |  | '04'H |  |  |
| Application Layer ID 1 |  | Same as the source user info in DIRECT LINK IDENTIFIER UPDATE ACCEPT message with Tx condition | New application Layer ID in target UE side |  |
| Target link local IPv6 address |  | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |
| Update_user_info | NR-SS-UE receives the source user info in the DIRECT LINK IDENTIFIER UPDATE ACCEPT |

–	DIRECT LINK IDENTIFIER UPDATE REJECT

Table 4.7.4-26: DIRECT LINK IDENTIFIER UPDATE REJECT

| Derivation Path: TS 24.587 [54] Table 7.3.21.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DIRECT LINK IDENTIFIER UPDATE REJECT message identity |  | '0001 0110'B |  |  |
| Sequence number |  | Not Checked | 0~255, uniquely identify a PC5 signalling message being sent or received | Tx |
|  |  | Incremented by TTCN by 1 for each outgoing new PC5 Signalling message. | 0~255, uniquely identify a PC5 signalling message being sent or received | Rx |
| PC5 signalling protocol cause |  | '0110 1111'B | Protocol error, unspecified |  |

| Condition | Explanation |
| --- | --- |
| Tx | UE transmits and NR-SS-UE receives |
| Rx | UE receives and NR-SS-UE transmits |

<End of Changed Section>