3GPP TSG-RAN5 Meeting #97	R5-227236

# Toulouse, France, 14th Nov 2022 – 18th Nov 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5166 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EIEI test case 11.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Test case 11.3.6 checks that the UE is able to place a manual ecall over IMS when it is in eCALL-INACTIVE.LIMITED-SERVICE state. However, current implementation refers to generic procedure of TS 36.508 subclause 4.5A.5.3-1 which refers to Steps 1-5 of C.22 from TS 34.229-1. This is for normal emergency call establishment. - After eCall and emergency bearers are released in limited service, the UE may perform a detach procedure in order to gain normal services. This is not handled in the current prose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified the test procedure sequence to refer to C.47 of TS 34.229-1 instead of C.22. - Added handling of optional detach procedure before RRC Connection release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 11.3.6	eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted

#### 11.3.6.1	Test Purpose (TP)

(1)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make an eCall to URI for test service}

then { UE does not transmit any RRCConnectionRequest message }

}

(2)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make a manual eCall}

then { UE transmits an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency’ }

}

#### 11.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clauses 2

[TS 23.122 clause 2]

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS. The signalling procedure for the release of the NAS signalling connection is initiated by the network.

#### 11.3.6.3	Test description

11.3.6.3.1	Pre-test conditions

System Simulator:

-	Cell A

-	The PLMN is defined in Table 11.3.6.3.1-1.

Table 11.3.6.3.1–1: PLMN identifier

| Cell | PLMN name | MCC / MNC |
| --- | --- | --- |
| A | PLMN4 | 004 / 31 |

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

-	The UE is equipped with a USIM containing values shown in Table 11.3.6.3.1-2.

Table 11.3.6.3.1-2: USIM configuration

| USIM field | Value |
| --- | --- |
| EFFPLMN | PLMN4 |

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

11.3.6.3.2	Test procedure sequence

Table 11.3.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on Cell A as an acceptable cell. | - | - | - | - |
| 3 | An eCall to URI for test service is initiated. (Note 1) | - | - | - | - |
| 4 | Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds? | --> | RRC: RRCConnectionRequest | 1 | F |
| 5 | A manual eCall is initiated. (Note 2) | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message with 'establishmentCause' set to 'emergency'? | --> | RRC: RRCConnectionRequest | - | - |
| 7 | SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message to attach for emergency bearer services, together with a PDN CONNECTIVITY REQUEST message with 'Request type' set to 'emergency? | --> | NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST | 2 | P |
| 9-18 | Steps 5 to 14 of the generic test procedure for IMS Emergency call establishment in EUTRA: Limited Service (TS 36.508 subclause 4.5A.5.3-1). | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 19 to 23 below, the behaviour in table 11.3.6.2-2 occurs. | - | - | - | - |
| 19-23 | Steps 15 to 19 of the generic test procedure for IMS emergency call establishment in EUTRA: Limited Service (subclause 4.5A.5.3-1) | - | - | - | - |
| 24-27 | Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33. | - | - | - | - |
| - | EXCEPTION: Step 27Aa1 and 27Aa2 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 27Aa1 | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “EPS detach” to regain normal service. | --> | DETACH REQUEST | - | - |
| 27Aa2 | The SS responds the DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 28 | Void | - | - | - | - |
| 29 | The SS releases the RRC connection after 5secs of Step 27. | <-- | RRCConnectionRelease | - | - |
| Note 1: The request to originate an eCall to URI for test service may be performed by MMI or AT command. Note 2: The request to originate a manual eCall may be performed by MMI or AT command. |  |  |  |  |  |

Table 11.3.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Step 1 describes behaviour that depends on the UE behaviour. | - | - |
| 1 | If initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 2-8 | Steps 1-7 defined in annex C.47 of TS 34.229-1 [35] Generic procedure for NG eCall Setup. (See Note 1) | - | - |
| Note 1: Content of INVITE request as specified in TS 34.229-1 [35] Step-1 of Annex C.47 using conditions A6 and A20 |  |  |  |

11.3.6.3.3	Specific message contents

Table 11.3.6.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

<End of modifed section>