3GPP TSG-RAN5 Meeting #97	R5-227240

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3409 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case 8.1.3.4.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the WP in R5-225103 test case 8.1.3.4.2.1.1 “RLM and BFD relaxation / RLM / RRC Re-establishment”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 8.1.3.4.2.1.1 “RLM and BFD relaxation / RLM / RRC Re-establishment”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test coverage will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.4.2.1.1 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0291, TS 38.508-2 CR 0413, TS 38.509 CR 0076 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.1.3.4.2	RLM and BFD relaxation

##### 8.1.3.4.2.1	RLM and BFD relaxation / RLM / RRC Re-establishment

8.1.3.4.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and having been configured with goodServingCellEvaluationRLM, lowMobilityEvaluationConnected and rlm-RelaxationReportingConfig }

ensure that {

when {  UE detects that the goodServingCellEvaluationRLM and lowMobilityEvaluationConnected criteria are both met for the first time }

then { UE initiates the UE assistance information procedure to inform the network that its relaxation state for RLM measurements for cell group has changed }

}

(2)

with { UE in NR RRC_CONNECTED state configured with lowMobilityEvaluationConnected and rlm-RelaxationReportingConfig and T346j is running }

ensure that {

when { UE detects that the lowMobilityEvaluationConnected criteria has changed}

then { UE does not initiates the UE assistance information procedure to inform the network that its relaxation state for RLM measurements for cell group has changed }

}

(3)

with { UE has been configured with rlm-RelaxationReportingConfig }

ensure that {

when { UE re-selects to a new cell while T311 is running}

then { UE releases  rlm-RelaxationReportingConfig for MCG}

}

8.1.3.4.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clause 5.7.13.

[TS 38.331 clause 5.7.13]

In case both low mobility criterion and good serving cell criterion are configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when both relaxed measurement criterion for low mobility and relaxed measurement criterion for good serving cell quality are met.

In case only the good serving cell quality criterion is configured for RLM/BFD relaxation, the UE is allowed to perform RLM and/or BFD relaxation according to requirements specified in TS 38.133 [14] when the relaxed measurement criterion for good serving cell quality is met.

...

The relaxed measurement criterion for UE with low mobility in RRC_CONNECTED is fulfilled when:

-	(SS-RSRPRef – SS-RSRP) < SSearchDeltaP-Connected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the SpCell based on SSB (dB).

-	SS-RSRPRef = reference L3 RSRP measurement of the SpCell based on SSB (dB), set as follows:

-	After receiving low mobility criterion configuration, or

-	After MAC of the CG successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig of the CG while low mobility criterion is configured, or

-	If (SS-RSRP - SS-RSRPRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Connected:

-	The UE shall set the value of SS-RSRPRef to the current SS-RSRP value of the SpCell.

...

The relaxed measurement criterion of good serving cell quality for RLM is fulfilled when the downlink radio link quality on the configured RLM-RS resource is evaluated to be better than the threshold Qin+XdB, wherein

-	Qin is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationRLM.

The relaxed measurement criterion of good serving cell quality for BFD is fulfilled when the downlink radio link quality on the configured BFD-RS resource is evaluated to be better than the threshold Qin+XdB, wherein

-	Q in is specified in clause 8.1 of TS 38.133 [14].

-	X is the parameter offset in goodServingCellEvaluationBFD.

8.1.3.4.2.1.3	Test description

8.1.3.4.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 2

UE:

-	None

Preamble:

-	The UE is in state 2N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.3.4.2.1.3.2	Test procedure sequence

Table 8.1.3.4.2.1.3.2-1-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", “T2” and "T3", are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.3.4.2.1.3.2-1-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -78 | Off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | Off | -78 |  |

Table 8.1.1.1a.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -73 | Off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | Off | -73 |  |

Table 8.1.3.4.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are sent and received on NR Cell 1 unless otherwise specified. | - | - | - | - |
| 1 | The SS transmits an RRCReconfiguration message to the UE with goodServingCellEvaluationRLM and lowMobilityEvaluationConnected. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits and RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 4 | Start Timer T1=10 seconds. NOTE: This is an arbitrary value to wait for UE to initiate deregistration. | - |  | - | - |
| - | EXCEPTION: Steps 5a1-5b4 describes optional behaviour that depends on the UE implementation. | - | - | - | - |
| 5a1 | Check: Does the UE transmit an UEAssistanceInformation message containing rlm-MeasRelaxationState-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 5a2 | The SS cancels timer T1 started in step 4. | - | - | - | - |
| 5b1 | Timer T1 started in step 4 expires. | - | - | - | - |
| 5b2 | The SS transmits an ACTIVATE MEASUREMENT RELAXATION REQUEST message to the UE. | <-- | NR RRC: DLInformationTransfer TC: ACTIVATE MEASUREMENT RELAXATION REQUEST | - | - |
| - | EXCEPTION: Steps 5b3 and 5b4 can occur in any order. | - | - | - | - |
| 5b3 | The UE transmits an ACTIVATE MEASUREMENT RELAXATION RESPONSE message to the SS. | --> | NR RRC: DLInformationTransfer TC: ACTIVATE MEASUREMENT RELAXATION RESPONSE | - | - |
| 5b4 | Check: Does the UE transmit an UEAssistanceInformation message containing rlm-MeasRelaxationState-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the cell-specific reference signal level according to row "T0". | - | - | - | - |
| 7 | Check: Does the UE transmit an UEAssistanceInformation message in the next 6s? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| 8 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 9 | The SS waits 4s (for rlm-RelaxtionReportingProhibitTimer to expire). | - | - | - | - |
| 10 | The SS re-adjusts the cell-specific reference signal level according to row "T0". | - | - | - | - |
| 11 | Check: Does the UE transmit an UEAssistanceInformation message containing rlm-MeasRelaxationState-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 12 | The SS re-adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 13 | The SS waits T310 seconds. | - | - | - | - |
| 14 | The SS re-adjusts the cell-specific reference signal level according to row "T3". The following messsages are sent and received on NR CEll 11 unless otherwise specified. | - | - | - | - |
| 15 | The UE transmits an RRCReestablishmentRequest message. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 16 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 17 | The UE transmits an RRCReestablishmentComplete message | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 18 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 19 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 20 | Check: Does the UE transmit an UEAssistanceInformation message in the next 5s? | --> | NR RRC: UEAssistanceInformation | 3 | F |
| 21 | SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

8.1.3.4.2.1.3.3	Specific message contents

Table 8.1.3.4.2.1.3.3-1: RRCReconfiguration (Step 1, Table 8.1.3.4.2.1.3.2-3)

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig as per Table 8.1.3.4.2.1.3.3-2 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| otherConfig-v1700 ::= SEQUENCE { |  |  |  |  |
| ul-GapFR2-PreferenceConfig-r17 |  | Not present |  |  |
| musim-GapAssistanceConfig-r17 |  | Not present |  |  |
| musim-LeaveAssistanceConfig-r17 |  | Not present |  |  |
| successHO-Config-r17 |  | Not present |  |  |
| maxBW-PreferenceConfigFR2-2-r17 |  | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 |  | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 |  | Not present |  |  |
| rlm-RelaxationReportingConfig-r17::= SEQUENCE { |  |  |  |  |
| rlm-RelaxtionReportingProhibitTimer |  | s10 |  |  |
| } |  |  |  |  |
| bfd-RelaxationReportingConfig-r17 |  | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 |  | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 |  | Not present |  |  |
| propDelayDiffReportConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.3.4.2.1.3.3-2: CellGroupConfig (Table 8.1.3.4.2.1.3.3-1)

| Derivation Path: TS 38.508-1, table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig |  | Not present |  |  |
| physicalCellGroupConfig |  | Not present |  |  |
| spCellConfig SEQUENCE { |  | Not present |  |  |
| lowMobilityEvaluationConnected-r17 ::= SEQUENCE { |  |  |  |  |
| s-SearchDeltaP-Connected-r17 |  | dB3 |  |  |
| t-SearchDeltaP-Connected-r17 |  | s5 |  |  |
| } |  |  |  |  |
| goodServingCellEvaluationRLM-r17 ::= SEQUENCE { |  |  |  |  |
| offset-r17 |  | db2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |