3GPP TSG-RAN5 Meeting #97	R5-227257

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0292 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for NR EIEI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Applicability of NR EIEI test cases need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicability of new test cases as per NR EIEI WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will remain in the spec without corresponding applicability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 CR 3412 TS/TR 38.523-1 CR 3413 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

| 11.5 | eCall over IMS |  |  |  |
| --- | --- | --- | --- | --- |
| 11.5.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS / 5GS to EPS | Rel-16 | C170 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / 5GS to EPS | Rel-16 | C171 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.3 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.4 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.5 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS | Rel-16 | C174 | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.6 | eCall capable / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA | Rel-16 | C185 | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| 11.5.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success / 5GS | Rel-16 | C186 | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| 11.5.8 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported / RACH failure in NR cell / eCall using the CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.9 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C187 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.10 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.11 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.12 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.13 | eCall over IMS / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN / 5GS | Rel-16 | C189 | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.14 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall using CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.6 | 3GPP PS Data Off |  |  |  |
| 11.6.1 | Data Off / MO Voice Call | Rel-15 | C162 | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| 11.6.2 | Data Off / MO Video Call | Rel-15 | C172 | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video |
| 11,7 | eDRX / Redcap |  |  |  |
| 11.7.1 | eDRX / IDLE | Rel-17 | C210 | UEs supporting 5G Core and RedCap and eDRX |
| 11.6.3 | Data Off / SMSoIP | Rel-15 | C162A | UEs supporting 5G Core and NG.114 v2.0 and 3GPP PS data off and Initiating session and SMS over IP |
| 11.8 | Inter-system mobility between untrusted Non-3GPP and 3GPP system |  |  |  |
| 11.8.5 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from 5GS to EPC/ePDG | Rel-15 | C208 | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |

Table 4.1-5b: Additional Information of Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Specific IXIT | Number of TC Executions | Release other RAT |
| --- | --- | --- | --- | --- |
| 11 |  |  |  |  |
| 11.1 |  |  |  |  |
| 11.1.1 |  |  |  | Rel-15 E-UTRA |
| 11.1.2 |  |  |  | Rel-15 E-UTRA |
| 11.1.3 |  |  |  | Rel-15 E-UTRA |
| 11.1.4 |  |  |  | Rel-15 E-UTRA |
| 11.1.5 |  |  |  | Rel-15 E-UTRA |
| 11.1.6 |  |  |  | Rel-15 E-UTRA |
| 11.1.7 |  |  |  | Rel-15 E-UTRA |
| 11.1.8 |  |  |  | Rel-16 E-UTRA |
| 11.1.9 |  |  |  | Rel-16 E-UTRA |
| 11.2 |  |  |  |  |
| 11.2.1 |  |  |  | Rel-16 UTRA |
| 11.3 |  |  |  |  |
| 11.3.1 | pc_inactiveState |  |  |  |
| 11.3.6 | pc_inactiveState |  |  |  |
| 11.4 |  |  |  |  |
| 11.4.10 |  |  |  | Rel-15 E-UTRA |
| 11.4.11 |  |  |  | Rel-15 E-UTRA |
| 11.5 |  |  |  |  |
| 11.5.1 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.2 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.3 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.4 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.5 |  |  | Note 1 |  |
| 11.5.6 |  |  | Note 1 |  |
| 11.5.7 |  |  |  | Rel-16 UTRA |
| 11.5.8 |  |  | Note 1 |  |
| 11.5.9 |  |  | Note 1 |  |
| 11.5.10 |  |  | Note 1 |  |
| 11.5.11 |  |  | Note 1 |  |
| 11.5.12 |  |  | Note 1 | Rel-15 E-UTRA |
| 11.5.13 |  |  | Note 1 |  |
| 11.5.14 |  |  | Note 1 |  |
| Note 1: This test case can optionally be executed from Release 15 onwards. |  |  |  |  |

<End of modifed section>