3GPP TSG-RAN5 Meeting #97	R5-227259

Toulouse, France, 14th November – 18th November 2022

Title: 	Overview of open items for FR1 UL Intra-band Non-contiguous Carrier Aggregation

Source:	      Keysight Technologies UK Ltd

Agenda Item:	5.3.5.11

Document for:	Discussion and Endorsement

# 1.	Introduction

According to UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work item definition [1], FR1 UL Intra-band Non-contiguous Carrier Aggregation test cases for UEs power class 3 should be added into TS 38.521-1 ([2]) before the completion of the work item (expected by Dec’22 according to latest status report [3]).

This document analyses the remaining open points for addition of FR1 UL Intra-band Non-contiguous Carrier Aggregation test cases for UEs power class 3 and proposes how to move forward on this topic.

# 2.	Discussion

# 2.1 Band combinations

The reference band combination to complete FR1 UL Intra-band Non-contiguous Carrier Aggregation test cases for UEs power class 3 is CA_n77(2A) (Responsible company: Huawei, HiSilicon).

Other band combos will be part of NR_CADC_NR_LTE_DC_R16-UEConTest Work Item [4], being CA_n41(2A) and CA_n78(2A) (no other band combinations for FR1 UL Intra-band Non-contiguous Carrier Aggregation) as on-going band combinations in PRD21 [5]:

- CA_n41(2A): Fallback to CA_n24A-n41(2A). However, CA_n24A-n41(2A) does not include UL CA_n41(2A) so far. There is no other band combination using CA_n41(2A) in the uplink. Hence, this band combo should be corrected from “On-going” to “Pending” in PRD21.

- CA_n78(2A): Fallback to CA_n1A-n78(2A). However, CA_n1A-n78(2A) does not include UL CA_78(2A) so far but there is another band combination, CA_n3A-n78(2A) using CA_n78(2A) in the uplink with China Telecom as interested operator. Comments for CA_n78(2A) should be corrected in PRD21 as Fallback to CA_n3A-n78(2A).

Proposal 1: Correct PRD21 to set CA_n41(2A) UL configuration from “On-going” to “Pending” removing the fallback comment and to change the comment for CA_n78(2A) from “Fallback to CA_n1A-n78(2A)” to “Fallback to CA_n3A-n78(2A)”.

# 2.2 Test frequencies

The procedure to determine NR Intra-Band non-contiguous carrier aggregation test frequencies is defined in Annex C.2.4.3.2 in [6], although some corrections and additional work is required to complete the definition of the procedure as per [7] and [8].

Regarding test frequency values, all test frequencies for NR Intra-Band non-contiguous Carrier Aggregation with UL CA are FFS in section 4.3.1.1.5 in TS38.508-1 [6]. At least, test frequencies for CA_n77(2A) need to be defined to complete the work item.

Proposal 2: Complete the definition of test frequencies for CA_n77(2A) with UL CA.

# 2.2 Test cases

The following test cases for FR1 UL intra-band non-contiguous are at 0% completion in corresponding work plan [9]:

| ## WP Item ID | ## Is new test case? | ## TS/area | ## Clause | ## Title | ## Test purpose / Task | ## Company | ## Target | ## TDOC(s) | ## Comments | ## Weight | ## Overall | ## SA Option 2 (SA NR) | ## NSA Option 3 (EN-DC) |  | ## FR1 | ## FR2 | ## FR1 + FR2 | ## FR1 | ## FR2 | ## FR1 + FR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## 24 |  | ## 38.521-1 Tx | ## 6.2A.4.1 | ## Configured transmitted power for CA (2 UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing for CA_n77(2A) |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.2A.4.1.2 TS 38.522 TR 38.905 Note: CA_n77(2A) is selected as example CA configuration to complete the test case | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 30 |  | ## 38.521-1 Tx | ## 6.3A.1.1 | ## Minimum output power for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.3A.1.2 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 36 |  | ## 38.521-1 Tx | ## 6.3A.3.1 | ## Transmit ON/OFF time mask for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## 50 |  | ## 38.521-1 Tx | ## 6.4A.1.1 | ## Frequency error for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.4A.1.2 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 55 |  | ## 38.521-1 Tx | ## 6.4A.2.1.1 | ## Error Vector Magnitude for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.4A.2.2.1 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 58 |  | ## 38.521-1 Tx | ## 6.4A.2.2.1 | ## Carrier leakage for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing for details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.4A.2.2 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 61 |  | ## 38.521-1 Tx | ## 6.4A.2.3.1 | ## In-band emissions for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Adding testing for details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1 6.4A.2.2.2 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 77 |  | ## 38.521-1 Tx | ## 6.5A.3.1.1 | ## General spurious emissions for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Intra-band contiguous UL CA: adding testing details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1, 6.5A.3.1 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 77 |  | ## 38.521-1 Tx | ## 6.5A.3.1.1 | ## General spurious emissions for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Intra-band contiguous UL CA: adding testing details |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1, 6.5A.3.1 TS 38.522 TR 38.905 | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |
| ## 80 |  | ## 38.521-1 Tx | ## 6.5A.3.2.1 | ## Spurious emissions for UE co-existence for CA (2UL CA) | ## [Intra-band UL non-contiguous CA] Intra-band contiguous UL CA: adding testing for CA_n77(2A) |  | ## RAN5#93 (Nov-21) |  | ## TS 38.101-1, 6.5A.3.2.2 TS 38.522 TR 38.905 Note: CA-n77(2A) is selected as example CA configuration to complete the test case | ## 1 | ## 0% | ## 0% |  |  | ## 0% |  |  |  |  |  |

Observation 1: At least 10 test cases have definition pending in UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work plan.

For test cases already defined, testing frequencies are not aligned:

- Option 1: Low and high ranges.

This is the option used in the following test cases:

  - 6.2A.2.1 UE maximum output power reduction for CA (2UL CA)

  - 6.5A.2.4.1.1 NR ACLR for CA (2UL CA)

- Option 2: testing frequencies corresponding to Max Wgap (being Wgap defined as the widest possible on band based on the PCC and SCC configuration).

This option is the one used in the following test cases:

  - 6.3A.4.1.1 Absolute power tolerance for CA (2UL CA)

  - 6.3A.4.2.1 Relative power tolerance for CA (2UL CA)

  - 6.3A.4.3.1 Aggregate power tolerance for CA (2UL CA)

  - 6.5A.1.1 Occupied bandwidth for CA (2UL CA)

This option is aligned with LTE test cases, but it is not aligned with TS 38.508-1 ([6]) Annex C.2.4.3

- Option 3: Low range for PCC and SCC and High range for PCC and SCC.

This is the option used in test case 6.5A.2.2.1 Spectrum emission mask for CA (2UL CA) and could seem misleading as it could be read as:

  - overlapping PCC and SCC on either low range and high range or

  - PCC on low + SCC on high and SCC on low + PCC on high or

  - PCC and SCC according to low/high range defined in TS 38.508-1 ([6]) Annex C.2.4.3

Proposal 3: Test cases owners to align testing frequencies for all FR1 Intra-Band Non-Contiguous UL CA test cases testing frequencies as “Low range and High range as defined in TS 38.508-1 ([5]) Annex C.2.4.3” updating as needed initial conditions table and align 38.905 accordingly.

# 2.3 Target Completion date

According to UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work item status report [3], completion of the work item is expected by Dec’22.

Taking into account missing test cases definition as per Observation 1 above, missing test frequencies values definition and alignment on test frequencies ranges, it is proposed to delay WI completion at least 2 meeting cycles.

Proposal 4: Delay UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work item completion date at least 2 meeting cycles considering the UL Intra-band Non-contiguous Carrier Aggregation test cases (including missing test case definition, test point definitions and test frequencies).

# 3. 	Conclusion

This document analyses the remaining open points for addition of FR1 UL Intra-band Non-contiguous Carrier Aggregation test cases for UEs power class 3 and proposes how to move forward on this topic.

The following observations and proposals are made:

Proposal 1: Correct PRD21 to set CA_n41(2A) UL configuration from “On-going” to “Pending” removing the fallback comment and to change the comment for CA_n78(2A) from “Fallback to CA_n1A-n78(2A)” to “Fallback to CA_n3A-n78(2A)”.

Proposal 2: Complete the definition of test frequencies for CA_n77(2A) with UL CA.

Observation 1: At least 10 test cases have definition pending in UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work plan.

Proposal 3: Test cases owners to align testing frequencies for all FR1 Intra-Band Non-Contiguous UL CA test cases testing frequencies as “Low range and High range as defined in TS 38.508-1 ([5]) Annex C.2.4.3” updating as needed initial conditions table and align 38.905 accordingly.

Proposal 4: Delay UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1) work item completion date at least 2 meeting cycles considering the UL Intra-band Non-contiguous Carrier Aggregation test cases (including missing test case definition, test point definitions and test frequencies).

# 4.	References

- RP-200094, “Work Item Description for WI: UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1)”, RAN#87-e

- 3GPP TS 38.521-1 v17.6.1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

- RP-222459, “Status report for WI: UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1); rapporteur: Huawei, RAN#97

- RP-190321, “New WID on UE Conformance Test Aspects - Rel-16 NR CA and DC; and NR and LTE DC Configurations”, CMCC, Ericsson, China Unicom, RAN#83

- R5-224228, “3GPP RAN5 PRD 21 v1.2.0, NR bands and 5G NR CADC configuration handling in RAN5”, RAN5#96-e.

- 3GPP TS 38.508-1-1 v17.6.1 User Equipment (UE) conformance specification; Part 1: Common test environment

- R5-227258, “Revision of determination of test frequencies for NR Intra-band Non-Contiguous CA”, Keysight Technologies UK Ltd, RAN5#97

- R5-227261, “Updates to determination of test frequencies for NR Intra-band Non-Contiguous CA”, Keysight Technologies UK Ltd, RAN5#97

- R5-224849, “WP UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1)”, Huawei, HiSilicon, RAN5#96e