3GPP TSG-RAN5 Meeting #97	R5-227272
Toulouse, France, 14th– 18th November 2022

Agenda item:	5.4.17

Source:	Ericsson

Title:	FR2 SEM test time reduction by utilizing coarse TRP grid

Document for:	Endorsement

# Introduction

There have recently been proposals in RAN4 and RAN5 to reduce the test time in the FR2 SEM test case by changing the test metric from TRP to EIRP [1]. Due to concerns raised on e.g. the accuracy of this metric change, the proposals were not endorsed.

In this paper we suggest an alternative approach to test time reduction without the drawbacks associated with a metric change.

# Discussion

The proposal in [1] claims a 32% test time reduction which in our opinion does not justify changing the metric considering the drawbacks associated.

Some drawbacks being brought up in previous RAN5 discussions:

- It does not ensure that emissions in other directions than the beam peak are fulfilling the requirements

- The frequency dependency of the TRP to EIRP transformation (directivity) is large which is acknowledged in [1] but not compensated for resulting in under-estimation of the emission at a frequency offset. This will result in non-conformant UEs passing conformance test.

- It is unknown if regulations will accept the metric change. If not, the TRP testing is anyway required

- Testability issues caused by the large dynamic range of the signal resulting in MU degradation.

RAN5 have in the past discussed test time reduction for Spurious testing and have already developed a coarse TRP method [2] which has resulted in an even bigger test time reduction than achievable with TRP to EIRP metric change. It is worth highlighting that this coarse TRP method was developed in RAN5 and is therefore not documented in the RAN4 TR 38.810.

Test procedure in 38.521-2 Spurious test case:

The coarse TRP grid definition:

Observation 1: The coarse TRP method for Spurious result in a 75% reduction of number of grid points at the expense of an approximately 5 dB stricter test requirement. UEs with at least 5 dB margin to passing the requirement can utilize the coarse TRP method.

Observation 2: If using coarse TRP also for SEM the potential test time savings are bigger than in the metric change proposal while not increasing the risk of passing non-conformant UEs.

It is probable that the test requirement offset used for coarse TRP in spurious emission cannot be directly re-used for SEM since the directivity assumptions may not be the same, meaning a new MU evaluation needs to be performed. It should be noted that a new MU evaluation is required also for the metric change.

The Spurious test is using TT=0 whereas SEM use TT>0. This means that the test requirement offset does not need to be equal to the full MU value, but only the MU increase due to the coarse TRP grid.

Observation 3: If agreeing to proceed with coarse TRP for SEM an updated MU analysis need to be performed

Proposal 1: For RAN5 to decide if improving test time for SEM is essential enough to justify optimization which require a revised MU analysis.

Proposal 2: If proposal 1 is agreed, develop a coarse TRP method for SEM.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: The coarse TRP method for Spurious result in a 75% reduction of number of grid points at the expense of an approximately 5 dB stricter test requirement. UEs with at least 5 dB margin to passing the requirement can utilize the coarse TRP method.

Observation 2: If using coarse TRP also for SEM the potential test time savings are bigger than in the metric change proposal while not increasing the risk of passing non-conformant UEs.

Observation 3: If agreeing to proceed with coarse TRP for SEM an updated MU analysis need to be performed

Proposal 1: For RAN5 to decide if improving test time for SEM is essential enough to justify optimization which require a revised MU analysis.

Proposal 2: If proposal 1 is endorsed, develop a coarse TRP method for SEM.

# References

- R5-225029	“EIRP-based test metric for FR2 SEM test", 3GPP TSG-RAN5 Meeting #96-e, Apple

- R5-185347	“Discussion on FR2 TRx spurious test procedure” , 3GPP TSG-RAN5 Meeting #80, Anritsu