3GPP TSG-RAN WG5 Meeting # 97	R5-227291

# Toulouse, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1505 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on test requirements for spurious emission for UE co-existence limits for EN-DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test requirements for spurious emission for UE co-existence for EN-DC configuration are specified in different tables in accordance with the different releases in the spec. In R5-225651, the proposal 2 to merge the tables of test conformance requirements for EN-DC configurations for UE co-existence with protected bands with all the releases in one table and add a colun “Release” in the table to indicate from which release the configuration supports was endorsed in RAN5#96-e. The following guidelines will be applied in this CR. - If the requirements for a specified configuration have not changed since an early release, the column of “Release” should be noted as all the supported releases in one row. Otherwise, a new row in the table for the later release with new requirements for the specified configuration should be added. - If the Note number in the table is different among releases, an additional information for release should be applied, such as NOTE XR15, NOTE XR16, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merge the tables with different releases for the test requirements for spurious emission for UE co-existence for EN-DC configuration and add a new column of “Release”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirements for spurious emission for UE co-existence for EN-DC configuration will be redundant. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.3.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

<< Unchanged sections omitted >>

6.5B.3.3.2.5	Test Requirement

The test requirements are in Table 6.5B.3.3.2.5-4 for Release 15 and Release 16, respectively. For EN-DC only capable devices, in addition to Table 6.5B.3.3.2.5-4, the test requirements as in clause 6.5.3.2.5 in TS 38.521-1 [8] are also needed.

For inter-band EN-DC with the uplink assigned to one LTE band and one NR band, the test requirements in Table 6.5B.3.3.2.5-4 can be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.2.5-1: Void

Table 6.5B.3.3.2.5-2: Void

Table 6.5B.3.3.2.5-3: Void

Table 6.5B.3.3.2.5-4: Requirements for inter-band within FR1

| EN-DC Configuration | Release |  | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| DC_1_n3 |  | Rel-16 Rel-17 |  | E-UTRA Band 1, 11, 21, 32, 43, 50, 65, 74, 75 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band 77, 78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_1_n28 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 7, 31, 41, 72, 73 NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 470 | - | 694 |  | -42 |  | 8 |  | 5, 17 |
|  |  |  |  | Frequency range |  | 470 | - | 710 |  | -26.2 |  | 6 |  | 14 |
| DC_1_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 11, 21, 34, 65, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1880 | - | 1895 |  | -40 |  | 1 |  | 5, 8 |
|  |  |  |  | Frequency range |  | 1895 | - | 1915 |  | -15.5 |  | 5 |  | 5, 7, 8 |
|  |  |  |  | Frequency range |  | 1915 | - | 1920 |  | 1.6 |  | 5 |  | 5, 7, 8 |
| DC_1_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 11, 21, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1880 | - | 1895 |  | -40 |  | 1 |  | 5, 8 |
| DC_1_n79 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 5, 7, 8, 18, 19, 26, 28, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_2_n5 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 42, 48 NR band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41, 43 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n41 |  | Rel-16 Rel-17 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Bands 2 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_2_n66 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 4, 10, 24, 50, 66, 70, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  |  |  | E-UTRA Band 42, 48 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n77 |  | Rel-17 |  | E-UTRA Band 4, 65, 66, 70 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n71 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_2_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 24, 50, 51, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_3_n1 |  | Rel-16 Rel-17 |  | E-UTRA Band 1, 11, 21, 32, 50, 65, 74, 75 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_3_n5 |  | Rel-16 |  | E-UTRA Band 5, 7, 8, 18,19, 26, 38 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42, 52 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_3_n7 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44, 67 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_3_n28 |  | Rel-15 Rel-16 Rel-17 |  | NR band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 7, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_3_n41 |  | Rel-16 Rel-17 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 44 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 18, 19 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 14, 20 |
|  |  |  |  | E-UTRA Band 42, NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_3_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 34, 39 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_3_n79 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 5, 8, 11, 18, 19, 21, 41, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_5_n2 |  | Rel-16 Rel-17 |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41, 43 NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_5_n66 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 5, 6, 7, 8, 38 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 26 |  | 859 | - | 869 |  | -27 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41, 42, 52 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | NR Band n77 |  |  |  |  |  |  |  |  |  |  |
| DC_5_n77 |  | Rel-17 |  | E-UTRA Band 2, 4, 25, 30, 40, 65, 66, 70 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_5_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 2, 3, 4, 7, 25, 34, 38, 65, 66, 70 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2, 7 |
| DC_7_n1 |  | Rel-16 Rel-17 |  | E-UTRA Band 32, 50, 51, 75, 76 or NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_7_n3 |  | Rel-16 Rel-17 |  | E-UTRA Band 5, 8, 20, 26, 27, 28, 67, 68 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA band 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_7_n5 |  | Rel-16 |  | E-UTRA Band 5, 26, 28, 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 52 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_7_n8 |  | Rel-16 |  | E-UTRA Band 20, 28, 67, 68 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42, 52 NR band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_7_n28 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR band n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_7_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 50, 51, 67, 68, 74, 75, 76 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_7_n66 |  | Rel-16 Rel-17 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 85 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA band 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_8_n1 |  | Rel-16 Rel-17 |  | E-UTRA Band 43 or NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_8_n3 |  | Rel-16 Rel-17 |  | E-UTRA Band 20, 28, 38, 44 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 7, 22, 41, 42, 43 or NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 860 | - | 890 |  | -40 |  | 1 |  | 5, 12 |
| DC_8_n20 |  | Rel-16 |  | E-UTRA Band 67, 68 |  | 749 | - | 783 |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 7, 38, 69 |  | 2570 | - | 2690 |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 8, 20 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  |  |  | Frequency range |  | 758 | - | 788 |  | -50 |  | 1 |  |  |
| DC_8_n28 |  | Rel-16 |  | E-UTRA Band 38, 40 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 491 | - | 616 |  | -42 |  | 8 |  | 5, 17 |
|  |  |  |  | Frequency range |  | 491 | - | 616 |  | -26.2 |  | 6 |  | 14 |
| DC_8_n41 |  | Rel-16 Rel-17 |  | E-UTRA Band 28 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  |  |  | Frequency range |  | 860 | - | 890 |  | -40 |  | 1 |  | 5, 12 |
| DC_8_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 32, 33, 34, 38, 39, 40,50, 65, 69, 74, 75 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA band 3, 7, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_8_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA band 34, 39, 40, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 3, 7, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 12 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 12 |
| DC_11_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 945 | - | 960 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_11_n79 |  | Rel-15 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_11_n79 |  | Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 34, 42, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_12_n66 |  | Rel-15 |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_12_n66 |  | Rel-17 |  | E-UTRA Band 41, 53 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_12_n78 |  | Rel-16 Rel-17 |  | E-UTRA Band 2, 7, 25, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 4, 66 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_13_n2 |  | Rel-16 Rel-17 |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_13_n66 |  | Rel-16 Rel-17 |  | E-UTRA Band 41, 53 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_13_n77 |  | Rel-17 |  | E-UTRA Band 2, 4, 10, 25, 41, 66, 70 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 30 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_14_n2 |  | Rel-16 Rel-17 |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_14_n66 |  | Rel-16 Rel-17 |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_19_n1 |  | Rel-17 |  | E-UTRA Band 42 NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_19_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 34, 40, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 34, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_19_n79 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_20_n1 |  | Rel-16 Rel-17 |  | E-UTRA Band 22, 42, 43 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band 77, 78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20_n3 |  | Rel-16 Rel-17 |  | E-UTRA Band 7, 8 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 38, 42, 52 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20_n7 |  | Rel-16 |  | E-UTRA Band 68 |  | 776 | - | 783 |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42, 52 |  | 3332 | - | 3432 |  | -50 |  | 1 |  | 2 |
|  |  |  |  | NR band n77, n78 |  | 3332 | - | 3432 |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
| DC_20_n8 |  | Rel-16 |  | E-UTRA Band 28 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 7, 38 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_20_n28 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_20_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 7, 33, 34, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 38, 69 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_21_n1 |  | Rel-17 |  | E-UTRA Band 40, 42 NR Band n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 945 |  | 960 |  | -50 |  | 1 |  |  |
| DC_21_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 945 | - | 960 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_21_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_21_n79 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 21, 34, 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_25_n41 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 12, 13, 14, 17, 28, 29, 42, 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_26_n41 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 5, 18, 19, 26, 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 799 | - | 803 |  | -40 |  | 1 |  | 5 |
|  |  |  |  | Frequency range |  | 945 | - | 960 |  | -50 |  | 1 |  |  |
| DC_26_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 34, 39, 40, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_26_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 34, 39, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
|  |  |  |  | Frequency range |  | 2545 | - | 2575 |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 2595 | - | 2645 |  | -50 |  | 1 |  |  |
| DC_28_n3 |  | Rel-16 Rel-17 |  | NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 7, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_28_n5 |  | Rel-16 |  | E-UTRA Band 8, 40, 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 24 |  | 1527 | - | 1559 |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 30 |  | 2351 | - | 2360 |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 557 | - | 672 |  | -42 |  | 8 |  | 5, 17 |
|  |  |  |  | Frequency range |  | 557 | - | 672 |  | -26.2 |  | 6 |  | 14 |
|  |  |  |  | Frequency range |  | 662 | - | 672 |  | -26.2 |  | 6 |  | 5 |
| DC_28_n7 |  | Rel-16 |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 52 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | NR band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_28A_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 9 |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 7, 34, 39, 40, 41 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 9, 11 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_28_n79 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_30_n5 |  | Rel-15 |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_30_n5 |  | Rel-16 Rel-17 |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_39_n41 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 42, 44 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_39_n79 |  | Rel-15 |  | E-UTRA Band 8, 41, 44 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_39_n79 |  | Rel-16 Rel-17 |  | E-UTRA Band 8, 28, 41, 44 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_40_n1 |  | Rel-16 Rel-17 |  | E-UTRA Band 7, 31, 32, 41, 45, 50, 72, 73, 74, 75 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_40_n41 |  | Rel-15 Rel-16 Rel-17 |  | Bands 1, 34, 39, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3, 19 |
| DC_40_n78 |  | Rel-16 Rel-17 |  | E-UTRA Band 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 32, 44, 45, 50, 51, 74, 75, 76 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_40_n79 |  | Rel-16 |  | E-UTRA Band 1, 34, 41, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_41_n77 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n79 |  | Rel-15 |  | E-UTRA Band 1, 3, 5, 8, 18, 19, 26, 28, 34, 40, 44, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_41_n79 |  | Rel-16 Rel-17 |  | E-UTRA Band 1, 3, 5, 8, 9, 18, 19, 26, 28, 34, 40, 44, 65 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_48_n5 |  | Rel-16 Rel-17 |  | E-UTRA Band 25, 70 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 | - | 1915.7 |  | -41 |  | 0.3 |  | 3 |
| DC_48_n66 |  | Rel-16 Rel-17 |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_66_n2 |  | Rel-16 Rel-17 |  | E-UTRA Band 4, 10, 22, 24, 50, 66, 70, 74 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 5 |
|  |  |  |  | E-UTRA Band 42, 43 NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n5 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 5, 6, 7, 8, 26, 38 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 41, 42, 52 NR Band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n41 |  | Rel-16 Rel-17 |  | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 28, 29, 43, 85 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
|  |  |  |  | E-UTRA Band 42, 48, NR band n77 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |
| DC_66_n77 |  | Rel-17 |  | E-UTRA Band 2, 4, 12, 13, 14, 17, 29, 30, 65, 66, 70, 71 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| DC_66_n78 |  | Rel-15 Rel-16 Rel-17 |  | E-UTRA Band 3, 34, 39 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |

==============================================================

<< End of changes >>