3GPP TSG-WG5 Meeting #97	R5-227327

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0853 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on In-band blocking requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Hungary kft. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest. |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Aditiona Rx parameters and channel bandwidth were defined |  |  |  |  |  |  |  |  |
| Summary of change: |  | Channels n259, n262, n263 and channel bandwidth from 800 – 200MHz were added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements harmnizarion between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

7.6.2.3	Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1). The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.6.2.3-1: In-band blocking requirements

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14dB |  |  |  |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | REFSENS + 35.5 dB | N/A | N/A | N/A |
| PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | REFSENS + 34.5 dB | N/A | N/A | N/A |
| PInterferer for band n263 | dBm | N/A | REFSENS + 33.5 dB | N/A | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB | REFSENS + 33.5 dB |
| FIoffset | MHz | ≤ -100 & ≥ 100 NOTE 5 | ≤ -200 & ≥ 200 NOTE 5 | ≤ -400 & ≥ 400 NOTE 5 | ≤ -800 & ≥ 800 NOTE 5 | ≤ -1600 & ≥ 1600 NOTE 5 | ≤ -3200 & ≥ 3200 | ≤ -4000 & ≥ 4000 |
| FInterferer | MHz | FDL_low + 25 to FDL_high - 25 | FDL_low + 50 to FDL_high - 50 | FDL_low + 100 to FDL_high - 100 | FDL_low + 200 to FDL_high - 200 | FDL_low + 400 to FDL_high - 400 | FDL_low + 800 to FDL_high - 800 | FDL_low + 1600 to FDL_high - 1600 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) and set-up according to Annex C. NOTE2: The REFSENS power level is specified in Section 7.3.2.3, which are applicable according to different UE power classes. NOTE 3: The wanted signal consists of the reference measurement channel specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) and set-up according to Annex C. NOTE 4: FIoffset is the frequency separation between the centre of the aggregated CA bandwidth and the centre frequency of the Interferer signal. NOTE 5: The absolute value of the interferer offset FIoffset shall be further adjusted (CEIL(¦FInterferer¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 6: FInterferer range values for unwanted modulated interfering signals are interferer centre frequencies. NOTE 7: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [10] clause 7.6.2.

## <End of Change>