3GPP TSG-RAN5 Meeting #97	R5-227333
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0620 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PowerSaving test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 38.213 section 10.1, a CSS set is configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by PS-RNTI, while a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI. In section 5.3.2.1.3.4.3, 5.3.2.2.3.4.3, 5.3.3.1.3.4.3, 5.3.3.2.3.4.3, for Message content definition, it is defined one dedicated SearchSpace with searchSpaceType = ue-Specific to transmit DCI Format 1_1. However, it is also defined one SearchSpaceExt-r16 IE with searchSpaceType = common for DCI Format 2_6 transmission, that will override the searchSpaceType to common for the SearchSpace, so, the UE will not have a USS to monitor the transmission of PDCCH for DCI Format 1_1. In Table 5.3.2.1.3.3-2, Table 5.3.2.2.3.3-2, Table 5.3.3.1.3.3-2, Table 5.3.3.2.3.3-2, it is required to test AggregationLevel4 and AggregationLevel8, but in the SearchSpace IE in the message content definition, only AggregationLevel4 is configured. In section 5.3.2.1.3.4.2, 5.3.3.1.3.4.2 step 2 for Test procedure, and section 5.3.2.2.3.4.2, 5.3.3.2.3.4.2 step 3 for Test procedure, table for minimum conformace requirement is referenced to configure SNR, instead of table for conformance requirements. In section 5.3.2.2.3.4.2, 5.3.3.2.3.4.2 step 2 of test procedure, it is requested to configure PDCCH Reference Channel to R.PDCCH.5-1.2 TDD, but this Reference Channel is not defined in Table 5.3.2.2.3.3-2 nor Table 5.3.3.2.3.3-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Define two SearchSpace and tow SearchSpaceExt-r16 in PDCCH-Config: - SearchSpace and SearchSpaceExt-r16 with searchSpaceType = ue-Specific for the UE to monitor the transmission of DCI Format 1_1. - SearchSpace and SearchSpaceExt-r16 with searchSpaceType = common for the UE to monitor the transmission of DCI Format 2_6. - Add the configuration of AggregationLevel8 in the definition of the SearchSpace message. - Include correct reference for SNR configuration in test procedure. - Remove invalid reference for R.PDCCH.5-1.2 TDD in Test Procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test definition will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1.3, 5.3.2.2.3, 5.3.3.1.3, 5.3.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

##### 5.3.2.1.3	2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.2.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting DRX adaptation

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC that supporting DRX adaptation.

5.3.2.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.2.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.2.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.2.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.2.1.3.

5.3.2.1.3.4	Test description

5.3.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.1-1, Table 5.3.2.1.3.3-1 and Table 5.3.2.1.3.3-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.1.3.4.3.

5.3.2.1.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.1.3.3-1. The details of PDCCH are specified in Table 5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.3.3-1 and Table 5.3.2.1.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.3.4.4-1as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.3.4.4-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.2.1.3.3-2 as appropriate.

5.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.3.4.3.1	Message exceptions for SA

Table 5.3.2.1.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 2 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet1 |  |  |
| ControlResourceSet[2] | ControlResourceSet2 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-4: PDCCH-ControlResourceSet1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  |  |
| interleaverSize | n3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-5: PDCCH-ControlResourceSet2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  |  |
| interleaverSize | n3 |  |  |
| } |  |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList { | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-6: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 | Long_DCI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-6A: PDCCH Search Space2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | SearchSpaceId with condition CSS | CSS |
| controlResourceSetId | 2 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-7: PDCCH Search Space Ext1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 1 |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-7A: PDCCH Search Space Ext2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 2 |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common SEQUENCE { |  |  |  |
| dci-Format2-6-r16 SEQUENCE { | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.2.1.3.4.3.2	Message exceptions for NSA

Same as 5.3.2.1.3.4.3.1

5.3.2.1.3.4.4	Test requirement

Table 5.3.2.1.3.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.3.4.4-1.

Table 5.3.2.1.3.4.4-1: Test Requirements for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 6.4 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

## <<< Skip unchanged sections >>>

##### 5.3.2.2.3	2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.2.2.3.1	Test Purpose

This test verifies the demodulation performance of DCI format 2_6 PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Long DRX Cycle and DRX adaptation.

5.3.2.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.2.3.3-1 are valid for all TDD tests for power saving unless otherwise stated.

Table 5.3.2.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability.. |

For the parameters specified in Table 5.3.2.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.2.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.3.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.3.4	Test description

5.3.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.2.2.3.4.3.1and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.3.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.3.4.3.

5.3.2.2.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for C_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.	SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to  Table 5.3.2.2.3.5-1 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.3.5-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.2.2.3.3-1 as appropriate.

5.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.2.2.3.4.3.1	Message exceptions for SA

Table 5.3.2.2.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 2 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet1 |  |  |
| ControlResourceSet[2] | ControlResourceSet2 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-4: PDCCH-ControlResourceSet1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-5: PDCCH-ControlResourceSet2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList { | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-6: PDCCH Search Space1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 | Long_DCI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-6A: PDCCH Search Space2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | SearchSpaceId with condition CSS | CSS |
| controlResourceSetId | 2 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-7: PDCCH Search Space Ext1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 1 |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-7A: PDCCH Search Space Ext2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 2 |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common SEQUENCE { |  |  |  |
| dci-Format2-6-r16 SEQUENCE { | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.2.2.3.4.3.2	Message exceptions for NSA

Same as 5.3.2.2.3.4.3.1.

5.3.2.2.3.5	Test requirement

Table 5.3.2.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.3.5-1.

Table 5.3.2.2.3.5-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

## <<< Skip unchanged sections >>>

##### 5.3.3.1.3	4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting 4 Rx antenna ports and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward that supporting EN-DC, 4 Rx antenna ports and DRX adaptation.

5.3.3.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.3.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.3.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.3.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.3.1.3.

5.3.3.1.3.4	Test description

5.3.3.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1, Table 5.3.3.1.3.3-1 and Table 5.3.3.1.3.3-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.3.4.3.

5.3.3.1.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.1.3.3-1. The details of PDCCH are specified in Table 5.3.1, Table 5.3.3.1-1, Table 5.3.3.1.3.3-1 and Table 5.3.3.1.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

2.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.3.4.4-1 as appropriate.

3.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.3.4.4-1, pass the UE. Otherwise fail the UE.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.3.3.1.3.3-2 as appropriate.

5.3.3.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.3.4.3.1	Message exceptions for SA

Table 5.3.3.1.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 2 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet1 |  |  |
| ControlResourceSet[2] | ControlResourceSet2 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-4: PDCCH-ControlResourceSet1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n2 |  |  |
| interleaverSize | n3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-5: PDCCH-ControlResourceSet2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  |  |
| interleaverSize | n3 |  |  |
| } |  |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList { | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-6: PDCCH Search Space1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS |  |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| ue-Specific SEQUENCE { |  |  |  |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-6A: PDCCH Search Space2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | SearchSpaceId with condition CSS |  |
| controlResourceSetId | 2 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-7: PDCCH Search Space Ext1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 1 |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-7A: PDCCH Search Space Ext2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 2 |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common SEQUENCE { |  |  |  |
| dci-Format2-6-r16 SEQUENCE { | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.3.1.3.4.3.2	Message exceptions for NSA

Same as 5.3.3.1.3.4.3.1

5.3.3.1.3.4.4	Test requirement

Table 5.3.3.1.3.4.4-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.3.4.4-1.

Table 5.3.3.1.3.4.4-1: Test Requirements for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 1.1 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

## <<< Skip unchanged sections >>>

##### 5.3.3.2.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.2.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

5.3.3.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

For the parameters specified in Table 5.3.3.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

Table 5.3.3.2.3.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.3.4	Test description

5.3.3.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.3.2.3.4.3.1 and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.1.4.3.

5.3.3.2.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for C_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.	SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.3.5-1as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.3.5-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.3.2.3.3-2 as appropriate.

5.3.3.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.3.2.3.4.3.1	Message exceptions for SA

Table 5.3.3.2.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 2 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet1 |  |  |
| ControlResourceSet[2] | ControlResourceSet2 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-4: PDCCH-ControlResourceSet1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-5: PDCCH-ControlResourceSet2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList { | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-6: PDCCH Search Space1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | n1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 | Long_DCI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-6A: PDCCH Search Space2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 3 | SearchSpaceId with condition CSS |  |
| controlResourceSetId | 2 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | Test AL4 |  |
| aggregationLevel8 | N1 | Test AL8 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-7: PDCCH Search Space Ext1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 1 |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-7A: PDCCH Search Space Ext2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpaceExt-r16 ::= SEQUENCE { |  |  |  |
| controlResourceSetId-r16 | 2 |  |  |
| searchSpaceType-r16 SEQUENCE { |  |  |  |
| common SEQUENCE { |  |  |  |
| dci-Format2-6-r16 SEQUENCE { | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.3.2.3.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.3.4.3.1.

5.3.3.2.3.5	Test requirement

Table 5.3.3.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.3.5-1.

Table 5.3.3.2.3.5-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | 0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

## <<< END OF CHANGES >>>