3GPP TSG-RAN5 Meeting #97	R5-227343

Toulouse, France, 14th November – 18th November 2022

Title: 	On FR2 Inter-band DL CA

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.13.8

Document for:	Discussion and Endorsement

# 1.	Introduction

As part of the UE Conformance Test Aspects - NR RF requirement enhancements for frequency range 2 Work Item [1], one of the objectives is to add corresponding test cases to TS 38.521-2 [2] for FR2 Inter-band DL CA for band combination CA_n260_n261 based on independent beam management. This objective has not been achieved yet according to the progress in latest work plan [3].

This document performs initial analysis of required changes to introduce FR2 Inter-band DL CA and proposes a way forward.

# 2.	Discussion

2.1 Number of component carriers

Despite initially RAN4 only defined CA_n260A-n261A under Rel-16 core specification [4], either Rel-17 Further enhancements of NR RF requirements for frequency range 2 (FR2) work item [5] or RAN4 Rel-17 carrier aggregation basket work items added additional FR2 Inter-band DL CA band combinations with up to 16 DL CA (16CCs) distributed in 2 bands (CA_n260M-n261M). These additional band combinations were defined as applicable from Rel-16 onwards in TS 38.307 [6]:

However, in current work plan [3] only test cases up to 8DL CA (8CCs) have been considered for FR2 Inter-band DL CA.

Besides that, in PRD21 [7], it is indicated in section 4.3.1 that:

In PRD21 [7], the only Ongoing FR2 Inter-band DL CA band combination is CA_n260A-n261A. Hence, RAN5 can only progress in 2 DL CA test cases for FR2 inter-band DL CA.

Proposal 1: RAN5 to discuss whether work plan [3] should list FR2 RX Inter-band DL CA test cases up to 16 CCs although most of them should be on hold until any associated band combination status is moved to Ongoing.

Proposal 2: RAN5 to start progressing on FR2 inter-band DL CA for test cases up to 2 CCs.

2.2 Testing direction

One of the first aspects to be clarified is the testing direction in FR2 Inter-band DL CA, as this will drive whether a totally separate test procedure needs to be defined (even when having common test cases)​

Keysight’s understanding is that a non-written assumption was used in RAN4 such that the test systems used for Rel-16 FR2 Inter-band DL CA should be 1 AoA, although both co-located and non-co-located deployments should be considered as part of the scope, i.e. there is no deployment restriction​.

This means that the Rx beam peak directions of each band could be different. Hence, core requirements were defined based on testing of only one CC at a time while setting untested CCs in the other band in a low-stress state so the device can easily compensate for gain reduction due to misalignment from beam peak (i.e. EIS spherical coverage level):

Observation 1: Rx beam peak testing direction could be different for each band, i.e. more than one testing direction is required to measure REFSENS for inter-band DL CA.

Proposal 3: Decouple test procedure for inter-band and intra-band DL CA in FR2 Rx test cases for CA.

2.3 REFSENS test procedure and test requirements

For 2 CC FR2 inter-band CA REFSENS test, a very simplified version of the test procedure could be described as follows:​

- Set Rx beam peak search of PCC​

- Set all CCs DL power level to EIS spherical coverage​

- Measure actual EIS on PCC​

- Set Rx beam peak search of SCC​

- Set all CCs DL power level to EIS spherical coverage

- Measure actual EIS on SCC​

Values measured in steps 3 and 6 should comply with single CC REFSENS requirements​

For higher number of component carriers (combination of intra-band contiguous and inter-band, ex: CA_n260M-n261M), a more generic test procedure would be required:

- Set Rx beam peak search of CCs in first band

- Set all CCs DL power level to its EIS spherical coverage

- For each CC in first band:

  - Measure actual EIS on CC under test

  - Set back DL power level of the CC under test to EIS spherical coverage

- Set Rx beam peak search of CCs in second band

- For each CC in second band:

  - Measure actual EIS on CC under test

  - Set back DL power level of the CC under test to EIS spherical coverage

Values measured in steps 3a and 5a should comply with single CC REFSENS requirements​

However, in the case of more than 2CCs, it is not clear:

- The EIS spherical coverage power level which each DL CC should be configured to?

  - Should it be single CC EIS spherical coverage?

  - Should it be single CC EIS spherical coverage + REFSENS relaxation in table 7.3A.2.3?

(To be noticed that there are no core requirements defined for Spherical coverage for intra-band DL CA)

- Whether other CCs within the band of the CC under test should be also set to EIS spherical coverage value

Proposal 4: Define FR2 Inter-band DL CA Rx test procedures for 2 DL CA as described above (without making it generic for higher number of CCs).

Proposal 5: Send LS to RAN4 looking for clarifications for the cases of more than 2 DL CCs as described above.

Proposal 4 has been implemented in reference sensitivity test case in [8] as a reference.

# 3. 	Conclusion

This document performs initial analysis of required changes to introduce FR2 Inter-band DL CA and proposes a way forward.

The following observation and proposals are done:

Proposal 1: RAN5 to discuss whether work plan [3] should list FR2 RX Inter-band DL CA test cases up to 16 CCs although most of them should be on hold until associated band combinations status is moved to Ongoing.

Proposal 2: RAN5 to start progressing on FR2 inter-band DL CA for test cases up to 2 CCs.

Observation 1: Rx beam peak testing direction could be different for each band, i.e. more than one testing direction is required to measure REFSENS for inter-band DL CA.

Proposal 3: Decouple test procedure for inter-band and intra-band DL CA in FR2 Rx test cases for CA.

Proposal 4: Define FR2 Inter-band DL CA Rx test procedures for 2 DL CA as described above (without making it generic for higher number of CCs).

Proposal 5: Send LS to RAN4 looking for clarifications for the cases of more than 2 DL CCs as described above.

# 4.	References

- RP-212304, “Revised WID - UE Conformance Test Aspects for NR RF Requirement Enhancements for FR2”, Apple, Nokia, RAN#93e

- 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-224144, “WP UE Conformance Test Aspects for NR RF Requirement Enhancements for FR2”, Nokia, Nokia Shanghai Bell, Apple, RAN5#95e

- 3GPP TS 38.101-2 v17.7.0, NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

- RP-221784, “Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2)”, Nokia, Nokia Shanghai Bell, RAN#96e

- 3GPP TS 38.307 v 16.11.0, NR; Requirements on User Equipments (UEs) supporting a release-independent frequency band

- R5-224228, 3GPP RAN5 PRD 21 v1.2.0 (2022-8), Permanent Reference Document (PRD); NR bands and 5G NR CADC configuration handling in RAN5, RAN5#96e

- R5-227344, “Test procedure update for Reference sensitivity power level for CA (2DL CA) for inter-band DL CA”, Keysight Technologies UK Ltd RAN5#97e