3GPP TSG-RAN5 Meeting #97	R5-227465

# Toulouse, France, 14th Nov 2022 – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3300 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC 12.2.1.6- Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA,CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The synchronised information for NR-SS-UE1 and UE should be aligned with TS 38.508-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the synchronised information for NR-SS-UE1 and UE to GNSS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 12.2.1.6	Inter-carrier concurrent operation / Sidelink communication / RRC_CONNECTED / Reception

12.2.1.6.1	Test Purpose (TP)

(1)

with {  UE being in RRC_CONNECTED state and its serving cell not broadcasting SIB12 }

ensure that {

when { UE is configured by upper layer to perform NR sidelink reception}

then { UE is able to monitor NR sidelink reception using sl-RxPool included in pre-configuration}

}

(2)

with {  UE being in RRC_CONNECTED state and its serving cell broadcasting SIB12 }

ensure that {

when { UE is configured by upper layer to perform NR sidelink reception}

then { UE is able to monitor NR sidelink reception using sl-RxPool included in SIB12}

}

(3)

with {  UE being in RRC_CONNECTED state and having received a RRCReconfiguration including sl-RxPool }

ensure that {

when { UE is configured by upper layer to perform NR sidelink reception}

then { UE is able to monitor NR sidelink reception using sl-RxPool included in sl-ConfigDedicatedNR}

}

(4)

with {  UE being in RRC_CONNECTED state and being configured by upper layer to perform NR sidelink reception }

ensure that {

when { UE receives a RRCReconfiguration including reconfigurationWithSync and sl-RxPool}

then { UE is able to monitor NR sidelink reception using sl-RxPool included in the received RRCReconfiguration message during handover procedure}

}

12.2.1.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.8.1, clause 5.8.7. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.1]

The PC5-RRC signalling, as specified in sub-clause 5.8.9, can be initiated after its corresponding PC5 unicast link establishment (TS 23.287 [55]).

[TS 38.331, clause 5.8.7]

A UE capable of NR sidelink communication that is configured by upper layers to receive NR sidelink communication shall:

1>	if the conditions for NR sidelink communication operation as defined in 5.8.2 are met:

2>	if the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in RRCReconfiguration message or sl-FreqInfoList included in SIB12:

3>	if the UE is configured with sl-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover):

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated by sl-RxPool;

3>	else if the cell chosen for NR sidelink communication provides SIB12:

4>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated by sl-RxPool in SIB12;

2>	else:

3>	configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources that were preconfigured by sl-RxPool in SL-PreconfigurationNR, as defined in sub-clause 9.3;

12.2.1.6.3	Test description

12.2.1.6.3.1	Pre-test conditions

System Simulator:

-	SS-NW

- NR Cell 1 is the serving cell and NR Cell 3 is the inter-frequency neighbour cell of NR Cell 1.

- System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.

-	NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication receiving and transmitting device on the resources that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 is synchronised on GNSS.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	The UE is equipped with below information in UE or in a USIM containing default values (as per TS 38.508-1 [4] clause 4.8.3.3.3) except for those listed in Table 12.2.1.6.3.1-1.

-	UE is synchronised on GNSS.

Table 12.2.1.6.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | service no. 119 is available |  |
| EFVST |  | Service no.2 V2X policy configuration data over PC5 is supported, i.e. value is '01 02' HEX |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [4], table 4.10.1-1, except SL-BWP-PoolConfigCommon field as defined in Table 12.2.1.6.3.3-1 with condition SL-PRECONFIG |  |

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL and Test Loop Function (On) with UE test loop mode E as defined in TS 38.508-1 [4] subclause 4.4A on NR Cell 1, using generic parameters Sidelink (On), Cast Type (Unicast) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.2.1.6.3.2	Test procedure sequence

Tables 12.2.1.6.3.2-1 and 12.2.1.6.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1 and NR Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 12.2.1.6.3.2-3.

Table 12.2.1.6.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for NR Cell 3 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 3 |

Table 12.2.1.6.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | Power levels are such that entry condition for event A3 is not satisfied for NR Cell 3 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | FFS | FFS | Power levels are such that entry condition for event A3 is satisfied for NR Cell 3 |

Table 12.2.1.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.31 indicate that the UE monitors NR sidelink reception using sl-RxPool included in pre-configuration? | --> |  | 1 | - |
| 2 | SS starts broadcasting SIB12 in NR Cell 1 using system information combination NR-14 | - |  | - | - |
| 3 | NR Cell 1 transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message |  | - |
| 4 | void | - | - | - | - |
| 5 | The UE transmits a SidelinkUEInformationNR message to indicate it is (interested in) receiving NR sidelink communication. | --> | NR RRC: SidelinkUEInformationNR | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.31 indicate that the UE monitors NR sidelink reception using sl-RxPool included in SIB12? | - | - | 2 | - |
| 7 | SS-NW sends an RRCReconfiguration message including sl-RxPool. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | UE sends an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.31 indicate that the UE monitors NR sidelink reception using sl-RxPool included in sl-ConfigDedicatedNR? | - | - | 3 | - |
| 10 | The SS changes NR Cell 3 parameters according to the row "T1" in Table 12.2.1.6.3.2-1/2. | - | - | - | - |
| 11 | SS-NW sends an RRCReconfiguration message including reconfigurationWithSync and sl-RxPool on NR Cell 1 to order the UE to perform intra handover to NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | UE sends an RRCReconfigurationComplete message on NR Cell 3 to confirm the successful handover. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 13 | Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.31 indicate that the UE monitors NR sidelink reception using sl-RxPool included in the received RRCReconfiguration? | - | - | 4 | - |

12.2.1.6.3.3	Specific message contents

Table 12.2.1.6.3.3-1: SL-BWP-PoolConfigCommon (Preamble, Table 12.2.1.6.3.1-1 and Table 12.2.1.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  |  |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool with condition RX_TX1 as defined in Table 12.2.1.6.3.3-13 | entry 1 | SL-PRECONFIG |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool with condition RX_TX2 as defined in Table 12.2.1.6.3.3-13 |  | SIB-12 |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePoolID-r16 |  | 1 | Index of the resource pool for normal case |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool with condition RX_TX1 as defined in Table 12.2.1.6.3.3-13 |  | SL-PRECONFIG |
| sl-ResourcePool-r16 |  | SL-ResourcePool with condition RX_TX2 as defined in Table 12.2.1.6.3.3-13 |  | SIB-12 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.2.1.6.3.3-2: SIB12 (step 2, Table 12.2.1.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-14 with condition LAST_SEG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB12-r16 ::= SEQUENCE { |  |  |  |
| segmentContainer-r16 | OCTET STRING (CONTAINING SIB12-RESOURCEPOOL) |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-3: SIB12-RESOURCEPOOL (Table 12.2.1.6.3.3-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.2-14A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB12-IEs-r16 ::= SEQUENCE { |  |  |  |
| sl-ConfigCommonNR-r16 SEQUENCE { |  |  |  |
| sl-FreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16{ |  |  |  |
| SL-FreqConfigCommon-r16[1] | SL-FreqConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-4: SL-FreqConfigCommon (Table 12.2.1.6.3.3-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-FreqConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { |  |  |  |
| SL-BWP-ConfigCommon-r16[1] | SL-BWP-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-5: SL-BWP-ConfigCommon (Table 12.2.1.6.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |
| sl-BWP-PoolConfigCommon-r16 | SL-BWP-PoolConfigCommon with condition SIB-12 as defined in Table 12.2.1.6.3.3-1 |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-6: RRCReconfiguration (step 7, Table 12.2.1.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 | SL-ConfigDedicatedNR with condition SCHEDULING as defined in Table 12.2.1.6.3.3-7 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-7: SL-ConfigDedicatedNR (Table 12.2.1.6.3.3-6 and Table 12.2.1.6.3.3-11)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-7 with condition SCHEDULING |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { | one entry |  |  |
| SL-FreqConfig-r16[1] | SL-FreqConfig with condition SCHEDULING as defined in Table 12.2.1.6.3.3-8 |  | SCHEDULING |
| SL-FreqConfig-r16[1] | SL-FreqConfig with condition EXCEPTIONAL as defined in Table 12.2.1.6.3.3-8 |  | EXCEPTIONAL |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-8: SL-FreqConfig (Table 12.2.1.6.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-FreqConfig-r16 ::= SEQUENCE { |  |  |  |
| sl-BWP-ToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-Config-r16 { | one entry |  |  |
| SL-BWP-Config-r16[1] | SL-BWP-Config with condition RX_ SCHEDULINGTX as defined in Table 12.2.1.6.3.3-9 |  | SCHEDULING |
| SL-BWP-Config-r16[1] | SL-BWP-Config with condition RX_ EXCEPTIONALTX as defined in Table 12.2.1.6.3.3-9 |  | EXCEPTIONAL |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-9: SL-BWP-Config (Table 12.2.1.6.3.3-8)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-BWP-Config-r16 ::= SEQUENCE { |  |  |  |
| sl-BWP-PoolConfig-r16 SEQUENCE { |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { | 1 entry |  | RX_ SCHEDULINGTX, RX_ EXCEPTIONALTX |
| SL-ResourcePool-r16[1] | SL-ResourcePool with condition RX_TX2 as defined in Table 12.2.1.6.3.3-13 | entry 1 |  |
| SL-ResourcePool-r16[1] | SL-ResourcePool with condition RX as defined in Table 12.2.1.6.3.3-13 |  | RX_ EXCEPTIONALTX |
| } |  |  |  |
| sl-TxPoolScheduling-r16 | Not Present |  |  |
| sl-TxPoolScheduling-r16 SEQUENCE { |  |  | RX_ SCHEDULINGTX |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-ResourcePoolID-r16 | 1 |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool with condition RX_TX2 as defined in Table 12.2.1.6.3.3-13 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-TxPoolExceptional-r16 | Not Present |  |  |
| sl-TxPoolExceptional-r16 SEQUENCE { |  |  | RX_ EXCEPTIONALTX |
| sl-ResourcePoolID-r16 | 2 |  |  |
| sl-ResourcePool-r16 | SL-ResourcePool with condition EXCEPTIONAL as defined in Table 12.2.1.6.3.3-13 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-10: RRCReconfigurationComplete (step 8 and step 12, Table 12.2.1.6.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-14

Table 12.2.1.6.3.3-11: RRCReconfiguration-HO (step 11, Table 12.2.1.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sl-ConfigDedicatedNR-r16 | SL-ConfigDedicatedNR with condition EXCEPTIONAL as defined in Table 12.2.1.6.3.3-7 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.2.1.6.3.3-12: SidelinkUEInformationNR (step 5, Table 12.2.1.6.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-28A with condition SIDELINK_RX

Table 12.2.1.6.3.3-13: sl-ResourcePool (Table 12.2.1.6.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |
| sl-TimeResource-r16 | 1100000000 |  | RX_TX1 |
| sl-TimeResource-r16 | 0011000000 |  | RX_TX2 |
| sl-TimeResource-r16 | 0000110000 |  | RX |
| sl-TimeResource-r16 | 0000000011 |  | EXCEPTIONAL |
| } |  |  |  |

<End of Change>