3GPP TSG-RAN5 Meeting #97	R5-227473

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2649 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EHC parametrization in 38.508-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_IIoT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update common environment parametrization for EHC testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added DS-TT Ethernet port MAC address and EHC configuration to PDU session establishment request. Added EHC configuration to PDU session accept. Added DNN-APN configuration for ethernet. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EHC testing is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2, 4.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.508-2 CR 0409 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of R5-227047 and one revision of it. |  |  |  |  |  |  |  |  |

### ------------------------------------ Start of first change ---------------------------------------

### 4.7.2	Contents of 5GSM messages

#### –	PDU session establishment request

Table 4.7.2-1: PDU SESSION ESTABLISHMENT REQUEST

| Derivation Path: 24.501 clause 8.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID |  | Any value according to TS 24.501 [25] subclause 9.4 |  |  |
| PTI |  | Any value from 1 to 254 |  |  |
| PDU SESSION ESTABLISHMENT REQUEST message identity |  | ‘1100 0001’B |  |  |
| Integrity protection maximum data rate |  | Present but contents not checked |  |  |
| PDU session type |  | Any value between '001'B, '010'B, '011'B and ‘101’B | The allowed values are respectively IPv4, IPv6, IPv4v6 and Ethernet (EtherType as defined in IEEE 802.3) |  |
| SSC mode |  | If present: contents not checked |  |  |
| 5GSM capability |  | If present: contents not checked |  |  |
| Maximum number of supported packet filters |  | If present: contents not checked |  |  |
| Always-on PDU session requested |  | If present: contents not checked |  |  |
| Always-on PDU session requested |  |  |  | SST_URLLC |
| APSR |  | ‘1’B | Always-on PDU session requested |  |
| SM PDU DN request container |  | If present: contents not checked |  |  |
| Extended protocol configuration options |  | If present: contents not checked | The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of PDU SESSION ESTABLISHMENT ACCEPT. |  |
| Extended protocol configuration options |  | Present including at least the following container |  | DATA_OFF |
| Container ID n |  | ‘0017’H | 3GPP PS data off UE status |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | ‘01’H | ‘deactivated’ |  |
| IP header compression configuration |  | If present: contents not checked |  |  |
| DS-TT Ethernet port MAC address |  | If present: contents not checked |  |  |
| DS-TT Ethernet port MAC address |  | Any value according to TS 24.501 [25] subclause 9.11.4.25 |  | Ethernet |
| UE-DS-TT residence time |  | If present: contents not checked |  |  |
| Port management information container |  | If present: contents not checked |  |  |
| Ethernet header compression configuration |  | If present: contents not checked |  |  |
| Ethernet header compression configuration |  | Any value according to TS 24.501 [25] subclause 9.11.4.28 |  | Ethernet |
| Suggested interface identifier |  | If present: contents not checked |  |  |
| Service-level-AA container |  | If present: contents not checked |  |  |
| Requested MBS container |  | If present: contents not checked |  |  |
| PDU session pair ID |  | If present: contents not checked |  |  |
| RSN |  | If present: contents not checked |  |  |

| Condition | Explanation |
| --- | --- |
| DATA_OFF | If the UE supports 3GPP PS data off as specified in TS 38.508-2 [10], Table A.4.3.7-1/xx |
| Ethernet | If the PDU session type = '101'B. |

### ------------------------------------- End of first change ---------------------------------------

### ----------------------------------- Start of second change -----------------------------------

#### –	PDU session establishment accept

Table 4.7.2-2: PDU SESSION ESTABLISHMENT ACCEPT

| Derivation Path: 24.501 clause 8.3.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0010 1110’B | 5GS session management messages |  |
| PDU session ID | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PTI | The same value as the value set in PDU SESSION ESTABLISHMENT REQUEST message |  |  |
| PDU SESSION ESTABLISHMENT ACCEPT message identity | ‘1100 0010’B |  |  |
| Selected PDU session type | '001'B |  | IPv4 |
|  | '010'B |  | IPv6 |
|  | '011'B |  | IPv4v6 |
|  | '101'B |  | Ethernet |
| Selected SSC mode | ‘001’B | SSC mode 1 |  |
| Authorized QoS rules | 5GC QoS rule of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| Session AMBR |  |  |  |
| Unit for Session-AMBR for downlink | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for downlink | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| Unit for Session-AMBR for uplink | ‘000 00101’ | Value is incremented in multiples of 256 Kbps |  |
| Session-AMBR for uplink | ‘0000 0000 0000 0100’B | 1024 Kbps |  |
| 5GSM cause | Not Present |  |  |
| PDU address |  |  | IPv4 |
| Length of PDU address contents | 5 octets |  |  |
| PDU type value | ‘001’B | IPv4 |  |
| PDU address information | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| PDU address |  |  | IPv6 |
| Length of PDU address contents | 9 octets |  |  |
| PDU type value | ‘010’B | IPv6 |  |
| PDU address information | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address |  |  | IPv4v6 |
| Length of PDU address contents | 13 octets |  |  |
| PDU type value | ‘011’B | IPv4v6 |  |
| PDU address information (Octets 4 to 11) | IPv6 interface identifier | The SS provides a valid IPv6 interface identifier |  |
| PDU address information (Octets 12 to 15) | IPv4 address | The SS provides a valid IPv4 address | NOT IPv4-DHCP |
|  | 0.0.0.0 | DHCPv4 is to be used to allocate the IPv4 address | IPv4-DHCP |
| RQ timer value | Not Present |  |  |
| S-NSSAI |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |
| SST | ‘0000 0001’B | SST value 1 (eMBB) | SST_eMBB |
| SST | ‘0000 0010’B | SST value 2 (URLLC) | SST_URLLC |
| SST | ‘0000 0011’B | SST value 3 (MIoT) | SST_MIoT |
| SST | ‘0000 0100’B | SST value 4 (V2X) | SST_V2X |
| Always-on PDU session indication | Not Present |  |  |
| Always-on PDU session indication |  |  |  |
| APSI | ‘0’B | Always-on PDU session not allowed | Always_On_Requested |
| APSI | ‘1’B | Always-on PDU session required | Always_On_Requested AND SST_URLLC |
| Mapped EPS bearer contexts | Not Present |  |  |
| Mapped EPS bearer contexts |  |  | Interworking_with_EPS |
| Mapped EPS bearer context |  |  |  |
| EPS bearer identity | The same value as the one specified in the Reference QoS flow referred to from the Reference QoS rule indicated in the IE Authorized QoS rules |  |  |
| Operation code | ‘001’B | Create new EPS bearer |  |
| E bit | '1'B | Parameters list is included |  |
| Number of EPS parameters | ’0001’B | 1 parameter |  |
| Mapped EPS QoS parameters | EPC default bearer context of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| EAP message | Not Present |  |  |
| Authorized QoS flow descriptions | The QoS flow referred to in the relevant Authorized QoS rules IE |  |  |
| Extended protocol configuration options | Not Present |  |  |
| Extended protocol configuration options |  |  | P-CSCF_IPv6 OR P-CSCF_IPv4 |
| Container ID 1 | ‘0001’H |  | P-CSCF_IPv6 |
| Length of container ID 1 contents |  | Length value determined by testimplementation |  |
| Container ID 1 contents | IPv6 address | P-CSCF IPv6 Address |  |
| Container ID 2 | ‘000C’H |  | P-CSCF_IPv4 |
| Length of container ID 2 contents |  | Length value determined by test implementation |  |
| Container ID 2 contents | IPv4 address | P-CSCF IPv4 Address |  |
| Container ID 3 | ‘0001’H |  | Additional_P-CSCF_IPv6 |
| Length of container ID 3 contents |  | Length value determined by testimplementation |  |
| Container ID 3 contents | IPv6 address | Additional P-CSCF IPv6 Address |  |
| Container ID 4 | ‘000C’H |  | Additional_P-CSCF_IPv4 |
| Length of container ID 4 contents |  | Length value determined by test implementation |  |
| Container ID 4 contents | IPv4 address | Additional P-CSCF IPv4 Address |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv4 address | DNS IPv4 address |  |
| DNN | The DNN/APN ID of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |  |  |
| 5GSM network feature support | Not Present |  |  |
| Serving PLMN rate control | Not Present |  |  |
| ATSSS container | Not Present |  |  |
| Control plane only indication | Not Present |  |  |
| IP header compression configuration | Not Present |  |  |
| Ethernet header compression configuration | Not Present |  |  |
| Ethernet header compression configuration | Any value according to TS 24.501 [25] subclause 9.11.4.28 |  | Ethernet |
| Service-level-AA container | Not Present |  |  |
| Received MBS container | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '001'B |
| IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '010'B |
| IPv4v6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '011'B |
| IPv4-DHCP | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the IE Extended protocol configuration options contains a configuration protocol option = '000BH' ("IPv4 address allocation via DHCPv4", length of contents = 0). Note: This condition is used in conjunction with IPv4 or IPv4v6 as indicated in the "PDU address information" just above. |
| Ethernet | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message, the PDU session type = '101'B. |
| Always_On_Requested | If the last PDU SESSION ESTABLISHMENT REQUEST message included the Always-on PDU session requested IE |
| P-CSCF_IPv6 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '0001H' (“P-CSCF IPv6 Address Request", length of contents = 0) |
| P-CSCF_IPv4 | If in the last PDU SESSION ESTABLISHMENT REQUEST sent prior to this message the IE Extended protocol configuration options contains a configuration protocol option = '000CH' (“P-CSCF IPv4 Address Request", length of contents = 0) |
| Interworking_with_EPS | If the UE has indicated support of S1, then the SS shall include this IE to provide details for the interworking with EPS being supported for a PDU session. This requirement is set up for the purpose of facilitating the test description. It is not mandatory for the Network to support Mapped EPS bearer contexts. |
| Additional_P-CSCF_IPv6 | P-CSCF_IPv6 AND SS requires to be configured with second P-CSCF Ipv6 address |
| Additional _P-CSCF_IPv4 | P-CSCF_IPv6 AND SS requires to be configured with second P-CSCF Ipv4 address |
| DNS IPv6 | If in the last PDU Session Establishment REQUEST the Extended protocol configuration options was included with a “DNS IPv6 Address Request”. |
| DNS IPv4 | If in the last PDU Session Establishment REQUEST the Extended protocol configuration options was included with a “DNS IPv4 Address Request”. |

### ------------------------------------ End of second change -----------------------------------

### ------------------------------------- Start of third change -------------------------------------

### 4.8.4	DNN/APN configurations

The present subclause provides DNN/APN configurations required for flexible PDU/PDN handling. Table 4.8.4-1 provides configurations for the types on DNN/APN handled in the present version of the test specification. If in the future new PDU types need to be handled, then new DNN/APN configuration(s) may be added.

Table 4.8.4-1: DNN/APN configurations, first set

| Configurations | Config #1 | Config #2 | Config #3 |
| --- | --- | --- | --- |
| DNN/APN type | internet | ims | urllc |
| DNN/APN ID | pc_APN_ID_Internet | pc_APN_ID_IMS | pc_APN_ID_URLLC |
| 5GC QoS rule | Reference QoS rule #1 as specified in subclause 4.8.2.1. | Reference QoS rule #2 as specified in subclause 4.8.2.1. | Reference QoS rule #8 as specified in subclause 4.8.2.1. |
| EPC default bearer context | Reference default EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.1-1. | Reference default EPS bearer context #2 as specified in TS 36.508 [10], Table 6.6.1-1. | N/A |
| EPC dedicated bearer context | Reference dedicated EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.2-1. | Reference dedicated EPS bearer context #4 as specified in TS 36.508 [10], Table 6.6.2-1. | N/A |
| IP address allocation | Yes | Yes | Yes |
| IMS registration | No | Yes NOTE 1 | No |
| SST condition (NOTE 2) | SST_eMBB | SST_eMBB | SST_URLLC |
| NOTE 1: For PDN establishment the Procedure for IMS signalling according to TS 36.508 [2], subclause 4.5A.3 applies; for PDU establishment the Procedure for IMS signalling according to TS 34.229-5 [47], Annex A.2 applies. NOTE 2: The possible values of the SST condition are defined in Table 4.6.0.1-5. |  |  |  |

Table 4.8.4-2: DNN/APN configurations, second set

| Configurations | Config #4 | Config #5 | Config #6 |
| --- | --- | --- | --- |
| DNN/APN type | miot | v2x | ethernet |
| DNN/APN ID | pc_APN_ID_MIoT | pc_APN_ID_V2X | pc_APN_ID_Ethernet |
| 5GC QoS rule | FFS | FFS | Reference QoS rule #1 as specified in subclause 4.8.2.1. |
| EPC default bearer context | N/A | N/A | N/A |
| EPC dedicated bearer context | N/A | N/A | N/A |
| IP address allocation | Yes | Yes | No |
| IMS registration | No | No | No |
| SST condition (NOTE 1) | SST_MIoT | SST_V2X | SST_URLLC |
| NOTE 1: The possible values of the SST condition are defined in Table 4.6.0.1-5. |  |  |  |

### ------------------------------------ End of third change -----------------------------------