3GPP TSG-RAN5 Meeting #97	R5-227474

Toulouse, France, 14th Nov 2022 - 18nd Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0297 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update applicabilities for test cases 8.1.1.4.4-9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SCell activation applicability is missing for test cases 8.1.1.4.4-9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The SCell activation applicability haved been added for test cases 8.1.1.4.4-9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability for test cases 8.1.1.4.4-9 will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | ’RR’ and ‘UU’ shall be resolved according to R5-227475 Table A.4.3.5-1 of TS 38.508-2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

# 4	Recommended Test Case Applicability

## 4.0	Introduction

The applicability of each individual test is identified in subclause 4.1. This is just a recommendation based on the purpose for which the test case was written.

<<<Skipped part >>>

Table 4.1-3a: Applicability of Protocol conformance RRC test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 8 | RRC |  |  |  |
| 8.1 | NR RRC |  |  |  |
| 8.1.1 | RRC connection management procedures |  |  |  |
| 8.1.1.1 | Paging |  |  |  |
| 8.1.1.1.1 | RRC / Paging for connection / Multiple paging records | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.1.2 | RRC / Paging for connection / Shared network environment | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2 | RRC connection establishment |  |  |  |
| 8.1.1.2.1 | RRC connection establishment / Return to idle state after T300 expiry | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.2 | Void |  |  |  |
| 8.1.1.2.3 | RRC connection establishment / RRC Reject with wait time | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.2.4 | RRC connection establishment / Extended and spare fields in SI | Rel-15 only | C21 | UEs supporting 5G Core |
| 8.1.1.3 | RRC release |  |  |  |
| 8.1.1.3.1 | RRC connection release / Redirection to another NR frequency | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.2 | RRC connection release / Redirection from NR to E-UTRA | Rel-15 | C32 | UEs supporting 5G Core and E-UTRA |
| 8.1.1.3.3 | RRC connection release / Success / With priority information | Rel-15 | C21 | UEs supporting 5G Core |
| 8.1.1.3.4 | RRC connection release / Success / With priority information / E-UTRA | Rel-15 | C26 | UEs supporting 5GS and E-UTRA |
| 8.1.1.3.5 | Void |  |  |  |
| 8.1.1.3.6 | Void |  |  |  |
| 8.1.1.3.7 | RRC connection release / Success / Deprioritisation / Frequency / T325 expiry | Rel-15 | C133 | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| 8.1.1.3.7a | RRC connection release / Success / Deprioritisation / NR / T325 expiry | Rel-15 | C148 | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |
| 8.1.1.3.7b | RRC connection release / Success / Deprioritisation / Deletion of Stored deprioritisation request | Rel-15 | C161 | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |
| 8.1.1.4 | RRC resume |  |  |  |
| 8.1.1.4.1 | RRC resume / Suspend-Resume / RNA update / Success | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.2 | RRC resume / Suspend-Resume / RRC setup / T319 expiry | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 8.1.1.4.3 | Void |  |  |  |
| 8.1.1.4.4 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Intra-band Contiguous CA | Rel-16 | C154 | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.5 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Intra-band non-Contiguous CA | Rel-16 | C155 | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.6 | RRC resume / Suspend-Resume / RRC reconfiguration / Active MCG SCell addition / Inter-band CA | Rel-16 | C156 | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR MCG SCell activation |
| 8.1.1.4.7 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band Contiguous CA | Rel-16 | Cyy1 | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |
| 8.1.1.4.8 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band non-Contiguous CA | Rel-16 | Cyy2 | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |
| 8.1.1.4.9 | RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Inter-band CA | Rel-16 | Cyy3 | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR SCG SCell activation |

<<<Skipped part >>>

## 4.2	Protocol conformance test cases Applicability Condition

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

<<<Skipped part >>>

| C154 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/RR THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| --- | --- | --- |
| C155 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/RR THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |
| C156 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/RR THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE- and direct NR MCG SCell activation |
| C157 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |

<<<Skipped part >>>

| C209 | IF A.4.1-5/1 AND FFS THEN R ELSE N/A | UEs supporting 5G Core and RedCap |
| --- | --- | --- |
| C210 | IF A.4.1-5/1 AND FFS AND FFS THEN R ELSE N/A | UEs supporting 5G Core and RedCap and eDRX |
| C211 | IF A.4.3.2-1/85 THEN R ELSE N/A | UEs supporting repetition of Message 3 PUSCH |
| C212 | IF A.4.1-5/1 AND A.4.3.12-1/2 THEN R ELSE N/A | UEs supporting 5G Core and RedCap |
| Cyy1 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/UU THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |
| Cyy2 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/UU THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |
| Cyy3 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/UU THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR SCG SCell activation |

<End of modified section>

##### << informal Note for the implementer: /RR is defined in TS 38:508-2 as : >>

| RR | Support direct NR MCG SCell activation, as specified in TS 38.321, upon reception of an RRCResume message, as specified in TS 38.331. | 38.306, 4.2.6 | Rel-17 | pc_directMCG_SCellActivationResume_r17 | No |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

##### << informal Note for the implementer: /UU is defined in TS 38:508-2 as : >>

| UU | Support direct NR SCG SCell activation, as specified in TS 38.321: - upon reception of an RRCReconfiguration included in an RRCConnectionResume message, as specified in TS 38.331 and TS 36.331, if the UE indicates support of EN-DC or NGEN-DC, and support of resumeWithSCG-Config-r16 as specified in TS 36.331, - upon reception of an RRCReconfiguration included in an RRCResume message, as specified in TS 38.331, if the UE indicates support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. | 38.306, 4.2.6 | Rel-17 | pc_directSCG_SCellActivationResume_r17 | No |  | A UE indicating support of directSCG-SCellActivationResume-r16 shall indicate support of EN-DC or NGEN-DC and support of resumeWithSCG-Config-r16 as specified in TS 36.331 or indicate support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. |
| --- | --- | --- | --- | --- | --- | --- | --- |