3GPP TSG-RAN5 Meeting #97	R5-227479

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3421 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MUSIM test case 9.1.7.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of new MUSIM test case 9.1.7.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 9.1.7.4 as per WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP will be incomplete without addition of this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0295 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-227479 is the final version of R5-227310 with below changes: 1. Updated step 1 of main behavior and added Note 1. 2. Hardcoded Table 4.9.xx.2.2-1 to Table 4.9.36.2.2-1. 3. Hardcoded USIM configurations. 4. Removed RRC signaling related procedures in Main behavior. 5. Updated step 5 to include Check. |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 9.1.7.4	Service request / MUSIM / Rejection of paging

9.1.7.4.1	Test Purpose (TP)

(1)

with { the UE in 5GMM-idle state }

ensure that {

when { UE is configured to reject the paging message }

then { UE transmits SERVICE REQUEST message with Request type set to “Rejection of paging” in the UE request type IE and service type set to “mobile terminated services” }

}

9.1.7.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.6.1.1, 5.6.2.2.1 and 5.6.1.4.1. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 5.6.1.1]

…

p)	the network supports the reject paging request, the MUSIM UE in 5GMM-IDLE mode when responding to paging rejects the paging request from the network, requests the network to release the NAS signalling connection and, if the network supports the paging restriction, optionally includes paging restriction

[TS 24.501, clause 5.6.2.2.1]

…

The MUSIM UE based on implementation may use the paging cause indicated by lower layers (see 3GPP TS 38.331 [30]), if any, to accept the paging, reject the paging or ignore the paging indication.

Upon reception of a paging indication, if the network supports the rejection of paging request and if a MUSIM UE decides not to accept the paging, the UE may initiate a service request procedure to reject the paging as specified in clause 5.6.1.1

[TS 24.501, clause 5.6.1.4.1]

For cases other than h) in subclause 5.6.1.1, the UE shall treat the reception of the SERVICE ACCEPT message as successful completion of the procedure. The UE shall reset the service request attempt counter, stop timer T3517 and enter the state 5GMM-REGISTERED.

9.1.7.4.3	Test description

9.1.7.4.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A (home PLMN1) and cell G (home PLMN2) are configured according to TS 38.508-1 [4] tables 6.3.2.2-1 and 6.3.2.2-3.

UE:

-	The UE is a MUSIM UE.

-	The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4].

9.1.7.4.3.2	Test procedure sequence

Table 9.1.7.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | UE is configured to reject Paging. (Note 1) | - | - | - | - |
| - | The following messages are to be observed on NR Cell G unless explicitly stated otherwise. | - | - | - | - |
| 2 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 3-4 | Steps 2-3 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 5 | Check: Does the UE transmit an RRCSetupComplete message and a SERVICE REQUEST message? | --> | 5GMM: SERVICE REQUEST | 1 | P |
| 6-7 | Steps 5 and 6 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 8 | The SS transmits a SERVICE ACCEPT message. | <-- | 5GMM: SERVICE ACCEPT | - | - |
| 9 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The configuration of Page rejection may be performed by MMI or AT Command. |  |  |  |  |  |

9.1.7.4.3.3	Specific message contents

Table 9.1.7.4.3.3-1: SERVICE REQUEST (step 5, Table 9.1.7.4.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE request type | ‘0010’ | Rejection of paging |  |

<End of modified section>