3GPP TSG-RAN5 Meeting #97	R5-227522

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3269 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eDRX TC 11.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR eDRX test case 11.7.1 needs to be added as part of RedCap WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 11.7.1 - eDRX / IDLE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap WP will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.1 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.523-2 CR 0282 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-226558r1: - Missing Source to TSG added. - Editorial issue fixed in the conformance requirements section and Main behaviour (step1 and Step 10). - Fixed table numbers in specific message contents. - According to 38.331, if NW does not enable the eDRX-AllowedIdle-r17, eDRX-AllowedInactive-r17 in SIB1,UE could not use eDRX in IDLE/INACTIVE for CN-paging and could not use eDRX in INACTIVE mode for RAN paging. So Update the SIB1 in the test case to enable eDRX for IDLE - Set the PTM and eDRX value of Requested extended DRX parameters in the registration request to Present but contents not checked. And set the ePTM value of Requested extended DRX parameters in the the registration request ot Not checked because UE may not include ePTM. R5-226558r2: - Fixed Other Spec affected in coversheet with new CR number. - Fixed table number in specifc message contents. R5-227522: Is the final Tdoc for R5-226558r2 |  |  |  |  |  |  |  |  |

<Start of modified section >

#### 11.7.1	eDRX / IDLE

11.7.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and configured eDRX (eDRX cycle > 10.24s) }

ensure that {

when { network sends Paging message within a Paging Time Window (PTW) }

then { UE establishes RRC connection }

}

(2)

with { UE in NR RRC_IDLE state and configured eDRX (eDRX cycle <= 10.24s) }

ensure that {

when { network sends Paging message within a paging occasion }

then { UE establishes RRC connection }

}

11.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501 clauses 5.3.1, TS 38.304 clause 7.4.

[TS 24.501, clause 5.3.16]

Extended DRX (eDRX) cycle is supported for a UE in N1 mode. When eDRX is requested by the UE and accepted by the network:

-	if the UE is not in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode or in 5GMM-CONNECTED mode with RRC inactive indication; or

-	if the UE is in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode.

The UE may request the use of eDRX cycle during a registration procedure by including the Requested extended DRX parameters IE (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]). The UE shall not request the use of eDRX during a registration procedure for emergency services. The UE may use the extended idle mode DRX cycle length stored in the USIM (see 3GPP TS 31.102 [22]) when requesting the use of eDRX.

The UE and the network may negotiate eDRX parameters during a registration procedure when the UE has an emergency PDU session.

The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session.

NOTE:	If the UE wants to keep using eDRX, the UE includes the Extended DRX parameters IE in each registration procedure.

If the UE received the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the UE shall resume eDRX.

If the network has provided the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the network shall resume eDRX.

If the UE or the network locally releases an emergency PDU session, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a registration procedure with PDU session context synchronization or upon successful completion of a service request procedure with PDU session context synchronization.

If the UE did not receive the Negotiated extended DRX parameters IE, or if the UE has an emergency PDU session, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has an emergency PDU session, the network shall use the stored UE specific DRX parameter, if available.

If the network provided the Negotiated extended DRX parameters IE and also assigned a new 5G-GUTI for the UE as described in subclause 5.5.1.3.4 during the last registration procedure, the network shall use the stored UE specific DRX parameter, if available, with the old 5G-GUTI and use the eDRX provided by the network with the new 5G-GUTI until the old 5G-GUTI can be considered as invalid by the network (see subclauses 5.4.4.4 and 5.5.1.3.4).

[TS 38.304, clause 7.4]

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN. The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1. If the UE is configured with an extended DRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE configured with eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX_CN= (UE_ID_H mod TeDRX_CN), where

-	UE_ID_H: 13 most significant bits of the Hashed ID.

-	TeDRX_CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_CN = 2, …, 1024 Hyper-frames) configured by upper layers.

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX_CN, where

-	ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of 5G-S-TMSI.

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [23].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any 5G-S-TMSI value. An example of hashed ID calculation is in Annex A.

11.7.1.3	Test description

11.7.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE supporting extended DRX and eDRX configured to 5,12 seconds and Paging Time Window configured to 2,56 seconds.

Preamble:

-	UE is in state 1N-A according to TS 38.508-1 [4] Table 4.4A.2-1.

11.7.1.3.2	Test procedure sequence

Table 11.7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message within a paging occasion | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 5-8 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? | - | - | 2 | P |
| 10 | Switch off UE in RRC CONNECTED as described in TS 38.508-1 [4] subclause 4.9.6.3 | - |  |  |  |
| 11 | Switch on UE. | - |  |  |  |
| 12-30 | The generic test procedure in TS 38.508-1 [4] Table 4.5.2.2-2 indicate that the UE performs registration on NR Cell 1. | - | - | - | - |
| 31 | The SS transmits a Paging message in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX. | <-- | NR RRC: Paging | - | - |
| 32 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 33 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 34 | The UE transmit an RRCSetupComplete message including SERVICE REQUEST to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 35-38 | Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 39 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? | - | - | 1 | P |

11.7.1.3.3	Specific message contents

Table 11.7.1.3.3-1: SIB1 (preamble)

| Derivation path: TS 38.508-1 [4] , table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| eDRX-AllowedIdle-r17 |  | true |  |  |
| eDRX-AllowedInactive-r17 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 11.7.1.3.3-2: REGISTRATION REQUEST (preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not Checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

Table 11.7.1.3.3-3: REGISTRATION ACCEPT (preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | ‘00000001’B | 2,56 seconds |  |
| eDRX value | ‘0001’B | 5,12 seconds |  |

Table 11.7.1.3.3-4: REGISTRATION REQUEST (Step15, Table 11.7.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not Checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

Table 11.7.1.3.3-5: REGISTRATION ACCEPT (Step25, Table 11.7.1.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | Any allowed value | Note 1 |  |
| eDRX value | ‘0101’B | 81,92 seconds |  |
| Extended Paging Time Window | ‘00000011’B | 5,12 seconds |  |
| Note 1: In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field. |  |  |  |

<End of modified section >