3GPP TSG-RAN5 Meeting #97	R5-227527

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0169 | rev | 1 | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Include measObject as part of the measConfig message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to clause 5.5.2.1 from 3GPP 38.331, the measConfig message should also include a measObject for the cell to be measured. Current content of table 8.3.1-2 from 3GPP 37.571-2 does not include a measObject (measObjectToAddModList is set to Not present). Therefore, this CR proposes to include a measObject as part of the measConfig message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create Tables 8.3.1-4 and 8.3.1-5 to include the measObjectNR as part of the measConfig from Table 8.3.1-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated based on comments from TF 160 |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 8.3.1-2: MeasConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | MeasObjectToAddModList | Table 8.3.1-4 |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| s-MeasureConfig | Not present |  |  |
| quantityConfig | Not present |  |  |
| measGapConfig | MeasGapConfig | Table 8.3.1-3 |  |
| measGapSharingConfig | Not present |  |  |
| } |  |  |  |

Table 8.3.1-3: MeasGapConfig

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | Not present |  |  |
| gapFR1 | Not present |  |  |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 3 |  |  |
| mgl | ms3 |  |  |
| mgrp | ms160 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1-4: MeasObjectToAddModList

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList ::=SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.3.1-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1-5: MeasObjectNR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::=SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for NR Cell 1 |  |  |
| ssbSubcarrierSpacing | Subcarrier spacing of SSB for NR Cell 1 |  |  |
| smtc1 | Not present |  |  |
| smtc2 | Not present |  |  |
| refFreqCSI-RS | Not present |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility | Not present |  |  |
| csi-rs-ResourceConfigMobility | Not present |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| absThreshCSI-RS-Consolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| nrofCSI-RS-ResourcesToAverage | Not present |  |  |
| quantityConfigIndex | 1 |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB0 |  |  |
| rsrqOffsetSSB | dB0 |  |  |
| sinrOffsetSSB | dB0 |  |  |
| rsrpOffsetCSI-RS | dB0 |  |  |
| rsrqOffsetCSI-RS | dB0 |  |  |
| sinrOffsetCSI-RS | dB0 |  |  |
| } |  |  |  |
| cellsToRemoveList | Not present |  |  |
| cellsToAddModList | Not present |  |  |
| excludedCellsToRemoveList | Not present |  |  |
| excludedCellsToAddModList | Not present |  |  |
| allowedCellsToRemoveList | Not present |  |  |
| allowedCellsToAddModList | Not present |  |  |
| freqBandIndicatorNR | Not present |  |  |
| measCycleSCell | Not present |  |  |
| smtc3list-r16 | Not present |  |  |
| rmtc-Config-r16 | Not present |  |  |
| t312-r16 | Not present |  |  |
| associatedMeasGapSSB-r17 | Not present |  |  |
| associatedMeasGapCSIRS-r17 | Not present |  |  |
| smtc4list-r17 | Not present |  |  |
| measCyclePSCell-r17 | Not present |  |  |
| cellsToAddModListExt-v1710 | Not present |  |  |
| associatedMeasGapSSB2-v1720 | Not present |  |  |
| associatedMeasGapCSIRS2-v1720 | Not present |  |  |
| } |  |  |  |

# < Unchanged sections omitted >