3GPP TSG-RAN5 Meeting #97	R5-227532

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3391 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eDRX TC 11.7.2-eDRX inactive |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 38.331, if NW does not enable the eDRX-AllowedIdle-r17, eDRX-AllowedInactive-r17 in SIB1,UE could not use eDRX in IDLE/INACTIVE for CN-paging and could not use eDRX in INACTIVE mode for RAN paging. - The request eDRX parameter in the registration request is depended on UE implementation. Different UE may have different default value. UE may request different eDRX parameter in registration request but they will use the same eDRX parameter in the registration accept so we could just set the test eDRX parameter in the registration accept. - According to 38.304, if TeDRX, CN(eDRX configured by registration accept) is no long than 10.24s, and no TeDRX,RAN(ran-ExtendedPagingCycle-r17) is configured, the eDRX cycle T is determined by the shortest of UE specific DRX value configured by RRC(ran-PagingCycle) and TeDRX, CN (eDRX configured by registration accept). ran-PagingCycle is configure to 32rf in the 38.508-1 default configuration, so the TeDRX, CN what we configured is not used, UE use the normal DRX in with the cycle configured in ran-PagingCycle. What's more, if TeDRX, CN(eDRX configured by registration accept) and TeDRX,RAN(ran-ExtendedPagingCycle-r17) are no long than 10.24s, the eDRX cycle T is determined by the shortest TeDRX,RAN and TeDRX,CN. Finaly, according to 38.331 the TeDRX,RAN (ran-ExtendedPagingCycle-r17) is not be larger than TeDRX, CN(eDRX configured by registration accept). ran-ExtendedPagingCycle The extended DRX (eDRX) cycle for RAN-initiated paging to be applied by the UE. Value rf256 corresponds to 256 radio frames, value rf512 corresponds to 512 radio frames and so on. Value of the field indicates an eDRX cycle which is shorter or equal to the IDLE mode eDRX cycle configured for the UE. Therefore, we need to configure the TeDRX,RAN = TeDRX, CN and then UE will start to UE eDRX parameter - Step1 is a RAN-paging, so we need to add the condition for the paging. - According to the description in 38.304 “TeDRX, CN is no longer than 1024 radio frames”, it is more reasonable to change the TP1 from eDRX cycle < 10.24s to eDRX cycle ≤10.24s) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SIB1 in the test case to enable eDRX for IDLE and Inactive. Note : Since we only have two eDRX test case 11.7.1/2, so we could only enable eDRX in the specific test case message content. - set the PTM and eDRX value of Requested extended DRX parameters in the registration request to Present but contents not checked. And set the ePTM value of Requested extended DRX parameters in the the registration request ot Not checked because UE may not include ePTM. - configure the TeDRX,RAN in ran-ExtendedPagingCycle. - Add condition NR_RRC_RESUME for step1 paging. - change the TP1 from eDRX cycle < 10.24s to eDRX cycle ≤10.24s |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-227157r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.7.2	eDRX / Inactive

11.7.2.1	Test Purpose (TP)

(1)

with { UE supporting Redcap and in NR RRC_Inactive state and configured eDRX (eDRX cycle ≤ 10.24s) }

ensure that {

when { the network sends Paging message within a paging occassion }

then { the UE resumes RRC connection }

}

11.7.2.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.3.16 and TS 38.304, clause 7.1

[TS 24.501, clause 5.3.16]

Extended DRX (eDRX) cycle is supported for a UE in N1 mode. When eDRX is requested by the UE and accepted by the network:

-	if the UE is not in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode or in 5GMM-CONNECTED mode with RRC inactive indication; or

-	if the UE is in NB-N1 mode, eDRX is used when the UE is in 5GMM-IDLE mode.

The UE may request the use of eDRX cycle during a registration procedure by including the Requested extended DRX parameters IE (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]). The UE shall not request the use of eDRX during a registration procedure for emergency services. The UE may use the extended idle mode DRX cycle length stored in the USIM (see 3GPP TS 31.102 [22]) when requesting the use of eDRX.

The UE and the network may negotiate eDRX parameters during a registration procedure when the UE has an emergency PDU session.

The network accepts the request to use the eDRX by providing the Negotiated extended DRX parameters IE when accepting the registration procedure. The UE shall use eDRX only if it received the Negotiated extended DRX parameters IE during the last registration procedure and the UE does not have an emergency PDU session.

NOTE:	If the UE wants to keep using eDRX, the UE includes the Extended DRX parameters IE in each registration procedure.

If the UE received the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the UE shall resume eDRX.

If the network has provided the Negotiated extended DRX parameters IE during the last registration procedure, upon successful completion of the PDU session release procedure of the emergency PDU session, the network shall resume eDRX.

If the UE or the network locally releases an emergency PDU session, the UE or the network shall not use eDRX until the UE receives eDRX parameters during a registration procedure with PDU session context synchronization or upon successful completion of a service request procedure with PDU session context synchronization.

If the UE did not receive the Negotiated extended DRX parameters IE, or if the UE has an emergency PDU session, the UE shall use the stored UE specific DRX parameter, if available.

If the network did not accept the request to use eDRX, or if the UE has an emergency PDU session, the network shall use the stored UE specific DRX parameter, if available.

If the network provided the Negotiated extended DRX parameters IE and also assigned a new 5G-GUTI for the UE as described in subclause 5.5.1.3.4 during the last registration procedure, the network shall use the stored UE specific DRX parameter, if available, with the old 5G-GUTI and use the eDRX provided by the network with the new 5G-GUTI until the old 5G-GUTI can be considered as invalid by the network (see subclauses 5.4.4.4 and 5.5.1.3.4).

[TS 38.304, clause 7.1]

In RRC_INACTIVE state, if eDRX is configured by RRC, i.e., TeDRX, RAN , and/or upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

-	If both TeDRX, CN and TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN}.

-	If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured, T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN.

…

11.7.2.3	Test description

11.7.2.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE supporting RedCap and extended DRX and eDRX configured to 5,12 seconds and Paging Time Window configured to 2,56 seconds.

Preamble:

-	UE is in state 2N-A according to TS 38.508-1 [4] Table 4.4A.2-2.

11.7.2.3.2	Test procedure sequence

Table 11.7.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message within a paging occassion | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCResumeRequest message? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 3 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 4 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with default Internet PDU session? | - | - | 1 | P |

11.7.2.3.3	Specific message contents

Table 11.7.2.3.3-0: SIB1 (preamble)

| Derivation path: TS 38.508-1 [4] , table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| eDRX-AllowedIdle-r17 |  | true |  |  |
| eDRX-AllowedInactive-r17 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 11.7.2.3.3-1: REGISTRATION REQUEST (preamble)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not Checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

| Table 11.7.2.3.3-2: REGISTRATION ACCEPT (preamble)Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2,56 seconds |  |
| eDRX value | ‘0001’B | 5,12 seconds |  |

Table 11.7.2.3.3-3: RRCRelease (preamble)

| Derivation path: TS 38.508-1 [4], table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |  |
| ran-ExtendedPagingCycle-r17 |  | rf512 | 5.12 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 11.7.2.3.3-4: Paging (step 1, Table 11.7.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

<End of modified section 1>