3GPP TSG-RAN5 Meeting #97	R5-227547

# Toulouse, France, 14 – 18 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3343 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new eNS Test Case for NSAC De-registration rejected NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of new eNS Test Case for NSAC De-registration rejected NSSAI |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case needs to be defined to assure UE interoperability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A UE may fail interoperate with network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.12.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.12.3.3.3	Specific message contents

Table 9.1.12.3.3.3-1: REGISTRATION REJECT (step 15, Table 9.1.12.3.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘0011 1110’B | #62 “No network slices available” |  |
| Extended rejected NSSAI |  |  |  |
| Extended rejected NSSAI IEI |  |  |  |
| Length of Extended rejected NSSAI contents | ‘0101’B |  |  |
| Length of Extended rejected S-NSSAI | ‘0100’B | SST |  |
| Cause value | ‘0011’B | S-NSSAI not available due to maximum number of UEs reached. |  |
| SST | ‘00000001’B | SST value 1 |  |
| SD | ’FFFFFF’H | no SD value associated with the SST |  |

#### 9.1.12.5	NSAC / De-registration / 5GMM cause value #62 and rejected NSSAI

9.1.12.5.1	Test Purpose (TP)

(1)

with {  UE in 5GMM-REGISTERED state and stores one or more S-NSSAIs in the allowed NSSAI}

ensure that {

when {  UE receives a DEREGISTRATION REQUEST message including 5GMM cause value #62 and rejected NSSAI with rejection cause "S-NSSAI not available due to maximum number of UEs reached"}

then { UE shall stores the rejected NSSAI and start the timer T3526 }

}

(2)

with { UE in 5GMM-DEREGISTERED state}

ensure that {

when { UE has neither allowed NSSAI nor configured NSSAI and has rejected NSSAI for the reached maximum number of UEs}

then {  UE stays in the current serving cell and attempt to use the rejected S-NSSAI(s) for the maximum number of UEs reached in the current serving cell after the rejected S-NSSAI(s) are removed}

}

9.1.12.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 5.5.2.3.1, 5.5.2.3.2. Unless otherwise stated these are Rel-17 requirements

[TS 24.501, clause 5.5.2.3.1]

The network initiates the de-registration procedure by sending a DEREGISTRATION REQUEST message to the UE (see example in figure 5.5.2.3.1.1).

NOTE 1:	If the AMF performs a local de-registration, it will inform the UE with a 5GMM messages (e.g. SERVICE REJECT message or REGISTRATION REJECT message) with 5GMM cause #10 "implicitly de-registered" only when the UE initiates a 5GMM procedure.

The network may include a 5GMM cause IE to specify the reason for the DEREGISTRATION REQUEST message. The network shall start timer T3522. The network shall indicate whether re-registration is needed or not in the De-registration type IE. The network shall also indicate via the access type whether the de-registration procedure is:

a)	for 3GPP access only;

b)	for non-3GPP access only; or

c)	for 3GPP access, non-3GPP access or both when the UE is registered in the same PLMN for both accesses.

If the network de-registration is triggered due to network slice-specific authentication and authorization failure or revocation as specified in subclause 4.6.2.4, then the network shall set the 5GMM cause value to #62 "No network slices available" in the DEREGISTRATION REQUEST message. In addition, if the UE supports extended rejected NSSAI, the AMF shall include the Extended rejected NSSAI IE in the DEREGISTRATION REQUEST message; otherwise the AMF shall include the Rejected NSSAI IE in the DEREGISTRATION REQUEST message.

If the UE supports extended rejected NSSAI and the network de-registration is triggered due to mobility management based network slice admission control as specified in subclause 4.6.2.5, then the network shall set the 5GMM cause value to #62 "No network slices available" in the DEREGISTRATION REQUEST message. In addition, the network may include a back-off timer value for each S-NSSAI with the rejection cause "S-NSSAI not available due to maximum number of UEs reached" in the Extended rejected NSSAI IE of the DEREGISTRATION REQUEST message.

If the network de-registration is triggered for a UE supporting CAG due to CAG restrictions, the network shall set the 5GMM cause value to #76 "Not authorized for this CAG or authorized for CAG cells only" and should include the "CAG information list" in the CAG information list IE or the Extended CAG information list IE in the DEREGISTRATION REQUEST message.

[TS 24.501, clause 5.5.2.3.2]

Upon receiving the DEREGISTRATION REQUEST message, if the DEREGISTRATION REQUEST message includes the rejected NSSAI, the UE takes the following actions based on the rejection cause in the rejected S-NSSAI(s):

…

"S-NSSAI not available due to maximum number of UEs reached"

The UE shall add the rejected S-NSSAI(s) in the rejected NSSAI for the maximum number of UEs reached as specified in subclause 4.6.2.2 and shall not attempt to use this S-NSSAI in the current PLMN or SNPN over the current access until switching off the UE, the UICC containing the USIM is removed, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN is updated, or the rejected S-NSSAI(s) are removed as described in subclause 4.6.2.2.

If there is one or more S-NSSAIs in the rejected NSSAI with the rejection cause "S-NSSAI not available due to maximum number of UEs reached", then for each S-NSSAI, the UE shall behave as follows:

a)	stop the timer T3526 associated with the S-NSSAI, if running;

b)	start the timer T3526 with:

1)	the back-off timer value received along with the S-NSSAI, if a back-off timer value is received along with the S-NSSAI that is neither zero nor deactivated; or

2)	an implementation specific back-off timer value, if no back-off timer value is received along with the S-NSSAI; and

c)	remove the S-NSSAI from the rejected NSSAI for the maximum number of UEs reached when the timer T3526 associated with the S-NSSAI expires.

Upon sending a DEREGISTRATION ACCEPT message, the UE shall delete the rejected NSSAI as specified in subclause 4.6.2.2.

9.1.12.5.3	Test description

9.1.12.5.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A belongs to Home PLMN and TAI1 and set as serving cell;

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NGC cell A according to TS 38.508-1 [4].

9.1.12.5.3.2	Test procedure sequence

Table 9.1.12.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS sends a DEREGISTRATION REQUEST message indicates 5GMM cause IE = #62 "No network slices available" including S-NSSAI=1 in rejected NSSAI with back-off timer set to 60 seconds | <-- | 5GMM: DEREGISTRATION REQUEST | - | - |
| 2 | Check: Does the UE transmit a DEREGISTRATION ACCEPT message? Note: Now UE should start timer T3526. | --> | 5GMM: DEREGISTRATION ACCEPT | 1 | P |
| 3 | The SS releases the RRC connection. | - | - | - | - |
| 4 | Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause ‘S-NSSAI not available due to maximum number of UEs reached’’ associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? | - | - | 1 | P |
| 5 | The SS waits 60 seconds for T3526 to expire. | - | - | - | - |
| 6-23a1 | Does the UE perform Steps 2-19a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4]? | - | - | 2 | P |

9.1.12.5.3.3	Specific message contents

Table 9.1.12.5.3.3-1: DEREGISTRATION REQUEST (step 1, Table 9.1.12.5.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.7.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | 62 | No network slices available |  |
| Extended rejected NSSAI |  |  |  |
| Length of Extended rejected NSSAI contents | 1 entry |  |  |
| Partial extended rejected NSSAI list |  | entry 1 |  |
| Type of list | ‘001’B | S-NSSAIs with back-off timer |  |
| Number of elements | ‘0000’B | 1 element |  |
| Back-off timer value | ’10000010’B | 60 seconds |  |
| Rejected S-NSSAI 1 |  | S-NSSAI value 1 |  |
| Length of rejected S-NSSAI | ‘00000001’B | SST |  |
| Cause value | ‘0011’B | S-NSSAI not available due to maximum number of UEs reached |  |
| SST | ‘00000001’B | SST value 1 |  |
| SD | ’FFFFFF’H | no SD value associated with the SST |  |