3GPP TSG-RAN5 Meeting #97	R5-227554

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3352 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of testcase 8.2.6.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add EN-DC testcase 8.2.6.3.1 for early measurement reporting in which measurement configuration is provided through SIB5 and SIB24. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New testcase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early measurement reporting not tested for EN-DC use case where configuration is provided through SIB5 and SIB24. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0286 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of R5-227037 and two revisions of it. |  |  |  |  |  |  |  |  |

##### ------------------------------------------------------ Start of changes ------------------------------------------------------

##### 8.2.6.3.1	Idle/Inactive measurements / Idle mode / EN-DC / SIB5 & SIB24 configuration

8.2.6.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state after receiving RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR while being connected to a serving cell in which SIB2 includes idleModeMeasurementsNR and SIB5 is configured with measIdleCarrierListNR with ssb-MeasConfig and UE supports idle/inactive measurements for NR }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection re-establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in RRC_IDLE state after receiving RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR while being connected to a serving cell in which SIB2 includes idleModeMeasurementsNR and SIB5 is configured with measIdleCarrierListNR without ssb-MeasConfig and SIB24 is configured with ssb-MeasConfig parameters and UE supports idle/inactive measurements for NR }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection re-establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(3)

with { UE in RRC_IDLE state after receiving RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR while being connected to a serving cell in which SIB5 is not configured and UE supports idle/inactive measurements for NR }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and RRC connection re-establishment }

then { UE does not report measurement results in UEInformationResponse message }

}

8.2.6.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331 clauses 5.3.8.3, 5.6.5.3 and 5.6.20. Unless otherwise stated, these are Rel-16 requirements.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCConnectionRelease message includes the measIdleConfig:

2>	clear VarMeasIdleConfig and VarMeasIdleReport;

2>	store the received measIdleDuration in VarMeasIdleConfig;

2>	start or restart T331 with the value of measIdleDuration;

NOTE 2:	If the measIdleConfig contains neither measIdleCarrierListEUTRA nor measIdleCarrierListNR, UE may receive measIdleCarrierListEUTRA and/or measIdleCarrierListNR as specified in 5.6.20.1a.

[TS 36.331, clause 5.6.20.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE or RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 36.331, clause 5.6.20.1a]

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED; or

1>	upon update of system information (SIB5, or SIB24), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCConnectionRelease message:

2>	if the UE is capable of idle/inactive measurements for NR:

3>	if the SIB5 includes the measIdleConfigSIB-NR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB-NR of SIB5 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCConnectionRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB-NR of SIB5 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB5 into maxRS-IndexCellQual, threshRS-Index, measTimingConfig, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in carrierFreqListNR of SIB24 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB24 into maxRS-IndexCellQual, threshRS-Index, measTimingConfig, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

[TS 36.331, clause 5.6.20.2]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [89].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the SIB2 contains idleModeMeasurementsNR and VarMeasIdleConfig includes the measIdleCarrierListNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports (NG)EN-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreqNR and subCarrierSpacingSSB within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreqNR and subCarrierSpacingSSB within the corresponding entry;

5>	if the reportQuantitiesNR is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive the cell measurement results for the measurement quantities indicated by reportQuantitiesNR;

5>	store the derived measurement results as indicated by reportQuantitiesNR within the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThresholdNR is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThresholdNR;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports nr-IdleInactiveBeamMeasFR1 or nr-IdleInactiveBeamMeasFR2 for the FR of the carrier frequency indicated by carrierFreqNR within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-IndexNR, as described in TS 38.215 [89];

6>	if the reportQuantityRS-IndexNR is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultRS-IndexList to include up to maxReportRS-Index SS/PBCH block indexes in order of decreasing sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if threshRS-Index is included, the remaining beams whose sorting quantity is above threshRS-Index;

6>	if the reportRS-IndexResultsNR is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-IndexNR;

3>	if, as the result of the procedure in this clause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR:

4>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdle in VarMeasIdleReport;

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signaling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 36.331, clause 5.6.20.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig;

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB5 and SIB24 configuration or according to NR SIB11 and NR SIB4 configuration as specified in TS 38.331 [82] upon inter-RAT cell reselection to NR, after T331 has expired or stopped.

[TS 36.331, clause 5.6.5.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	except for NB-IoT, if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultListIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

2>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.2.6.3.1.3	Test Description

8.2.6.3.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1, serving cell. System information combination 30 as defined in 36.508 [7] clause 4.4.3.1.1, with additions to SIB2 specified in table 8.2.6.3.1.3.3-7 and SIB5 specified in table 8.2.6.3.1.3.3-8.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	None.

Preamble:

-	The UE is in state 3 according to TS 36.508 [2], clause 4.5.3.

8.2.6.3.1.3.2	Test procedure sequence

Table 8.2.6.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | RRCConnectionRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 3-10 | Steps 2 to 9 of procedure described in TS 36.508 table 4.5.3.3-1: Generic Radio Bearer establishment procedure (state 2 to state 3) are performed. | - | - | - | - |
| 11 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | UEInformationResponse | 1 | P |
| 13 | The SS changes system information by removing ssb-MeasConfig from SIB5 specified in table 8.2.6.3.1.3.3-8 and adding SIB24 with ssb-MeasConfig parameters specified in table 8.2.6.3.1.3.3-11. | - | - | - | - |
| 14 | Transmit a Paging message with systemInfoModification = true | <-- | Paging | - | - |
| 15 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 16 | The SS transmits RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | RRCConnectionRelease | - | - |
| 17 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 18-25 | Steps 2 to 9 of procedure described in TS 36.508 table 4.5.3.3-1: Generic Radio Bearer establishment procedure (state 2 to state 3) are performed. | - | - | - | - |
| 26 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | UEInformationRequest | - | - |
| 27 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | UEInformationResponse | 2 | P |
| 28 | The SS changes system information combination to combination 1 defined in TS 36.508 [7] clause 4.4.3.1.2, with additions to SIB2 specified in table 8.2.6.3.1.3.3-7. | - | - | - | - |
| 29 | Transmit a Paging message with systemInfoModification = true | <-- | Paging | - | - |
| 30 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 31 | The SS transmits RRCConnectionRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | RRCConnectionRelease | - | - |
| 32 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 33-40 | Steps 2 to 9 of procedure described in TS 36.508 table 4.5.3.3-1: Generic Radio Bearer establishment procedure (state 2 to state 3) are performed. | - | - | - | - |
| 41 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | UEInformationRequest | - | - |
| 42 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | --> | UEInformationResponse | 3 | P |

8.2.6.3.1.3.3	Specific message contents

Table 8.2.6.3.1.3.3-1: RRCConnectionRelease (steps 1, 16 and 31, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], clause 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r15 | MeasIdleConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-2: MeasIdleConfigDedicated (steps 1, 16 and 31, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.331 [11], clause 6.3.5 - MeasIdleConfig |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r15 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | Not Present |  |  |
| measIdleDuration-r15 | sec120 |  |  |
| measIdleCarrierListNR-r16 | Not Present |  |  |
| validityAreaList-r16 | Not Present |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-3: UEInformationRequest (steps 11, 26 and 41, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], clause 4.6.1 table 4.6.1-23A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r15 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-4: UEInformationResponse (steps 12 and 27, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], clause 4.6.1 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measResultListIdleNR-r16 | MeasResultListIdleNR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-5: UEInformationResponse (step 42, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.508 [7], clause 4.6.1 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measResultListIdleNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-6: MeasResultListIdleNR (steps 12 and 27, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.331 [11], clause 6.3.5 - MeasResults |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultListIdleNR-r16 ::= SEQUENCE (SIZE (1..maxIdleMeasCarriers-r16)) OF MeasResultIdleNR-r16 { | 1 entry |  |  |
| MeasResultIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreqNR-r16 |  | ARFCN value corresponding to NR cell 3 |  |  |  |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r15)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellIdNR-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrpResultNR-r16 | Any allowed value |  |  |
| rsrqResultNR-r16 | Any allowed value |  |  |
| resultRS-IndexList-r16 SEQUENCE (SIZE (1..maxRS-IndexReport-r15)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-7: SIB2 (Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.308 [7], clause 4.4.3.3 - SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| idleModeMeasurementsNR-r16 | true |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-8: SIB5 (Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.308 [7], clause 4.4.3.3 – SystemInformationBlockType5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| measIdleConfigSIB-NR-r16 | MeasIdleConfigSIB-NR-r16 |  |  |
| } |  |  |  |

Table 8.2.6.3.1.3.3-9: MeasIdleConfigSIB-NR (preamble, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.331 [11], clause 4.4.3.3 – SystemInformationBlockType5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment |  | Condition |  |
| MeasIdleConfigSIB-NR-r16 ::= SEQUENCE { |  |  |  |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierNR-r16 { |  |  |  |  |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  |  |  |  |  |  |
| carrierFreq-r16 |  | ARFCN value of configured NR cell |  |  |  |  |  |
| ssbSubcarrierSpacing-r16 |  | SubcarrierSpacing |  | 38.508-1, Table 4.6.3-188 |  |  |  |
| frequencyBandList |  | Not present |  |  |  |  |  |
| measCellListNR-r16 |  | Not present |  |  |  |  |  |
| reportQuantities-r16 |  | both |  |  |  |  |  |
| qualityThreshold-r16 |  | Not present |  |  |  |  |  |
| ssb-MeasConfig-r16 SEQUENCE { |  |  |  |  |  |  |  |
| nrofSS-BlocksToAverage-r16 |  | 2 |  |  |  |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |  |  |  |  |
| thresholdRSRP |  | RSRP-Range |  | 38.508-1, Table 4.6.3-152 |  |  |  |
| thresholdRSRQ |  | Not present |  |  |  |  |  |
| thresholdSINR |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| smtc-r16 |  | SSB-MTC |  | 38.508-1, Table 4.6.3-185 |  |  |  |
| ssb-ToMeasure-r16 |  | SSB-ToMeasure |  | 38.508-1, Table 4.6.3-187 |  |  |  |
| deriveSSB-IndexFromCell-r16 |  | false |  |  |  |  |  |
|  |  | true |  |  |  | FR1_TDD, FR2_TDD |  |
| ss-RSSI-Measurement-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| beamMeasConfigIdle-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 8.2.6.3.1.3.3-10: MeasIdleConfigSIB-NR (step 13, Table 8.2.6.3.1.3.2-1)

| Derivation Path: TS 36.331 [11], clause 4.4.3.3 – SystemInformationBlockType5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigSIB-NR-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r15)) OF MeasIdleCarrierNR-r16 { |  |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN value of configured NR cell |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1, Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 | Not present |  |  |
| beamMeasConfigIdle-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### ------------------------------------------------------- End of changes -----------------------------------------------------