3GPP TSG-RAN5 Meeting #97	R5-227599

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1512 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS testcase 15.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Purpose of this test case is to test a Consultative Call Transfer, being the UE tested the Transferor. At the preamble a MO call is established and then an MMI command is set to start the call transfer (call A will be put On Hold and a new call will be established) as it is Consultative an additional action should be taken on UE side to finish the transfer procedure, transfer confirmation. At the moment, test case implementation is missing the transfer call confirmation MMI, translated as AT command with AT+CHLD=4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a step to include a MMI command to complete the Call Transfer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is final version of R5-227278. |  |  |  |  |  |  |  |  |

## 15.25	MO Explicit Communication Transfer – Consultative Call Transfer

### 15.25.1	Definition

Test to verify that the transferor UE correctly performs IMS Multimedia Telephony Consultative Explicit Communication Transfer (ECT). This process is described in 3GPP TS 24.629 [104].

### 15.25.2	Conformance requirement

[TS 24.629 Rel 12, clause 4.5.2.1]:

A UE that initiates a transfer operation shall:

-	issue a REFER request in the original communications dialog as specified in RFC 3515, where:

a)	the request URI shall contain the SIP URI of the transferee as received in the Contact header field.

b)	the Refer-To header field shall indicate the public address of the transfer Target.

c)	in case of consultative transfer, the transferor UE has a consultation communication with the transfer Target, a Replaces header field parameter shall be added to the Refer-To URI together with a Require=replaces header field parameter.

d)	the Referred-By header field can be used to indicate the identity of the transferor. When privacy was required in the original communications dialog and a Referred-By header field is included, the UE shall include a Privacy header field set to "user".

After the REFER request is accepted by the other end with a 2xx response, the transferor UE should get notifications of how the transferee's communication setup towards the transfer Target is progressing.

When a NOTIFY request is received on the REFER dialog that indicates that the transferee and the transfer Target have successfully setup a communication, the transferor UE may terminate the original communication with the transferee UE, by sending a BYE message on the original dialog.

Reference(s)

3GPP TS 24.629 [104], clause 4.5.2.1.

### 15.25.3	Test purpose

1)	To verify that the transferor UE puts the call on hold before the transfer with a correct exchange of SIP/SDP protocol signalling messages; and

2)	To verify that the transferor UE has a consultative communication with the transfer target UE; and

3)	To verify that the transferor UE issues a correctly composed REFER request to initiate the call transfer; and

4)	To verify that the transferor UE correctly processes the NOTIFYs from the transferee UE; and

5)	To verify that the transferor UE correctly processes the BYE request releasing the call with the transfer target UE.

### 15.25.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up an MO call, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step and thereafter executing the generic test procedure in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14 for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1).

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-4)	UE is in an active call with the SS (simulating the transferee). Consultative Call Transfer is initiated at the UE. UE puts the ongoing call on hold.

5-16)	UE sets up an MO call with the transfer target (also simulated by the SS).

17-20)	UE puts the call with the transfer target UE on hold.

21)	SS waits for UE to send a REFER request to the transferee within the existing dialog between the UE and the transferee.

22)	SS responds to the REFER request with a valid 200 OK response.

23)	 SS sends UE an initial NOTIFY to indicate that the implicit refer subscription is pending.

24)	SS waits for UE to respond to NOTIFY with valid 200 OK response.

25-28)	 Call between UE and the transferee UE is put on hold by SS.

29)	SS releases call between UE and the transfer target by sending a BYE request.

30)	SS waits for UE to respond to the BYE request with valid 200 OK response.

31)	SS sends UE the final NOTIFY to indicate that the call transfer was successfully completed.

32)	SS waits for UE to respond to NOTIFY with valid 200 OK response.

33)	UE may send a BYE request to release the call with the transferee UE.

34)	If UE has sent a BYE request in Step 33, SS responds to this request with valid 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  | Make the UE put the call on hold (Note 1) |  |
| 1-4 |  |  | Steps defined in Annex C.8 to hold the call | UE holds the call with the transferee. |
| 5-16 |  |  | Steps 2-13 defined in Annex C.21 | In order to establish a call with the transfer target, the same messages as in Annex C.21 are used. |
| 16A |  |  | Make the UE confirm the Consultative Call Transfer (Note 2) |  |
| 17-20 |  |  | Steps defined in Annex C.8 to hold the call | UE holds call with transfer target. |
| 21 |  |  | REFER | The UE sends REFER to SS, simulating the transferee, referring to the transfer target |
| 22 |  |  | 200 OK | The SS responds to REFER with 200 OK |
| 23 |  |  | NOTIFY | The SS, simulating the transferee, sends initial NOTIFY for the implicit subscription created by the REFER request |
| 24 |  |  | 200 OK | The UE responds to NOTIFY with 200 OK |
| 25-28 |  |  | Steps defined in Annex C.9 to hold the call | The SS, simulating the transferee, puts the UE on hold, setting the direction attribute to inactive. |
| 29 |  |  | BYE | The SS, simulating the transfer target, releases the call between UE and the transfer target with BYE |
| 30 |  |  | 200 OK | The UE responds to BYE with 200 OK |
| 31 |  |  | NOTIFY | The SS, simulating the transferee, sends a NOTIFY to confirm that the call transfer has been completed |
| 32 |  |  | 200 OK | The UE responds to NOTIFY with 200 OK |
| 33 |  |  | BYE | Optional: UE may send BYE request to release call with the transferee |
| 34 |  |  | 200 OK | If the UE has sent BYE in step 33 then SS sends 200 OK for BYE |
| Note 1 : This could be done by e.g. MMI or AT command(AT+CHLD=2). Note 2 : This could be done by e.g. MMI or AT command (AT+CHLD=4). |  |  |  |  |

Specific Message Contents

INVITE(Step 5 resp Step 2 of C.21)

Use the default message “INVITE” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeUri2 px_IMS_CalleeUri2 may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting. |
| To |  |
| addr-spec | px_IMS_CalleeUri2 |

183 Session in Progress for INVITE (Step 7 resp Step 4 of C.21)

Use the default message “183 Session in Progress for INVITE” in annex A.2.3 with the following exceptions:

| Header/param |  | Value/remark |  |
| --- | --- | --- | --- |
| Contact |  |  |  |
| addr-spec |  | px_IMS_CalleeContactUri2 |  |
| Message-body |  | o=- 1111111112 1111111111 IN (addrtype) (unicast-address for SS) |  |

180 Ringing for INVITE (Step 12 resp Step 9 of C.21)

Use the default message “180 Ringing for INVITE” in annex A.2.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

200 OK for INVITE (Step 15 resp Step 12 of C.21)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | px_IMS_CalleeContactUri2 |

Messages in Steps 17-20

Messages in Steps 17-20 are the same as those specified in Annex C.8 with the following exceptions:

INVITE or UPDATE (Step 17) using condition A5 of A.2.1 (respectively corresponding requirements when using UPDATE) and with the following exceptions

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | px_IMS_CalleeContactUri2 |

REFER (Step 21)

Use the default message “MO REFER” in annex A.2.10 on the first dialog created in the preamble with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Refer-To |  |
| value | <public address of transfer target?Replaces=(dialog id of the dialog between the UE and the transfer target)&Require=replaces> |
| Referred-By |  |
| value | same value as addr-spec field in From header in the first INVITE during initial call setup, if header present |
| Privacy |  |
| value | user (shall be included if privacy was required during original communication dialog and Referred-By header field is included) |

NOTIFY (Step 23)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | SIP/2.0 100 Trying |

200 OK for NOTIFY (Step 24)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

Messages in Steps 25-28

Messages in Steps 25-28 are the same as those specified in Annex C.9 with the following exceptions:

-	each media line shall carry direction attribute “a=inactive”.

NOTIFY (Step 31)

Use the default message “MT NOTIFY for refer package” in annex A.2.11 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Subscription-State |  |
| substate-value | Terminated |
| expires | omitted from the request |
| reason | Noresource |
| Message-body | SIP/2.0 200 OK |

200 OK for NOTIFY (Step 32)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

### 15.25.5	Test requirements

None additional.