3GPP TSG-RAN5 Meeting #97	R5-227737

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1924 | rev | 1 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Table 6.5A.2.2.1.4.1-1 definition of powerBoostPi2BPSK setting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition of powerBoostPi2BPSK setting is missing in Table 6.5A.2.2.1.4.1-1, that make confuse for test case implemention 1: the p-Max of test ID1,2 was defined as 16dBm 2: For the SEM test the measurement UE mean power should fulfil requirement in table 6.2A.2.1.5-1, if the powerBoostPi2BPSK set to 0, the UE mean power will fail 20-TT power limitation (mapping to test ID6,7) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the defination of powerBoostPi2BPSK setting of test ID1,2 in Table 6.5A.2.2.1.4.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case failed while implemnet incorrect powerBoostPi2BPSK setting |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.2.2.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-226228r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

Table 6.5A.2.2.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.5A.3-1 |  |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  |  | PCC | SCC |
| 13 | Low | N/A | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left | Edge_1RB_Left |
| 23 | High |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right | Edge_1RB_Right |
| 3 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 4 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 5 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 6 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 7 | Default |  | DFT-s-OFDM 64 QAM | Outer_Full | Outer_Full |
| 8 | Default |  | DFT-s-OFDM 256 QAM | Outer_Full | Outer_Full |
| 9 | Low |  | CP-OFDM QPSK | Edge_1RB_Left | Edge_1RB_Left |
| 10 | High |  | CP-OFDM QPSK | Edge_1RB_Right | Edge_1RB_Right |
| 11 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left | Edge_1RB_Left |
| 12 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right | Edge_1RB_Right |
| 13 | Default |  | CP-OFDM 64 QAM | Outer_Full | Outer_Full |
| 14 | Default |  | CP-OFDM 256 QAM | Outer_Full | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1 and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

<End of Changes>