3GPP TSG-RAN5 Meeting #97	R5-227777

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0824 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 Redcap UL configuration and UE type definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FR2 redcap common uplink configuration and UE type definiation has not yet been added in the TS 38.521-2 spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added FR2 Redcap PC7 common UL configuration and UE type definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 Redcap work item will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-226782 -> r1 -> R5-227777 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-226782r1: resolving 3GU issue of missing clauses affected R5-227777: removed UE types 5 & 6 from Table 6.2.1.0-1 |  |  |  |  |  |  |  |  |

< Start of changes >

## 6.1	General

Editor’s Note: Test configurations/environments that require new spherical scan shall be included in test procedure section and identifying such scenarios is currently FFS and owned by RAN5.

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA) with a single or multiple transmit chains.

Unless otherwise stated, for power class 3 UEs, the beam correspondence side condition for SSB and CSI-RS specified in subclause 6.6 shall apply to the transmission tests.

Transmitter requirements for CA operation apply only when the DMRS initialization parameters (including the case when the UE applies cell ID as DMRS scrambling ID) are different across all CCs. The UE may use higher MPR values outside this limitation.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

For Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated, Channel Bandwidth shall be prioritized in the selecting of test points. Subcarrier spacing shall be selected after Test Channel Bandwidth is selected.

Uplink RB allocations given in Table 6.1-1 and Table 6.1-2 are used throughout this section, unless otherwise stated by the test case.

The UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

Table 6.1-1: Common Uplink Configuration for PC2, PC3, PC4 and PC7

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full (Note 1) | Inner_1RB_Left | Inner_1RB_Right | Inner_Partial_Left | Inner_Partial_Right | Inner_Partial2_Left |  | Inner_Partial2_Right |
| 50MHz | 60 | DFT-s | 64@0 | 1@0 | 1@65 | 20@223 20@204 | 1@223 1@14 | 1@433 1@644 | 4@223 8@84 | 4@403 8@504 | 6@64 |  | 6@544 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 1@223 1@14 | 1@433 1@644 | 4@223 7@74 | 4@403 7@524 | 6@64 |  | 6@544 |
|  | 120 | DFT-s | 32@0 | 1@0 | 1@31 | 10@113 10@104 | 1@113 1@14 | 1@213 1@304 | 4@113 4@44 | 4@183 4@244 | 3@34 |  | 3@264 |
|  |  | CP | 32@0 | 1@0 | 1@31 | 11@113 10@104 | 1@113 1@14 | 1@213 1@304 | 4@113 4@44 | 4@183 4@244 | 3@34 |  | 3@264 |
| 100MHz | 60 | DFT-s | 128@0 | 1@0 | 1@131 | 40@443 40@404 | 1@443 1@14 | 1@873 1@1304 | 4@443 8@84 | 4@843 8@1164 | 6@64 |  | 6@1204 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 1@443 1@14 | 1@873 1@1304 | 4@443 7@74 | 4@843 7@1184 | 6@64 |  | 6@1204 |
|  | 120 | DFT-s | 64@0 | 1@0 | 1@65 | 20@223 20@204 | 1@223 1@14 | 1@433 1@644 | 4@223 4@44 | 4@403 4@584 | 3@34 |  | 3@604 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 1@223 1@14 | 1@433 1@644 | 4@223 4@44 | 4@403 4@584 | 3@34 |  | 3@604 |
| 200MHz5 | 60 | DFT-s | 256@0 | 1@0 | 1@263 | 81@883 81@814 | 1@883 1@14 | 1@1753 1@2624 | 4@883 8@84 | 4@1723 8@2484 | 6@64 |  | 6@2524 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 88@88 | 1@883 1@14 | 1@1753 1@2624 | 4@883 7@74 | 4@1723 7@2504 | 6@64 |  | 6@2524 |
|  | 120 | DFT-s | 128@0 | 1@0 | 1@131 | 40@443 40@404 | 1@443 1@14 | 1@873 1@1304 | 4@443 4@44 | 4@843 4@1244 | 3@34 |  | 3@1264 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 1@443 1@14 | 1@873 1@1304 | 4@443 4@44 | 4@843 4@1244 | 3@34 |  | 3@1264 |
| 400MHz5 | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
|  | 120 | DFT-s | 256@0 | 1@0 | 1@263 | 64@66 | 1@66 | 1@197 | 4@66 | 4@194 | N/A |  | N/A |
|  |  | CP | 264@0 | 1@0 | 1@263 | 66@66 | 1@66 | 1@197 | 4@66 | 4@194 | N/A |  | N/A |
| Note 1: RB allocation is left aligned within inner region. Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 inner allocation defined in 6.2.2.3.3; Inner_Partial_Left and Inner_Partial_Right are selected as partial allocation within Region 1 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.3; Inner_Partial2_Left and Inner_Partial2_Right are selected as partial allocation within Region 1 inner allocation which are impacted by MPRnarrow defined in 6.2.2.3.3 when MPRnarrow=2 dB. Note 3: Applicable to Rel-15 PC3 devices which do not support modifiedMPR-Behaviour bit 0 capability (according to Annex P.1) and to Rel-15 and forward PC2 and PC4 devices.. Note 4: Applicable to Rel-15 PC3 devices which supports modifiedMPR-Behaviour bit 0 capability (according to Annex P.1) and Rel-16 and forward PC3 devices. Note 5: The 200MHz and 400MHz bandwidths are not applicable to PC7 RedCap UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1-2: Common Uplink Configuration for PC1

| Channel Bandwidth | SCS(kHz) | OFDM | RB allocation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Outer_Full | Outer_1RB_Left | Outer_1RB_Right | Inner_Full_Region1 | Innner_partial_Left_Region1 | Inner_Partial_Right_Region1 | Inner_Full_Region2 | Innner_Partial_Left_Region2 | Inner_Partial_Right_Region2 |
| 50MHz | 60 | DFT-s | 64@0 | 1@0 | 1@65 | 20@22 | 16@22 | 16@28 | 32@16 | 16@8 | 16@42 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 16@22 | 16@28 | 33@16 | 16@8 | 16@42 |
|  | 120 | DFT-s | 32@0 | 1@0 | 1@31 | 10@11 | 8@11 | 8@14 | 16@8 | 8@4 | 8@20 |
|  |  | CP | 32@0 | 1@0 | 1@31 | 11@11 | 8@11 | 8@14 | 16@8 | 8@4 | 8@20 |
| 100MHz | 60 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 16@44 | 16@72 | 64@32 | 16@8 | 16@108 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 16@44 | 16@72 | 66@33 | 16@8 | 16@108 |
|  | 120 | DFT-s | 64@0 | 1@0 | 1@65 | 20@23 | 8@22 | 8@36 | 32@16 | 8@4 | 8@54 |
|  |  | CP | 66@0 | 1@0 | 1@65 | 22@22 | 8@22 | 8@36 | 33@16 | 8@4 | 8@54 |
| 200MHz | 60 | DFT-s | 256@0 | 1@0 | 1@263 | 81@88 | 16@88 | 16@160 | 128@64 | 16@8 | 16@240 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 88@88 | 16@88 | 16@160 | 132@66 | 16@8 | 16@240 |
|  | 120 | DFT-s | 128@0 | 1@0 | 1@131 | 40@44 | 8@44 | 8@80 | 64@32 | 8@4 | 8@120 |
|  |  | CP | 132@0 | 1@0 | 1@131 | 44@44 | 8@44 | 8@80 | 66@33 | 8@4 | 8@120 |
| 400MHz | 60 | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | CP | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 120 | DFT-s | 256@0 | 1@0 | 1@263 | 64@66 | 8@66 | 8@190 | 128@64 | 8@4 | 8@252 |
|  |  | CP | 264@0 | 1@0 | 1@263 | 66@66 | 8@66 | 8@190 | 132@66 | 8@4 | 8@252 |
| Note 1: RB allocation is left aligned within inner region 1 or inner region 2 as defined in clause 6.2.2.3.1. Note 2: Inner_Full allocation is selected as the largest RB allocation within Region 1 or Region 2 inner allocation defined in 6.2.2.3.1; Inner_partial_Left and Inner_partial_Right are selected as minimum allocation within Region 1 or Region 2 inner allocation which are not impacted by MPRnarrow defined in 6.2.2.3.1. |  |  |  |  |  |  |  |  |  |  |  |

## 6.2	Transmit power

### 6.2.1	UE maximum output power

#### 6.2.1.0	General

Note: Power class 1, 2, 3, and 4 are specified based on the assumption of certain UE types with specific device architectures. The UE types can be found in Table 6.2.1.0-1.

Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

< End of changes >