3GPP TSG-WG5 Meeting #97	R5-227780

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0849 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on EIS spherical coverage for inter-band DL CA Power Class 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Hungary kft. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adittional NR CA band combinations were defined for EIS spherical coverage requirement relaxation for inter-band CA for power class 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Combos n257-n259, n258-n260, n258-n261 were added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements harmonizarion between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3.0.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-227316r1, minor typo correction in the cover page This contribution was withdrawn |  |  |  |  |  |  |  |  |

## <Start of Change>

### 7.3A.3	EIS spherical coverage for DL CA

#### 7.3A.3.0	Minimum Conformance Requirements

##### 7.3A.3.0.1	Void

##### 7.3A.3.0.2	Void

##### 7.3A.3.0.3	EIS spherical coverage for inter-band CA

The inter-band CA requirement applies per operating band, for all active component carriers with UL assigned to one band and one DL component carrier per band. The requirement on each component carrier shall be met when the power in the component carrier in the other band is set to its EIS spherical coverage requirement for inter-band CA specified in this sub-clause.

The inter-band CA spherical coverage requirement for each power class will be satisfied if the intersection set of spherical coverage areas exceeds the common coverage requirement. Intersection set of spherical coverage areas is defined as a fraction of area of full sphere measured around the UE where both bands meet their defined individual EIS spherical coverage requirements for inter-band CA operation. The common coverage requirement is determined as <100-percentile rank> %, where ‘percentile rank’ is the percentile value in the specification of spherical coverage for that power class from clause 7.3.4.The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link angle).

The reference measurement channels and throughput criterion shall be as specified in clause 7.3A.2.0.3. The requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in clause 7.3.2.

Unless otherwise specified, the minimum requirements for reference sensitivity shall be verified with the network signalling value NS_200 (Table 6.2.3.3.1-1) configured.

The required spherical coverage EIS for each band in inter-band CA operation is given in clause 7.3.4 and modified by ΔRIB,S,n. The value of ∆RIB,S,n is defined in Table 7.3A.3.0.3-1.

Table 7.3A.3.0.3-1: ΔRIB,S,n EIS spherical coverage requirement relaxation for inter-band CA for power class 3

| NR CA band combination | NR band | ΔRIB,S,n (dB) |
| --- | --- | --- |
| CA_n257-n259 | n257 | 3.5 |
|  | n259 | 3.5 |
| CA_n258-n260 | n258 | 3.5 |
|  | n260 | 3.5 |
| CA_n258-n261 | n258 | 3.5 |
|  | n261 | 3.5 |
| CA_n260-n261 | n260 | 3.5 |
|  | n261 | 3.5 |
| Note: For each power class, band combinations without specified ΔRIB,S,n are not enabled for inter-band downlink carrier aggregation in this release. |  |  |

## <End of Change>