3GPP TSG-RAN5 Meeting #97	R5-227827

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2025 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex A for RedCap RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add RMC configurations for RedCap RRM tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RMC configurations for RedCap RRM tests are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-226179r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

A.3B	Reference SSB configuration for RedCap

A.3B.1	SSB configuration for FR1

Table A.3B.1-1: SSB allocation for FR1

| SSB Parameters |  | Unit | Value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 RedCap FR1 |  | SSB.2 RedCap FR1 | SSB.3 RedCap FR1 | SSB.4 RedCap FR1 |  | SSB.5 RedCap FR1 |  | SSB.6 RedCap FR1 | SSB.7 RedCap FR1 |
| Channel bandwidth |  | MHz | 20 |  | 20 | 20 | 10 |  | 20 |  | 10 | 20 |
| SSB SCS |  | kHz | 30 |  | 30 | 30 | 15 |  | 30 |  | 15 | 30 |
| SSB periodicity (TSSB) |  | ms | 20 |  | 20 | 20 | 20 |  | 20 |  | 80 | 80 |
| Number of SSBs per SS-burst |  |  | 1 |  | 2 | 1 | 1 |  | 1 |  | 1 | 1 |
| Symbol numbers containing SSB Note 4 | SSB #0 |  | 4-7 or 2-5 Note 3 |  | 4-7 or 2-5 Note 3 | 4-7 or 2-5 Note 3 | 2-5 |  | 4-7 or 2-5 Note 3 |  | 2-5 | 4-7 or 2-5 Note 3 |
|  | SSB #1 |  | N/A |  | 8-11 | N/A | N/A |  | N/A |  | N/A | N/A |
| Slot numbers containing SSB Note 4 |  |  | 0 |  |  |  |  |  |  |  |  |  |
| SFN containing SSB |  |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 1 |  | SFN mod (TSSB /10ms) = 0 |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  | (RBJ, RBJ+1,.…, RBJ+19)Note 2 |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. NOTE 4: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |  |

A.3B.2	SSB configuration for FR2

Table A.3B.2-1: SSB allocation for FR2

| SSB Parameters |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| SSB Pattern |  |  | SSB.1 RedCap FR2 | SSB.2 RedCap FR2 | SSB.3 RedCap FR2 |
| Channel bandwidth |  | MHz | 100 | 100 | 100 |
| SSB SCS |  | kHz | 120 | 120 | 120 |
| SSB periodicity (TSSB) |  | ms | 20 | 40 | 80 |
| Number of SSBs per SS-burst |  |  | 1 | 1 | 1 |
| Symbol numbers containing SSB Note 3 | SSB #0 |  | 4-7 | 4-7 | 4-7 |
|  | SSB #1 |  | N/A | N/A | N/A |
| Indices of slots containing SSB Note 3 |  |  | 0 |  |  |
| Indices of SFN containing SSB |  |  | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 | SFN mod (TSSB /10ms) = 0 |
| RB numbers containing SSB within channel BW |  |  | (RBJ,RBJ+1,.…,RBJ+19)Note 1 |  | (RBJ,RBJ+1,.…,RBJ+19)Note 2 |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: RBs containing SSB can be configured in any frequency location within the associated bandwidth part except the RBs for allowed synchronization raster defined in TS 38.104 [28]. NOTE 3: These values have been derived from other parameters for information purposes (as per TS 38.213). They are not settable parameters themselves |  |  |  |  |  |

A.4	Reference SMTC configuration

Table A.4-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 | SMTC.2 | SMTC.3 | SMTC.4 | SMTC.5 | SMTC.6 |
| SMTC periodicity | ms | 20 | 20 | 160 | 20 | 20 | 20 |
| SMTC offset | ms | 0 | 0 | 0 | 10 | 10 | 17 |
| SMTC duration | ms | 1 | 5 | 1 | 1 | 5 | 5 |

A.4A	Reference SMTC configuration for RedCap

Table A.4A-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 RedCap | SMTC.2 RedCap | SMTC.3 RedCap |  |  |  |
| SMTC periodicity | ms | 40 | 80 | 40 |  |  |  |
| SMTC offset | ms | 0 | 5 | 20 |  |  |  |
| SMTC duration | ms | 1 | 1 | 1 |  |  |  |

<End of modified section 1>

<Start of modified section 2>

A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  |  | ULBWP.1.3 |  |  |  | ULBWP.1.4 |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  | 0 |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 | 24 | 24 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |  |  |

A.8A	Reference BWP configurations for RedCap

This clause provides the typical BWP configurations used for RedCap RRM test cases defined in Annex A. RedCap downlink dedicated BWP configurations are specified in clause A.8A.1 and RedCap uplink dedicated BWP configurations are specified in clause A.8A.2. Other parameters not listed in this clause can be derived from the set up of each test.

A.8A.1	Downlink BWP configurations

Table A.8A.1-1: Downlink BWP patterns for RedCap dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 RedCap |  |  |  | DLBWP.1.2 RedCap |  |  |  | DLBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 51 | 32 | 16 | 25 | 51 | 32 | 16 | 25 | 51 | 32 | 16 |
| NOTE 1: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 3: RBc is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3A. |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.8A.2	Uplink BWP configurations

Table A.8A.2-1: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 RedCap |  |  |  | ULBWP.1.2 RedCap |  |  |  | ULBWP.1.3 RedCap |  |  |  |
| Starting PRB index |  | RBa Note 1 |  |  |  | RBb Note 2 |  |  |  | RBc Note 3 |  |  |  |
| SSB SCS | kHz | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | 25 | 51 | 32 | 16 | 25 | 51 | 32 | 16 | 25 | 51 | 32 | 16 |
| NOTE 1: RBa is the same as RBa for DLBWP.1.1 RedCap. NOTE 2: RBb is the same as RBb for DLBWP.1.2 RedCap. NOTE 3: RBc is the same as RBc for DLBWP.1.3 RedCap. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 2>