3GPP TSG-RAN5 Meeting #97-e	R5-227831

# Electronic Meeting, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2080 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of description of tables of test requirement for EN-DC FR2 eMIMO measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Tables of test requirement are needed to synchr with test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of description of tables of test requirement for eMIMO test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 members will not be aware of which test requirements are for which test steps |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.6.1, 5.6.6.2, 5.6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final outcome of R5-226818 and the result of 1 revision |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX

Many information are skipped

5.6.6.1.5	Test requirement

Table 5.6.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 5.6.6.1.5-1: CSI-RS specific test parameters for EN-DC FR2 CSI-based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |
| Beam assumptionNote 3 | 1~2 |  | Rough |  |
| Note1 | 1~2 | dBm/15kHz | -105 |  |
| Note1 | 1~2 | dBm/SSB SCS | -95.97 |  |
|  | 1~2 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1~2 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1~2 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1~2 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

After 480ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 5.6.6.1.5-2 and the corresponding relative accuracy requirements in Table 5.6.6.1.5-3.

Table 5.6.6.1.5-2: L1-SINR absolute accuracy requirements for the reported values R2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 51 |
| Highest reported value (CSI-RS #1) | 74 |

Table 5.6.6.1.5-3: L1-SINR relative accuracy requirements for the reported values R3

| Normal Conditions | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS #0) | 4 |
| Highest DIFF SINR reported (CSI-RS #0) | 13 |

The rate of correct events observed during repeated tests (R1 to R3) shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## << Unchanged sections omitted >>

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX

Many information are skipped

5.6.6.2.5	Test requirement

Table 5.6.6.2.5-1 and Table 5.6.6.2.5-1 define the primary level settings including test tolerances for all tests.

Table 5.6.6.2.5-1: SSB specific test parameters for FR2 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  |  | Rough |  |  |  |
| Note2 | 1~4 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -96 |  |  |  |
|  | 3,4 |  | -93 |  |  |  |
|  | 1~4 | dB | 0 | 0 | -Infinity | 9 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.5 | -94.5 | -Infinity | -87 |
|  | 3,4 |  | -91.5 | -91.5 | -Infinity | -84 |
| Io Note3 | 1,2 | dBm/95.04MHz | -63.2 | -63.2 | -67 | -57.5 |
|  | 3,4 |  | -63.2 | -63.2 | -67 | -57.5 |
|  | 1~4 | dB | 1.5 | 1.5 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table 5.6.6.2.5-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  |  | Rough |  |  |  |
| Note2 | 1~4 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1~4 | dBm/CSI-RS SCS | -96 |  |  |  |
|  | 1~4 | dB | 1.5 | 1.5 | -Infinity | 9 |
|  | 1~4 | dB | 1.5 | 1.5 | -Infinity | 9 |
| CSI-RS RSRP Note3 | 1~4 | dBm/ CSI-RS SCS | -94.5 | -94.5 | -Infinity | -87 |
| Io Note3 | 1~4 | dBm/95.04MHz | -63.2 | -63.2 | -67 | -57.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The UE shall send L1-SINR report every 640 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-SINR report including the results for both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.28.2, where X is

-	2880 for UE supporting power class 1

-	1920 for UE supporting power class 2, 3 or 4.

Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 5.6.6.2.5-3 for all test configurations and the corresponding relative accuracy requirements in Table 5.6.6.2.5-4 for all test configurations.

Table 5.6.6.2.5-3: L1-SINR absolute accuracy requirements for the reported values for all test configurations R1 and R3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 54 |
| Highest reported value (SSB#1) | - | 71 |

Table 5.6.6.2.5-4: L1-SINR relative accuracy requirements for the reported values for test configurations R1 and R4

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF SINR reported (SSB#0) | - | 4 |
| Highest DIFF SINR reported (SSB#0) | - | 10 |

For the test to pass, the ratio of successful reported valued for each requirement (R1 to R4) shall be at least 90% with a confidence level of 95%. Each requirement is evaluated independently the others.NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## << Unchanged sections omitted >>

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX

Many information are skipped

5.6.6.3.5	Test requirement

Table 5.6.6.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 5.6.6.3.5-1: CSI-RS specific test parameters for EN-DC FR2 CSI-RS based CMR and CSI-IM based IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.9 |  |
| Assumption for UE beamsNote 3 | 1~2 |  | Rough |  |
| Note1 | 1~2 | dBm/15kHz | -105 |  |
| Note1 | 1~2 | dBm/SSB SCS | -95.97 |  |
|  | 1~2 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1~2 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1~2 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1~2 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

After 480ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 5.6.6.3.5-2 and the corresponding relative accuracy requirements in Table 5.6.6.3.5-3. The reported L1-SINR value shall consider the Rx antenna gain in the range of [-10 ~ +20] dB when calculated.

Table 5.6.6.3.5-2: L1-SINR absolute accuracy requirements for the reported values R2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 53 |
| Highest reported value (CSI-RS #1) | 72 |

Table 5.6.6.3.5-3: L1-SINR relative accuracy requirements for the reported values R3

| Normal Conditions | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS #0) | 5 |
| Highest DIFF SINR reported (CSI-RS #0) | 12 |

The rate of correct events observed during repeated tests (R1 to R3) shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## << End of changes >>