3GPP TSG-RAN5 Meeting #97	R5-227891

Toulouse, France, 14th November – 18th November 2022

Title: 	Revision of determination of test frequencies for NR Intra-band Non-Contiguous CA

Source:	      Keysight Technologies UK Ltd

Agenda Item:	5.3.5.11

Document for:	Discussion and Endorsement

# 1.	Introduction

The procedure to determine test frequencies for NR Intra-band Non-Contiguous Carrier Aggregation are defined in TS38.508-1 annex C.2.4.3.2 [1].

However, some discussions are required to finalize such definition.

This document describes open issues in the procedure to determine test frequencies for NR Intra-band Non-Contiguous Carrier Aggregation and proposes a way forward to complete such definition.

# 2.	Discussion

# 2.1 FR1

According to TS38.508-1 annex C.2.4.3.2 [1], maxWgap is defined as:

| maxWgap = (maxFsBW – ∑k=1 to NSB SBCBWDL(k)) / (NSB-1) | C.2.4.3.3-Eq1 |
| --- | --- |

with maxFsBW for FR1 defined as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=100 MHz and <600 MHz | 600 MHz |
|  | >600 MHz | Full NR bandwidth |

On the other hand, intraBandFreqSeparationUL-AggBW-GapBW-r16 related capability is defined in TS 38.306 [5] as not mandatory and only for UL in the case of FR1:

Hence, if a device does not declare any intraBandFreqSeparationUL-AggBW-GapBW-r16 capability, it is assumed that it can support any intra-band non-contiguous allocation within the Full NR bandwidth.

For NR intra-band Non-Contiguous with UL CA, there are frequency bands with a full NR bandwidth lower than currently defined maxFsBW, e.g. CA_n41(2A) with max aggregated bandwidth of 190MHz()maxFsBW=200MHz) and Full NR bandwidth 194 MHz. In this case, test frequency values for one of the subblocks might fall outside the band even for low range.

In order to solve this issue, it is proposed to correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | min (Full NR bandwidth, 100 MHz) |
|  | >=100 MHz and <200 MHz | min (Full NR bandwidth, 200 MHz) |
|  | >=100 MHz and <600 MHz | min (Full NR bandwidth, 600 MHz) |
|  | >600 MHz | Full NR bandwidth |

Proposal 1: Correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | min (Full NR bandwidth, 100 MHz) |
|  | >=100 MHz and <200 MHz | min (Full NR bandwidth, 200 MHz) |
|  | >=100 MHz and <600 MHz | min (Full NR bandwidth, 600 MHz) |
|  | >600 MHz | Full NR bandwidth |

Proposal 2: Improve reading of step 2 of determination of test frequencies for a sub-block combination as shown below:

If the maxFsBW is smaller than the full bandwidth of the NR band then calculate test frequencies for both Low and High range. Else, only for the Low range.

Additionally, there is an ambiguity to define maxFsBW if carrier aggregation bandwidth is between 100 and 200 MHz.

In order to solve this issue, it is proposed to correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=200 MHz and <600 MHz | 600 MHz |
|  | >600 MHz | Full NR bandwidth |

Proposal 3: Correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=200 MHz and <600 MHz | 600 MHz |
|  | >600 MHz | Full NR bandwidth |

Finally, there is no frequency separation class higher than 600 MHz in FR1 nor intra-band Non-Contiguous CA configuration with more than 6 CCs. Hence, it is proposed to modify selection of maxFsBW as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=100 MHz and <600 MHz | 600 MHz |
|  | =600 MHz | Full NR bandwidth |

Proposal 4: Modify the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=100 MHz and <600 MHz | 600 MHz |
|  | =600 MHz | Full NR bandwidth |

# 2.2 FR2

Issues observed in FR1 are not applicable to FR2. Additionally, the related capabilities intraBandFreqSeparationUL and intraBandFreqSeparationDL are mandatory in FR2 case according to [5]:

# 2.3 Editor’s note: Low and High range vs Mid range

In annex C.2.4.3.2 in [4], there is an editor’s note indicating that

# Editor's note: The number of test points for intra-band non-contiguous CA configurations is under investigation, e.g. “Low” and “High”, or “Mid”.

With the criterion chosen to define maxWgap, there will be many combos of (frequency bands, aggregated bandwidth) where full NR bandwidth is considerable higher than maxFsBW, either for FR1 and FR2.

Examples:

| Band combination | Aggregated bandwidth [MHz] | maxFsBW [MHz] | Full NR bandwidth [MHz] |
| --- | --- | --- | --- |
| CA_n77(2A) | 20MHz+20MHz=40MHz | 100MHz | 900MHz |
| CA_n260(2A) | 100MHz+100 MHz=200MHz | 400MHz | 3000MHz |

In those cases, test coverage should include either Low and High range to ensure appropriate behaviour of the UE in both edges of the frequency band. Considering that, especially in FR1, if maxFsBW=Full NR bandwidth, only Low Range is tested, for simplicity, it is proposed to restrict existing editor’s note to FR2 and agree test frequencies for FR1 intra-band Non-Contiguous carrier aggregation as Low (if maxFsBW=Full NR bandwidth) or Low and High range(if maxFsBW<Full NR bandwidth) instead of Mid range.

Proposal 5: Restrict existing editor’s note in annex C.2.4.3.2 in TS 38.508-1 [4] to FR2, and agree test frequencies for FR1 intra-band Non-Contiguous carrier aggregation as Low (if maxFsBW=Full NR bandwidth) or Low and High range (if maxFsBW<Full NR bandwidth) instead of Mid range.

# 3. 	Conclusion

This document describes open issues in the procedure to determine test frequencies for NR Intra-band Non-Contiguous Carrier Aggregation and proposes a way forward to complete such definition.

The following proposals are made:

Proposal 1: Correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | min (Full NR bandwidth, 100 MHz) |
|  | >=100 MHz and <200 MHz | min (Full NR bandwidth, 200 MHz) |
|  | >=100 MHz and <600 MHz | min (Full NR bandwidth, 600 MHz) |
|  | >600 MHz | Full NR bandwidth |

Proposal 2: Improve reading of step 2 of determination of test frequencies for a sub-block combination as shown below:

If the maxFsBW is smaller than the full bandwidth of the NR band then calculate test frequencies for both Low and High range. Else, only for the Low range.

Proposal 3: Correct the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=200 MHz and <600 MHz | 600 MHz |
|  | >600 MHz | Full NR bandwidth |

Proposal 4: Modify the selection of maxFsBW for FR1 as:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | 100 MHz |
|  | >=100 MHz and <200 MHz | 200 MHz |
|  | >=100 MHz and <600 MHz | 600 MHz |
|  | =600 MHz | Full NR bandwidth |

Proposal 5: Restrict existing editor’s note in annex C.2.4.3.2 in TS 38.508-1 [4] to FR2, and agree test frequencies for FR1 intra-band Non-Contiguous carrier aggregation as Low (if maxFsBW=Full NR bandwidth) or Low and High range (if maxFsBW<Full NR bandwidth) instead of Mid range.

These proposals have been implemented in [6].

# 4.	References

- RP-200094, “Work Item Description for WI: UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1)”, RAN#87-e

- 3GPP TS 38.521-1 v17.6.1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

- RP-222459, “Status report for WI: UE Conformance Test Aspects for RF requirements for NR frequency range 1 (FR1); rapporteur: Huawei, RAN#97

- 3GPP TS 38.508-1-1 v17.6.1 User Equipment (UE) conformance specification; Part 1: Common test environment

- 3GPP TS 38.306 v17.2.0 User Equipment (UE) radio access capabilities

- R5-227892, “Updates to determination of test frequencies for NR Intra-band Non-Contiguous CA”, Keysight Technologies UK Ltd, RAN5#97