3GPP TSG-RAN5 Meeting #97	R5-227934

# Toulouse, France, 14th November 2022 – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2031 | rev | 1 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Limit transmit modulation quality for Tx Diversity to power per any antenna connector higher than a threshold |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A device could deactivate any of the transmitters used in Tx diversity mode if ouput power level is lower than maximum output power in order to save power consumption. When this happens, test system is not able to demodulate a Tx diversity signal in transmit modulation quality for Tx diversity test cases and test procedures get blocked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Limit transmit modulation quality for Tx diversity test cases to be executed on any antenna connector with the transmit power equal or higher than a threshold equal to Pmin and add test steps to execute carrier leakage and in-band emissions at maximum output power too |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transmit modulation quality test cacses for Tx diversity executions will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4G.2.2.4, 6.4G.2.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document depends on R5-227063. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-227067 and the outcome of 1 revision. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### |Δf| ≤ (0.1 PPM + 15 Hz)

### 6.4G.2	Transmit modulation quality for Tx Diversity

For UE supporting Tx diversity, the transmit modulation quality requirements are specified at each transmit antenna connector. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [6]), carrier leakage measurement requirement in clause 6.4G.2.2 and 6.4G.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

#### 6.4G.2.1	Error Vector Magnitude for Tx Diversity

FFS

#### 6.4G.2.2	Carrier leakage for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- Tests for Power Class 3 are FFS.

6.4G.2.2.1	Test purpose

Same test purpose as in 6.4.2.2.1.

6.4G.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 UE reporting Tx diversity release 15 and forward.

6.4G.2.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2.3-1 which is defined in clause 6.4.2.2.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.2.

6.4G.2.2.4	Test description

Same test description as in clause 6.4.2.2.4 with the following exceptions:

-	Adding the following test steps:

10. Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

11. Measure carrier leakage using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector. For TDD, only slots consisting of only UL symbols are under test.

- If the UE’s transmit power at both antenna connectors are higher than the minimum output power requirement in 6.3G.1.5, the carrier leakage is measured on each transmit antenna. Pass the UE if the carrier leakage on each antenna connector meets the requirements in 6.4G.2.2.5.

-	If UE’s transmit power is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the carrier leakage is measured only on this antenna connector. Pass the UE if the carrier leakage on this antenna connector meets the requirements in 6.4G.2.2.5.

6.4G.2.2.5	Test requirement

For each transmit antenna connector, each of the n carrier leakage results, derived in Annex E.3.1, shall not exceed the values in Table 6.4G.2.2.5-1. Allocated RBs are not under test. n is 10 for 15kHz SCS, 20 for 30kHz SCS and 30 for 60kHz SCS.

Table 6.4G.2.2.5-1: Test requirements for Relative Carrier Leakage Power

| Parameters UE output power | Relative limit (dBc) |
| --- | --- |
| 10 + MU to 10 + (MU + Uplink power control window size) dBm | -28 + TT |
| 0 + MU to 0 + (MU + Uplink power control window size) dBm | -25 + TT |
| -30 + MU to -30 + (MU + Uplink power control window size) dBm | -20 + TT |
| Pmin + MU to Pmin + (MU + Uplink power control window size) dBm | -10 + TT |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 2: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 3: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 4: MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW. NOTE 5: Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1. NOTE 6: Test tolerance TT = 0.8 dB. NOTE 7: Pmin is the minimum output power according to Table 6.3.1.3-1. |  |

#### 6.4G.2.3	In-band emissions for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- Tests for Power Class 3 are FFS.

6.4G.2.3.1	Test purpose

Same test purpose as in 6.4.2.3.1.

6.4G.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 UE reporting Tx diversity release 15 and forward.

6.4G.2.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the In-band Emission requirements specified in Table 6.4.2.3.3-1 which is defined in clause 6.4.2.3.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.3.

6.4G.2.3.4	Test description

Same test description as in clause 6.4.2.3.4 with the following exceptions:

-	Adding the following test steps:

Test procedure for PUSCH:

1.10     Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.11.    Measure In-band emission using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector. For TDD, only slots consisting of only UL symbols are under test.

Test procedure for PUCCH:

2.10.    Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.11     Measure In-band emission using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector.

- If the UE’s transmit power at both antenna connectors are higher than the minimum output power requirement in 6.3G.1.5, the carrier leakage is measured on each transmit antenna. Pass the UE if the carrier leakage on each antenna connector meets the requirements in 6.4G.2.3.5.

-	If UE’s transmit power is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the carrier leakage is measured only on this antenna connector. Pass the UE if the carrier leakage on this antenna connector meets the requirements in 6.4G.2.3.5.

6.4G.2.3.5	Test requirement

For each transmit antenna connector, the averaged In-band emission result, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.4G.2.3.5-1.

Table 6.4G.2.3.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General (NOTE 12) | dB | + TT |  | Any non-allocated (NOTE 2) |
| IQ Image (NOTE 12) | dB | -28 + TT | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 + TT | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage (NOTE 12) | dBc | -28 + TT | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 + TT | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 + TT | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 + TT | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Section 5.3). NOTE 7: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: is the limit specified in Table 6.4G.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: Test tolerance TT = 0.8 dB. NOTE 12: In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, IQ Image and Carrier leakage limit do not apply and General limit applies for all non-allocated frequencies. |  |  |  |  |

#### 6.4G.2.4	EVM equalizer spectrum flatness for Tx Diversity

Editor’s Note: The test case is not completed. The following aspects are either missing or not yet determined:

- Tests for Power Class 3 are FFS.

- Update to Annex E is FFS.

6.4G.2.4.1	Test purpose

Same test purpose as in 6.4.2.4.1.

6.4G.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 UE reporting Tx diversity release 15 and forward.

6.4G.2.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4.3-1 and Table 6.4.2.4.3-2 which are defined in clause 6.4.2.4.3. The composite EVM equalizer equalizer EC(f) is defined as

EC(f)=P1 ∙|EC1f|+P2 ∙|EC2(f)|P1+P2

where

ECn(f) represents equalizer coefficient for each antenna connector,  ，f is the allocated subcarriers within the transmission bandwidth ((|F|=12*);

P1 and P2 denote the linear power measured at each antenna connector respectively.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.4.

6.4G.2.4.4	Test description

Same test description as specified in clause 6.4.2.4.4 with following exceptions:

Step 3 of Test procedure as in 6.4.2.4.4.2 is replaced by:

3.	Measure spectrum flatness for each antenna connector using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

5.	Calculate the composite EVM using the values measured in step 3 and step 4 as in Annex TBD.

6.4G.2.4.5	Test requirement

Each of the n spectrum flatness functions, shall derive four ripple results in Annex E.4.4.1. The derived results shall not exceed the values in Figure 6.4G.2.4.5-1:

For normal conditions, the peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.4G.2.4.5-1. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 8.4 dB (see Figure 6.4G.2.4.5-1).

For extreme conditions, the EVM equalizer spectral flatness shall not exceed the values specified in Table 6.4G.2.4.5-2. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 7.4 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 11.4 dB (see Figure 6.4G.2.4.5-1).

Table 6.4G.2.4.5-1: Requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Table 6.4G.2.4.5-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 + TT (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 + TT (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 3: Test tolerance TT = 1.4 dB. |  |

Figure 6.4G.2.4.5-1: The test requirements for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated for unshaped modulations (the ETC test requirements are within brackets)

## <<< END OF CHANGES >>>