3GPP TSG-RAN5 Meeting #97	R5-227951

# Toulouse, France, 14 – 18 Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0809 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EIS spherical coverage for n259 power class 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing band n259 in EIS spherical coverage for power class 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add band n259 to EIS spherical coverage for power class 2 into Tables 7.3.4.3-2, 7.3.4.3-5 and 7.3.4.5-2. Remove editor’s note for band n259 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band n259 EIS spherical coverage for power class 2 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Reject removal of editor’s note for band n259 |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 7.3.4	EIS spherical coverage

Editor’s Note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- The test case is incomplete for band n259.

7.3.4.1	Test purpose

To verify that the EIS spherical coverage of the UE receiver is acceptable under conditions of low signal level, ideal propagation and no added noise.

7.3.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

7.3.4.3	Minimum conformance requirements

The reference sensitivity power level REFSENS at a single grid point of the spherical grid is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.3.

For power class 1, the maximum EIS at the 85th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-1 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -89.5 | -86.5 | -83.5 | -80.5 |
| n258 | -89.5 | -86.5 | -83.5 | -80.5 |
| n260 | -86.5 | -83.5 | -80.5 | -77.5 |
| n261 | -89.5 | -86.5 | -83.5 | -80.5 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

For power class 2, the maximum EIS at the 60th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-2 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-2: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -81 | -78 | -75 | -72 |
| n258 | -81 | -78 | -75 | -72 |
| n259 | -76.2 | -73.2 | -70.2 | -67.2 |
| n261 | -81 | -78 | -75 | -72 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

For power class 3, the maximum EIS at the 50th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-3 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

For power class 3, the UEs that support operation in multiple FR2 bands, the minimum requirement for EIS spherical coverage in Table 7.3.4.3-3 shall be increased per band, respectively, by the reference sensitivity relaxation parameter ∑MBS and ∆MBS,n as specified in Table 7.3.2.3.3-1a and 7.3.2.3.3-1b..

Table 7.3.4.3-3: EIS spherical coverage for power class 3

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 | -74.4 | -71.4 | -68.4 |
| n258 | -77.4 | -74.4 | -71.4 | -68.4 |
| n259 | -71.9 | -68.9 | -65.9 | -62.9 |
| n260 | -73.1 | -70.1 | -67.1 | -64.1 |
| n261 | -77.4 | -74.4 | -71.4 | -68.4 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

For power class 4, the maximum EIS at the 20th percentile of the CCDF of EIS measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 7.3.4.3-4 below. The requirement is verified with the test metric of EIS (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3.4.3-4: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.0 | -85.0 | -82.0 | -79.0 |
| n258 | -88.0 | -85.0 | -82.0 | -79.0 |
| n260 | -83.0 | -80.0 | -77.0 | -74.0 |
| n261 | -88.0 | -85.0 | -82.0 | -79.0 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

The REFSENS requirement shall be met for an uplink transmission using QPSK DFT-s-OFDM waveforms and for uplink transmission bandwidth less than or equal to that specified in Table 7.3.4.3-5.

Table 7.3.4.3-5: Uplink configuration for reference sensitivity

| NR Band / Channel bandwidth / NRB / SCS / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR Band | 50 MHz | 100 MHz | 200 MHz | 400 MHz | SCS | Duplex Mode |
| n257 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n258 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n259 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n260 | 32 | 64 | 128 | 256 | 120 kHz | TDD |
| n261 | 32 | 64 | 128 | 256 | 120 kHz | TDD |

Unless given by Table 7.3.4.3-6, the minimum requirements specified in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3 and Table 7.3.4.3-4 shall be verified with the network signalling value NS_200 configured.

Table 7.3.4.3-6: Network Signalling value for reference sensitivity

| NR Band | Network Signalling value |
| --- | --- |
| n258 | NS_201 |

For the UE which supports inter-band carrier aggregation, the minimum requirement for reference sensitivity in Table 7.3.4.3-1, Table 7.3.4.3-2, Table 7.3.4.3-3 and Table 7.3.4.3-4 shall be increased by the amount given in ΔRIB,P,n defined in subclause 7.3A.2.0.3 for the applicable operating bands.

The normative reference for this requirement is TS 38.101-2 [3] clause 7.3.4.

7.3.4.4	Test description

7.3.4.4.1	Initial conditions

Same initial conditions as in clause 7.3.2.4.1 except that only normal condition is tested.

7.3.4.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX.

4.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Rx beam selection to complete.

5.	Measure UE EIS value for each grid point according to EIS spherical coverage procedure defined in Annex K.1.6.0, and obtain a Complimentary Cumulative Distribution Function (CCDF) of all EIS dBm values. Alternatively, UE EIS measurement for each grid point could be done according to Rx Fast spherical coverage procedure defined in Annex K.1.6.1. After a rotation, allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for UE to find the best beam to use. EIS is calculated considering both polarizations, theta and phi.

6.	Identify the EIS dBm value corresponding to %-tile (UE power class dependent) value in the applicable test requirement table in section 7.3.4.5.

7.	Compare the EIS dBm value identified in step 6, to the limit value in the applicable test requirement table in section 7.3.4.5. If the EIS dBm value is lower or equal to the limit value, pass the UE. Otherwise fail the UE.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.2.

7.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.3.4.5	Test requirement

The reference measurement channels and throughput criterion shall be as specified in section 7.3.2.5.

Table 7.3.4.5-1: EIS spherical coverage for power class 1

| Operating band | EIS at 85th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| n258 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| n260 | -86.5 +TT | -83.5 +TT | -80.5 +TT | -77.5 +TT |
| n261 | -89.5 +TT | -86.5 +TT | -83.5 +TT | -80.5 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-2: EIS spherical coverage for power class 2

| Operating band | EIS at 60th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -81 +TT | -78 +TT | -75 +TT | -72 +TT |
| n258 | -81 +TT | -78 +TT | -75 +TT | -72+TT |
| n259 | -76.2 +TT | -73.2 +TT | -70.2 +TT | -67.2 +TT |
| n261 | -81 +TT | -78 +TT | -75 +TT | -72 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-3: EIS spherical coverage for power class 3 for single band UE or multi-band UE declaring MBs = 0 in all FR2 bands

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| n259 | -71.9 +TT | -68.9 +TT | -65.9 +TT | -62.9 +TT |
| n258 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| n260 | -73.1 +TT | -70.1 +TT | -67.1 +TT | -64.1 +TT |
| n261 | -77.4 +TT | -74.4 +TT | -71.4 +TT | -68.4 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

Table 7.3.4.5-3a: EIS spherical coverage for power class 3 for multi-band UE declaring MBs > 0 in any FR2 band (Rel-15)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| n258 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| n260 | -73.1 +TT+MBs | -70.1 +TT+MBs | -67.1 +TT+MBs | -64.1 +TT+MBs |
| n261 | -77.4 +TT+MBs | -74.4 +TT+MBs | -71.4 +TT+MBs | -68.4 +TT+MBs |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3b for details for MBs allowance corresponding to supported FR2 band set combination NOTE 4: For a Rel-15 UE supporting FR2 bands set not defined in Table 7.3.2.3.3-1a, Table 7.3.4.5-3c applies. |  |  |  |  |

Table 7.3.4.5-3b: EIS spherical coverage multiband relaxation factors for power class 3 (Rel-15)

| ID | Supported FR2 bands set | Maximum sum of MBs, ∑MBs (dB) (Note 3) | Comments |
| --- | --- | --- | --- |
| 1 | n257, n258 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 2 | n257, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 3 | n258, n260 | 0.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 4 | n258, n261 | 1.25 | Maximum 0.75 dB relaxation allowed for each band |
| 5 | n260, n261 | 0.75 | No relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 6 | n257, n258, n260 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 7 | n257, n258, n261 | 1.75 | Maximum 0.75 dB relaxation allowed for each band |
| 8 | n257, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 9 | n258, n260, n261 | 1.25 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| 10 | n257, n258, n260, n261 | 1.75 | Maximum 0.4 dB relaxation allowed for n260 and 0.75 dB relaxation allowed for all other bands |
| NOTE 1: MBs is the Multiband Relaxation factor declared by the UE for the tested band in Table A.4.3.9-3 of TS38.508-2 [11]. This declaration shall fulfil the requirements in Table 7.3.2.3.3-1a. NOTE 2: All UE supported bands needs to be tested to ensure the multiband relaxation declaration is compliant NOTE 3: Max allowed sum of MBs over all supported FR2 bands as defined in clause 7.3.2.3.3. |  |  |  |

Table 7.3.4.5-3c: EIS spherical coverage for power class 3 (Rel-16 and forward)

| Operating band | EIS at 50th%ile CCDF (dBm) / Channel bandwidth (NOTE 3) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| n258 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| n259 | -71.9 +TT+MBs,n | -68.9 +TT+MBs,n | -65.9 +TT+MBs,n | -62.9 +TT+MBs,n |
| n260 | -73.1 +TT+MBs,n | -70.1 +TT+MBs,n | -67.1 +TT+MBs,n | -64.1 +TT+MBs,n |
| n261 | -77.4 +TT+MBs,n | -74.4 +TT+MBs,n | -71.4 +TT+MBs,n | -68.4 +TT+MBs,n |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4. NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. NOTE 3: Refer Table 7.3.4.5-3d for details for MBs allowance corresponding to supported FR2 band set combination |  |  |  |  |

Table 7.3.4.5-3d: EIS spherical coverage multi-band relaxation factors for power class 3 (Rel-16 and forward)

| ID | FR2 bands/set | Comments |
| --- | --- | --- |
| 1 | n257 |  |
| 2 | n258 |  |
| 3 | n259 |  |
| 4 | n260 |  |
| 5 | n261 |  |
| 6 | n257, n261 | MBs,n relaxation is 0 dB |
| 7 | n260, n261 | MBs,n relaxation is 0 dB |
| NOTE 1: MBs,n is the Multiband Relaxation factor for the tested band. This shall fulfil the requirements in Table 7.3.2.3.3-1b. |  |  |

Table 7.3.4.5-3e: Test Tolerance (Reference sensitivity for power class 3)

| Test Metric | f ≤ 40.8 GHz |
| --- | --- |
| IFF (Max device size ≤ 30 cm) | 2.21 dB |

Table 7.3.4.5-4: EIS spherical coverage for power class 4

| Operating band | EIS at 20th%ile CCDF (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| n258 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| n260 | -83.0 +TT | -80.0 +TT | -77.0 +TT | -74.0 +TT |
| n261 | -88.0 +TT | -85.0 +TT | -82.0 +TT | -79.0 +TT |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 2: The EIS spherical coverage requirements are verified only under normal thermal conditions as defined in TS 38.508-1 [10] subclause 4.1.1. |  |  |  |  |

----- End of changes --------