3GPP TSG-RAN5 Meeting #97	R5-227963
Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0820 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC1 - SEM test case update in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MU and TT analysis has progressed for PC1 devices. Test case needs to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 6.5.2.1: -Editor’s note update. -TT tables definition/update. -Anex F update: MU and TT definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomple. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1, F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -Corrected current version from 17.0.0 to 17.0.1. -Removed changes for FR2b. -Updated values for FR2a. -Reverted TT formula and updated values accordingly. -Added Anritsu as co-source company. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.2.1	Spectrum Emission Mask

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1 FR2b, 2, and 4.

6.5.2.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5.2.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward.

6.5.2.1.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than FOOB as specified in Table 6.5.2.1.3-1 the spurious requirements in clause 6.5.3 are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.1.3-1 for the specified channel bandwidth. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

Table 6.5.2.1.3-1: General NR spectrum emission mask for Range 2.

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 | -5 | -5 | -5 | 1 MHz |
| 5-10 | -13 | -5 | -5 | -5 | 1 MHz |
| 10-20 | -13 | -13 | -5 | -5 | 1 MHz |
| 20-40 | -13 | -13 | -13 | -5 | 1 MHz |
| 40-100 | -13 | -13 | -13 | -13 | 1 MHz |
| 100-200 |  | -13 | -13 | -13 | 1 MHz |
| 200-400 |  |  | -13 | -13 | 1 MHz |
| 400-800 |  |  |  | -13 | 1 MHz |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.5.2.1.

6.5.2.1.4	Test description

6.5.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | - | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 2 |  | DFT-s-OFDM QPSK | Outer_Full |
| 3 |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 4 |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 5 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: All test points in this table must also exist in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-3 (MPR) for PC1 or Table 6.2.2.4.1-7, Table 6.2.2.4.1-8, Table 6.2.2.4.1-9 (MPR) for PC2, PC3 and PC4. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and clause A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5.2.1.4.3

6.5.2.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.1.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

3.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 2) for the UE Tx beam selection to complete.

4.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

5.	Measure the TRP of the transmitted signal with a measurement filter of bandwidths according to Table 6.5.2.1.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. TRP shall be recorded for each step. The measurement period shall capture the active time slots. Total radiated power is measured according to TRP measurement procedure defined in Annex K. The measurement grid used for TRP measurement defined in Annex M. TRP is calculated considering both polarizations, theta and phi.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in Table 6.5.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE 2:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

6.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

6.5.2.1.5	Test requirement

The measured TRP of any UE emission derived in step 5, shall fulfil requirements in Table.6.5.2.1.5-1.

Table 6.5.2.1.5-1: General NR spectrum emission mask for Range 2

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 5-10 | -13 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 10-20 | -13 + TT | -13 + TT | -5 + TT | -5 + TT | 1 MHz |
| 20-40 | -13 + TT | -13 + TT | -13 + TT | -5 + TT | 1 MHz |
| 40-100 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 100-200 |  | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 200-400 |  |  | -13 + TT | -13 +TT | 1 MHz |
| 400-800 |  |  |  | -13 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.1.5-1a NOTE 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. NOTE 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. |  |  |  |  |  |

Table 6.5.2.1.5-1a: Test Tolerance (Spectrum emission mask) for PC3

| Test Metric | 23.45GHz ≤ f ≤ 32.125GHz | 32.125GHz < f ≤ 40.8GHz |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 3.21 dB | 3.46 dB |

Table 6.5.2.1.5-1b: Test Tolerance (Spectrum emission mask) for PC1

| Test Metric | 23.45GHz ≤ f ≤ 32.125GHz | 32.125GHz < f ≤ 40.8GHz |
| --- | --- | --- |
| IFF (Max device size ≤ 30 cm) | 4.11 dB | FFS |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

## <<< Skip unchanged sections >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±4.89 dB (FR2a, NTC testing) ±5.09 dB (FR2b, NTC testing) ±5.17 dB (FR2a, ETC testing) ±5.37 dB (FR2b, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±[5.33] dB (FR2a, NTC testing) ±TBD dB (FR2b, NTC testing) ±TBD dB (FR2a, ETC testing) ±TBD dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.42 dB (FR2a, NTC testing) ±4.62 dB (FR2b, NTC testing) ±4.70 dB (FR2a, ETC testing) ±4.90 dB (FR2b, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±[4.64] dB (FR2a, NTC testing) ± TBD dB (FR2b, NTC testing) ± TBD dB (FR2a, ETC testing) ± TBD dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.60 dB (FR2a) ±5.20 dB (FR2b) PC1 Max Device size ≤ 30 cm TBD | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±4.92 dB (FR2a), NTC testing ±5.10 dB (FR2b), NTC testing ±5.20 dB (FR2a, ETC testing) ±5.38 dB (FR2b, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO | Same as 6.2.3 |  |
| 6.3.1 Minimum output power | PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm FFS (FR2a) FFS dB (FR2b) PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing ±6.41 dB (FR2a & FR2b, ETC testing)) | MTSU = 1.00 x MU (from Table B.7-1 in TR 38.903) |
| 6.3.2 Transmit OFF power | PC3: Max Device size ≤ 30 cm ±5.67 dB (FR2a) PC1: Max Device size ≤ 30 cm ±[5.67] dB (FR2a) | MTSU = 1.00 x MU (from Table B.8-1 in TR 38.903) |
| 6.3.3.2 General ON/OFF time mask | ON power: Same as 6.2.1.1 (EIRP) for the respective power class OFF power: Same as 6.3.1 for the respective power class |  |
| 6.3.3.4 PRACH time mask | PC3: PRACH power: TBD OFF power: Max Device size ≤ 30 cm ±6.15 dB (FR2a & FR2b, NTC testing) ±6.41 dB (FR2a & FR2b, ETC testing) |  |
| 6.3.3.6 SRS time mask | TBD |  |
| 6.3.4.2 Absolute power tolerance | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) | MTSU = SQRT (UL Meas Uncer2 + DL Meas Uncer2) UL Meas Uncer: Same as 6.3.1 DL Meas Uncer: Same as 7.3.2 |
| 6.3.4.3 Relative power tolerance | PC3 Max Device size ≤ 30 cm [±1.7 dB] (FR2a) [±1.7 dB] (FR2b) | MTSU = 1.00 x MU (from Table B.9a.2.2-2 in TR 38.903) |
| 6.3.4.4 Aggregate power tolerance | PC3 Max Device size ≤ 30 cm ±1.4 dB (FR2a) ±1.4 dB (FR2b) | MTSU = 1.00 x MU (from Table B.9a.3.2-2 in TR 38.903) |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 for each CC For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.3D.3.4 SRS time mask for UL MIMO | PC3: OFF Power Max Device size ≤ 30cm ± 6.15 dB (FR2a) ± 6.15 dB (FR2b) ON Power Quiet Zone size ≤ 30cm TBD (FR2a) TBD (FR2b) | OFF Power MTSU = 1.00 x MU (from Table B.8-2-4 in TR 38.903) ON Power TBD |
| 6.4.1 Frequency error | ± 0.01 ppm (NTC & ETC testing) | MTSU = 1.00 x MU (from B.10.1 and B.10.2 in TR 38.903) |
| 6.4.2.1 Error vector magnitude | PUSCH, PC3, FR2a: As defined in Table F.1.2-2. PUSCH, PC3, FR2b: As defined in Table F.1.2-3. Otherwise: TBD |  |
| 6.4.2.2 Carrier leakage | PC3 Max Device size ≤ 30 cm ±5.44 dB (FR2a) ±5.57 dB (FR2b) uplink absolute power measurement uncertainty: 6.15 dB (FR2a & FR2b, NTC testing) uplink relative power measurement uncertainty: 1.4 dB (FR2a & FR2b, NTC testing) | MTSU = 1.00 x MU (from Table B.11-1 in TR 38.903) |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) | TBD |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) | TBD |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) | TBD |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) | TBD |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) | TBD |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) | TBD |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) | TBD |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | TBD |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) | TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) | TBD |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) | TBD |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) | TBD |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) | TBD |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | TBD |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) | TBD |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) | TBD |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) | TBD |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) | TBD |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) | TBD |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) | TBD |  |
| 6.5.1 Occupied bandwidth | Max Device size ≤ 30cm PC3: FR2a: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.2 [%CBW] (BW 200MHz) ±1.2 [%CBW] (BW 400MHz) FR2b: ±0.4 [%CBW] (BW 50MHz) ±0.4 [%CBW] (BW 100MHz) ±1.3 [%CBW] (BW 200MHz) ±1.3 [%CBW] (BW 400MHz) FR2c: TBD |  |
| 6.5.2.1 Spectrum Emission Mask | PC3 Max Device size ≤ 30 cm ±4.94 dB (FR2a) ±5.32 dB (FR2b) PC1 Max Device size ≤ 30 cm ±6.32 dB (FR2a) ±FFS (FR2b) | MTSU = 1.00 x MU (from Table B.16-1 in TR 38.903) |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Max Device size ≤ 30cm FR2a, NTC & ETC testing: ±5.63 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) FR2b, NTC & ETC testing: ±6.09 dB (BW ≤ 50MHz) ±6.09 dB (50MHz < BW ≤ 100MHz) ±6.09 dB (100MHz < BW ≤ 200MHz) ±6.09 dB (200MHz < BW ≤ 400MHz) | MTSU = 1.00 x MU (from Table B.17-1B in TR 38.903) |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.87 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 3.04 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) [3.12] dB (FR2a, NTC) TBD dB (FR2b, NTC) TBD dB (FR2a, ETC) TBD dB (FR2b, ETC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – [0.13]) (FR2a) TT = TBD (FR2b) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.65 dB (FR2a, NTC) 2.77 dB (FR2b, NTC) 2.82 dB (FR2a, ETC) 2.94 dB (FR2b, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) [2.78] dB (FR2a, NTC) TBD dB (FR2b, NTC) TBD dB (FR2a, ETC) TBD dB (FR2b, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.58 dB (FR2a) 2.58 dB (FR2b) PC4 TBD | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) PC1 TBD |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.11 dB (FR2a, NTC) 3.11 dB (FR2b, NTC) 3.30 dB (FR2a, ETC) 3.30 dB (FR2b, ETC) | Minimum peak EIRP TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.2 UE maximum output power - EIRP and TRP for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.3 UE maximum output power - EIRP and TRP for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |
| 6.2A.1.1.4 UE maximum output power - EIRP and TRP for CA (5UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.5 UE maximum output power - EIRP and TRP for CA (6UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.6 UE maximum output power - EIRP and TRP for CA (7UL CA) | Intra-band contiguous CA, Intra-band non-contiguous CA TBD |  |
| 6.2A.1.1.7 UE maximum output power - EIRP and TRP for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.1 Spherical coverage for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.2 Spherical coverage for CA (3UL CA) | Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.3 Spherical coverage for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.1.2.4 Spherical coverage for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.5 Spherical coverage for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.6 Spherical coverage for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.1.2.7 Spherical coverage for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.1 UE maximum output power reduction for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.2 UE maximum output power reduction for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.3 UE maximum output power reduction for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.2 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.2A.2.4 UE maximum output power reduction for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.5 UE maximum output power reduction for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.6 UE maximum output power reduction for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.2A.2.7 UE maximum output power reduction for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.2D.2 UE maximum output power reduction for UL MIMO | Same as 6.2.2 |  |
| 6.2D.3 UE maximum output power with additional requirements for UL MIMO | Same as 6.2.3 |  |
| 6.3.1 Minimum output power | PC3 Minimum EIRP IFF (Max Device size ≤ 30 cm) NTC 4.21 dB (FR2a 50 MHz) 2.52 dB (FR2a 100 MHz) 0.66 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.17 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) ETC 4.37 dB (FR2a 50 MHz) 2.68 dB (FR2a 100 MHz) 0.82 dB (FR2a 200 MHz) 0 dB (FR2a 400 MHz) 1.33 dB (FR2b 50 MHz) 0 dB (FR2b 100 MHz) 0 dB (FR2b 200 MHz) 0 dB (FR2b 400 MHz) | Minimum EIRP TT = max(R, ΔSNRmr + 0.65 x (MTSUIFF – 1.0)) -R R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.3.1.5) ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level (-13 dBm) FR2a 50 MHz: ΔSNRmr = 0.86 dB FR2a 100 MHz: ΔSNRmr = 1.57 dB FR2a 200 MHz: ΔSNRmr = 2.71 dB FR2a 400 MHz: ΔSNRmr = 4.35 dB FR2b 50 MHz: ΔSNRmr = 2.32 dB FR2b 100 MHz: ΔSNRmr = 3.82 dB FR2b 200 MHz: ΔSNRmr = 5.82 dB FR2b 400 MHz: ΔSNRmr = 8.21 dB |
| 6.3.2 Transmit OFF power | 0 dB |  |
| 6.3.3.2 General ON/OFF time mask | PC3: ON Power Same as 6.2.1.1 (EIRP) OFF Power 0 dB | ON Power: Same as 6.2.1.1 (EIRP) OFF Power: Same as 6.3.1 |
| 6.3.3.4 PRACH time mask | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b)TBD | ON Power TBD |
| 6.3.4.2 Absolute power tolerance | PC3 Max Device size ≤ 30 cm ±8.05 dB (FR2a & FR2b, NTC testing) ±8.42 dB (FR2a & FR2b, ETC testing) | TT = MTSU |
| 6.3.4.3 Relative power tolerance | PC3 IFF (Max Device size ≤ 30 cm) [0.46 dB] (FR2a) [0.46 dB] (FR2b) | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |
| 6.3.4.4 Aggregate power tolerance | PC3 IFF (Max Device size ≤ 30 cm) 0.26 dB (FR2a) 0.26 dB (FR2b) | PC3 TT = 0.65 x (MTSUIFF – 1.0) (FR2a) TT = 0.65 x (MTSUIFF – 1.0) (FR2b) (assuming a power step ΔP = 1 dB) |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.2 Minimum output power for CA (3UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.3 Minimum output power for CA (4UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.4 Minimum output power for CA (5UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.5 Minimum output power for CA (6UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.6 Minimum output power for CA (7UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.1.7 Minimum output power for CA (8UL CA) | For UL CA aggregated BW ≤ 800 MHz: Same as 6.3.1 For UL CA aggregated BW > 800 MHz: TBD |  |
| 6.3A.3.1.1 General ON/OFF time mask for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.2 General ON/OFF time mask for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.3 General ON/OFF time mask for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.3 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.3.1.4 General ON/OFF time mask for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.5 General ON/OFF time mask for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.6 General ON/OFF time mask for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.3.1.7 General ON/OFF time mask for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.3D.3.1 General ON/OFF time mask for UL MIMO | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) | ON Power TBD |
| 6.3D.3.4 SRS time mask for UL MIMO | PC3: OFF Power Max Device size ≤ 30cm 0 dB ON Power Max Device size ≤ 30cm TBD (FR2a) TBD (FR2b) | ON Power TBD |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.2 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.1 Relative power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.2 Relative power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.3 Relative power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz TBD Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.3.4 Relative power tolerance for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.5 Relative power tolerance for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.6 Relative power tolerance for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.3.7 Relative power tolerance for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.1 Aggregate power tolerance for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.2 Aggregate power tolerance for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.3 Aggregate power tolerance for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.3.4.4 for each CC. Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.3A.4.4.4 Aggregate power tolerance for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.5 Aggregate power tolerance for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.6 Aggregate power tolerance for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.3A.4.4.7 Aggregate power tolerance for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.4.1 Frequency error | 0.005 ppm (NTC & ETC testing) | TT = 0.5 x MTSU |
| 6.4.2.1 Error vector magnitude | PUSCH, PC3, FR2a: As defined in Table 6.4.2.1.5-2. PUSCH, PC3, FR2b: As defined in Table 6.4.2.1.5-3. | Minimum requirement + TT EVM_meas_Increase = sqrt(Minimum requirement^2 + MTSU^2) - Minimum requirement; it is the increase of measured EVM due to test equipment uncertainty. EVM_meas_Increase_Relative = EVM_meas_Increase / Minimum requirement [%] If (EVM_meas_Increase_Relative < 7.5%) TT = 0% Else if (7.5% ≤ EVM_meas_Increase_Relative ≤ 50%) TT = EVM_meas_Increase Else Skip the test as not testable. |
| 6.4.2.2 Carrier leakage | IFF (Max Device size ≤ 30 cm) FR2a: ±3.54 dB (BW ≤ 400MHz) FR2b: ±3.62 dB (BW ≤ 400MHz) | TT = 0.65 x MTSUIFF |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.4A.1.1 Frequency error for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.2 Frequency error for CA (3UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.3 Frequency error for CA (4UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.4.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |
| 6.4A.1.4 Frequency error for CA (5UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.5 Frequency error for CA (6UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.6 Frequency error for CA (7UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.1.7 Frequency error for CA (8UL CA) | Intra-band contiguous CA TBD |  |
| 6.4A.2.1.1 Error Vector magnitude for CA (2UL CA) | TBD |  |
| 6.4A.2.1.2 Error Vector magnitude for CA (3UL CA) | TBD |  |
| 6.4A.2.1.3 Error Vector magnitude for CA (4UL CA) | TBD |  |
| 6.4A.2.1.4 Error Vector magnitude for CA (5UL CA) | TBD |  |
| 6.4A.2.1.5 Error Vector magnitude for CA (6UL CA) | TBD |  |
| 6.4A.2.1.6 Error Vector magnitude for CA (7UL CA) | TBD |  |
| 6.4A.2.1.7 Error Vector magnitude for CA (8UL CA) | TBD |  |
| 6.4A.2.2.1 Carrier leakage for CA (2UL CA) | TBD |  |
| 6.4A.2.2.2 Carrier leakage for CA (3UL CA) | TBD |  |
| 6.4A.2.2.3 Carrier leakage for CA (4UL CA) | TBD |  |
| 6.4A.2.2.4 Carrier leakage for CA (5UL CA) | TBD |  |
| 6.4A.2.2.5 Carrier leakage for CA (6UL CA) | TBD |  |
| 6.4A.2.2.6 Carrier leakage for CA (7UL CA) | TBD |  |
| 6.4A.2.2.7 Carrier leakage for CA (8UL CA) | TBD |  |
| 6.4A.2.3.1 In-band emissions for CA (2UL CA) | TBD |  |
| 6.4A.2.3.2 In-band emissions for CA (3UL CA) | TBD |  |
| 6.4A.2.3.3 In-band emissions for CA (4UL CA) | TBD |  |
| 6.4A.2.3.4 In-band emissions for CA (5UL CA) | TBD |  |
| 6.4A.2.3.5 In-band emissions for CA (6UL CA) | TBD |  |
| 6.4A.2.3.6 In-band emissions for CA (7UL CA) | TBD |  |
| 6.4A.2.3.7 In-band emissions for CA (8UL CA) | TBD |  |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |
| 6.5.2.1 Spectrum Emission Mask | PC3 IFF (Max Device size ≤ 30 cm) 3.21 dB (FR2a) 3.46 dB (FR2b) PC1 IFF (Max Device size ≤ 30 cm) 4.11 dB (FR2a) FFS dB (FR2b) | TT = 0.65 x MTSUIFF |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Absolute requirement 0 dB Relative requirement IFF (Max Device size ≤ 30 cm) FR2a: ±4.66 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) FR2b: ±4.96 dB (BW ≤ 50MHz) ±4.96 dB (50MHz < BW ≤ 100MHz) ±4.96 dB (100MHz < BW ≤ 200MHz) ±4.96 dB (200MHz < BW ≤ 400MHz) | TT = max(R, ΔSNRmr+0.65 x (MTSUIFF-1.0)) -R + TT due to metric change TT due to metric change : 1.0 dB R: Relaxation needed to limit influence of TE noise to 1 dB (specified in clause 6.5.2.3.5) ΔSNRmr: Systematic offset due to noise when measuring ACP at minimum requirement level |

## <<< END OF CHANGES >>>