3GPP TSG-RAN5 Meeting #97 	R5-227986

# Toulouse,  France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0843 | rev | 1 | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of TRP grids for spurious emissions for PC1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TRP grids for spurious emissions are not defined for PC1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tables M.4.5-1a, M.4.5-2a, M.4.5-3a were added to define antenna array assumption and TRP measurement grid requirement for PC1, and descriptions were corrected accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions cannot be tested for PC1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | M.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on the outcome of MU discussion R5-227983 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-227077 and the outcome of 2 revisions to this document. r1: - Corrected current version 17.0.0 -> 17.0.1 - Values of Systematic error due to TRP calculation/quadrature for PC1 were corrected. r2: Table M.4.5-3a was updated based on R5-226244. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

M.4.5	TRP Grids for Spurious Emissions

The worst antenna array assumptions for the MU simulations are outlined in Tables M.4.5-1 and M.4.5-2 for PC3, and Tables M.4.5-1a and M.4.5-2a for PC1.

Table M.4.5-1: Single Antenna Element Radiation Pattern for spurious emission measurements for PC3

| Antenna element horizontal radiation pattern | , Am =30 dB |
| --- | --- |
| Horizontal half-power beam width of single element | 260o |
| Antenna element vertical radiation pattern | , SLAv =30 dB |
| Vertical half-power beam width of single array element | 130o |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 1.5 dBi |

Table M.4.5-1a: Single Antenna Element Radiation Pattern for spurious emission measurements for PC1

| Antenna element horizontal radiation pattern | , Am = 25 dB |
| --- | --- |
| Horizontal half-power beam width of single element | 90o |
| Antenna element vertical radiation pattern | , SLAv = 25 dB |
| Vertical half-power beam width of single array element | 90o |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 5 dBi |

Table M.4.5-2: Composite Antenna Array Radiation Pattern for spurious emission measurements for PC3

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 8 x 2 |
| Horizontal radiating element spacing, dh/λ | 1 |
| Vertical radiating element spacing, dv/λ | 1 |

Table M.4.5-2a: Composite Antenna Array Radiation Pattern for spurious emission measurements for PC1

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | 12 x 12 |
| Horizontal radiating element spacing, dh/λ | 1 |
| Vertical radiating element spacing, dv/λ | 1 |

The TRP measurement grid selection for spurious emissions is up to test system implementation but shall meet the criteria shown in Table M.4.5-3 for PC3 and Table M.4.5-3a for PC1.

Table M.4.5-3: TRP measurement grid requirement for spurious emission measurements for PC3

| Level of Grid | Grid Type | Standard Deviation of MU Element ‘Influence of TRP Measurement’ | Systematic error due to TRP calculation/quadrature | Number of unique grid points |
| --- | --- | --- | --- | --- |
| Coarse | Constant Density | N/A | N/A | 35 |
|  | Constant-Step Size | N/A | N/A | 62 (==30o) |
| Fine | Constant Density | 0.32dB | 0dB | 135 |
|  | Constant-Step Size | 0.31dB | 0dB | ==15o) |

Table M.4.5-3a: TRP measurement grid requirement for spurious emission measurements for PC1

| Level of Grid | Grid Type | Standard Deviation of MU Element ‘Influence of TRP Measurement’ | Systematic error due to TRP calculation/quadrature | Number of unique grid points |
| --- | --- | --- | --- | --- |
| Coarse | Constant Density | N/A | N/A | TBD |
|  | Constant-Step Size | N/A | N/A | TBD |
| Fine | Constant Density | 0.25dB | 0dB | 1600 |
|  | Constant-Step Size | 0.25dB | 0dB | 2522==5o) |

For spurious emissions, TRP measurements with measurement antennas displaced up to 10o from the focal point (based on electrical switching) in an IFF (based on CATR) test system, alternate TRP approaches for constant-step size grids are allowed for the coarse and fine grids:

interpolation to the non-offset system coordinate system that allows the use of Clenshaw-Curtis or classical sin() quadratures

use of the advanced Jacobian matrix quadrature approach that uses triangulations of the sphere

<<End of change>>