3GPP TSG-RAN5 Meeting #97	R5-228005

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0390 | rev | 1 | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete TC 14.2.1 with TT analysis results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TT analysis for 14.2.1 is in draft status. The test case has an editor’s note stating that it needs alignment with core spec The calculation of the response time is incorrect.The details of the response time calculation are included in the TT analysis |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated with the final results of the TT analysis The alignment with core spec has been checked, no updates required, the editor’s note has been removed Corrected the calculation of the response time |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on R4-2218084 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: alignment to R4-2218084 |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 14.2.1	NR RSTD measurement period test case for single positioning frequency layer in FR1 SA

#### 14.2.1.1	Test purpose

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in TS 38.133 [50] Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

#### 14.2.1.2	Test applicability

This test applies to all types of NR UE release 16 onwards that supports DL-TDOA positioning.

#### 14.2.1.3	Minimum conformance requirements

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [49], the UE shall be able to measure and report multiple (up to the UE capability specified in TS 38.133 [50] Clause 9.9.2.3) DL RSTD measurements, during the measurement period TRSTD,Total defined as:

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

Where ,

i is the index of positioning frequency layer,

L is total number of positioning frequency layers, and

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast ,

where:

NRxBeam,i is the UE Rx beam sweeping factor. In FR1, NRxBeam,i = 1; and in FR2,  NRxBeam,i = 8.

CSSFPRS,i is the carrier-specific scaling factor for NR PRS-based positioning measurements in positioning frequency layer i as defined in TS 38.133[50] clause 9.1.5.2.

NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [56].

Nsample is the number of PRS RSTD samples and Nsample= 4.

Tlast is the measurement duration for the last PRS RSTD sample, including the sampling time and processing time, Tlast = Ti + Tavailable_PRS,i ,

Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where,

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [49],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi.

MGRPi is the repetition periodicity of the measurement gap applicable for measurement in the PRS frequency layer i.

TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive the measurement period of that positioning frequency layer i. Where,

TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

Nmuting is the scaling factor considering PRS resource muting. If bitmap b1  for higher-layer parameter DL-PRS-MutingPattern is provided, and TperPRS*TmutingPRS ≤10240ms, then Nmuting=TmutingPRS*min(L,10240TperPRS*TmutingPRS); otherwise, if bitmap b1 is not provided or TperPRS*TmutingPRS>10240ms, then Nmuting=1.

TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and L is the size of the bitmap b1.

- Note: For the purpose of calculating TPRS,i, only the PRS resources fully or partially covered by the MG are considered.

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [49] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [49] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [49].

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot specified in TS 37.355 [49].

The time TRSTD,i starts from the first MG instance aligned with a DL PRS resource(s) of positioning frequency layer i closest in time after both the NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message are delivered from LMF to the physical layer of UE via LPP.

#### 14.2.1.4	Test description

##### 14.2.1.4.1	Initial conditions

The test is defined with three possible Test Configurations. In the case that the UE supports more than one of these Test Configurations, then the UE is only required to be tested in one of the Test Configurations, chosen by the UE. The defined Test Configurations are specified in Table 14.2.1.4.1-1.

Table 14.2.1.4.1-1: Test Configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Test Environment: Normal, as defined in TS 38.508-1 [45] clause 4.1.

Frequencies to be tested: Mid, as defined in TS 38.508-1 [45] clause 4.3.1.

Channel bandwidth to be tested: are specified in Table 14.2.1.4-1.

1.	Connect the SS and AWGN noise sources to the UE antenna connector or antenna connectors as shown in Annex A, Figure A.12.

2.	The general test parameter settings are set up according to Table 14.2.1.5-1, Table 14.2.1.5-2 and Table 14.2.1.5-3.

3.	Propagation conditions are set according to clause 4.15.2.

4.	Message contents are defined in clause 14.2.4.1.3.

5.	In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the RSTD reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1

##### 14.2.1.4.2	Test procedure

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 is active in T1 and T2, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell.

The NR-DL-TDOA-ProvideAssistanceData as defined in TS 37.355 [49] clause 6.5.10.1, shall be provided to the UE during T1. The last TTI containing the NR-DL-TDOA-ProvideAssistanceData shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

- Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, according to TS 38.508-1 [45] clause 4.5.

2.	The SS shall send a RESET UE POSITIONING STORED INFORMATION message.

3.	Set the parameters according to Table 14.2.1.5-1, Table 14.2.1.5-2 and Table 14.2.1.5-3. Propagation conditions are set according to clause 4.15.2.

4.	T1 starts.

5.	The SS shall transmit an LPP REQUEST CAPABILITIES message.

6.	The UE shall transmit an LPP PROVIDE CAPABILITIES message indicating the DL-TDOA capabilities supported by the UE in the NR-DL-TDOA-ProvideCapabilities IE.

7.	The SS shall send a LPP PROVIDE ASSISTANCE DATA message, including the NR-DL-TDOA-ProvideAssistanceData IE. The position of neighbour Cell 2 and the position of neighbour Cell 3 are described in 3GPP TS 37.571-5 [20]. If the UE message at step 6 includes the ackRequested IE set to TRUE, then the SS shall send an acknowledgment in the LPP PROVIDE ASSISTANCE DATA message.

8.	The SS shall send a LPP REQUEST LOCATION INFORMATION message, including the NR-DL-TDOA-RequestLocationInformation IE such that the UE receives the message T ms before the start of T2, where T = 50 ms.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.2.1.5-2 and Table 14.2.1.5-3.

10.	The UE shall transmit a LPP PROVIDE LOCATION INFORMATION message including the NR-DL-TDOA-ProvideLocationInformation IE within the response time (see clause 4.15.3). The UE shall perform and report the RSTD measurements for both Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1. If the UE transmits an NR-DL-TDOA-ProvideLocationInformation IE including the nr-RSTD field for both Cell 2 and Cell 3 within the response time then the number of successful tests is increased by one. If the UE fails to report the NR-DL-TDOA-ProvideLocationInformation IE with both the nr-RSTD fields included within the response time then the number of failure tests is increased by one.

11.	If the UE message at step 10 includes the ackRequested IE set to TRUE, the SS shall send a LPP acknowledgement message.

12.	Repeat steps 2-11 in Tables 14.2.1.4-1 until the confidence level according to Annex D is achieved.

##### 14.2.1.4.3	Message contents

Table 14.2.1.4.3-1: RESET UE POSITIONING STORED INFORMATION

| Derivation Path: 38.509 [44] clause 6.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE Positioning Technology | 0 0 0 0 0 1 1 1 | DL-TDOA |  |

Table 14.2.1.4.3-2: LPP Request Capabilities

| Information Element | Value/remark |
| --- | --- |
| nr-DL-TDOA-RequestCapabilities-r16 | TRUE |

Table 14.2.1.4.3-3: LPP RequestLocationInformation

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | 1 |  |  |
| } |  |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsRequestLocationInformation SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsPreferred |  |  |
| triggeredReporting | Not present |  |  |
| periodicalReporting | Not present |  |  |
| additionalInformation | onlyReturnInformationRequested |  |  |
| qos SEQUENCE { |  |  |  |
| horizontalAccuracy | Not present |  |  |
| verticalCoordinateRequest | FALSE |  |  |
| verticalAccuracy | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |
| time | See 14.2.1.5 | Result of the response time calculation rounded up to the next second if response time <= 128s. Result of the response time calculation rounded up to the next multiple of ten seconds if response time > 128s |  |
| responseTimeEarlyFix-r12 | Not present |  | Rel-12 onwards |
| unit-r15 | Not present |  |  |
|  | ten-seconds |  | Calculated response time >128s |
| } |  |  |  |
| velocityRequest | FALSE |  |  |
| } |  |  |  |
| environment | Not present |  |  |
| locationCoordinateTypes | Not present |  |  |
| velocityTypes | Not present |  |  |
| } |  |  |  |
| a-gnss-RequestLocationInformation | Not present |  |  |
| otdoa-RequestLocationInformation | Not present |  |  |
| ecid-RequestLocationInformation | Not present |  |  |
| epdu-RequestLocationInformation | Not Present |  |  |
| sensor-RequestLocationInformation-r13 | Not present |  |  |
| tbs-RequestLocationInformation-r13 | Not present |  |  |
| wlan-RequestLocationInformation-r13 | Not present |  |  |
| bt-RequestLocationInformation-r13 | Not present |  |  |
| nr-ECID-RequestLocationInformation-r16 | Not present |  |  |
| nr-Multi-RTT-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-AoD-RequestLocationInformation-r16 | Not present |  |  |
| nr-DL-TDOA-RequestLocationInformation-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-RstdMeasurementInfoRequest-r16 | Not present |  |  |
| nr-RequestedMeasurements-r16 | bit 0 = 0 (prsrsrpReq) |  |  |
| nr-AssistanceAvailability-r16 | FALSE |  |  |
| nr-DL-TDOA-ReportConfig-r16 | Not present |  |  |
| additionalPaths-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.4.3-4: LPP ProvideAssistanceData

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData | Not present |  |  |
| otdoa-ProvideAssistanceData | Not present |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 | Not present |  |  |
| tbs-ProvideAssistanceData-r14 | Not present |  |  |
| wlan-ProvideAssistanceData-r14 | Not present |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 | Not present |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  |  |  |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 14.2.1.4.3-5 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 SEQUENCE { |  |  | Depending on UE capabilities, i.e. support for UE-based DL-TDOA |
| nr-TRP-LocationInfo-r16 | As defined in TS 37.571-5 [20] |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.4.3-5: NR-DL-PRS-AssistanceData

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | kHz15 |  | Config 1 and Config 2 |
|  | kHz30 |  | Config 3 |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | 0 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 16 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 | Cell 1 |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Cell 1 |  |  |
| nr-CellGlobalID-r16 | Cell 1 |  |  |
| nr-ARFCN-r16 | Cell 1 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 0 |  |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | Cell 2 |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Cell 2 |  |  |
| nr-CellGlobalID-r16 | Cell 2 |  |  |
| nr-ARFCN-r16 | Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 | 0 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 38 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | Cell 3 |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Cell 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | Cell 3 |  |  |
| mnc-r15 | Cell 3 |  |  |
| nr-cellidentity-r15 | Cell 3 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | Cell 3 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 23 | About 3 s |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 38 | About 5 s |  |
| nr-DL-PRS-Info-r16 | As specified in Table 14.2.1.4.3-6 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[4-16] | As specificied in TS 37.571-5 [20] |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.1.4.3-6: NR-DL-PRS-Info

| Derivation Path: TS 37.355 [49] clause 6.4.3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { |  | 1 entry |  |  |  |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |  |  |  |  |  |
| scs15-r16 CHOICE { |  |  |  |  |  | Config 1 and Config 2 |  |
| n160-r16 |  | 10 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| scs30-r16 CHOICE { |  |  |  |  |  | Config 3 |  |
| n160-r16 |  | 10 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 |  | n2 |  |  |  |  |  |
| dl-PRS-ResourceTimeGap-r16 |  | s1 |  |  |  |  |  |
| dl-PRS-NumSymbols-r16 |  | n4 |  |  |  |  |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |  |  |  |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 |  | Not present |  |  |  |  |  |
| nr-option1-muting-r16 CHOICE { |  |  |  |  |  |  |  |
| po2-r16 |  | 10 |  |  |  | Cell 1 and Cell 3 |  |
|  |  | 01 |  |  |  | Cell 2 |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| dl-PRS-MutingOption2-r16 |  | Not present |  |  |  |  |  |
| dl-PRS-ResourcePower-r16 |  | 27 |  |  |  |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { |  | 1 entry |  |  |  |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  |  |  | entry 1 |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-SequenceID-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |  |  |  |  |
| n2-r16 |  | 0 |  |  |  | Cell 1 and Cell 3 |  |
|  |  | 1 |  |  |  | Cell 2 |  |
| } |  |  |  |  |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-ResourceSymbolOffset-r16 |  | 0 |  |  |  |  |  |
| dl-PRS-QCL-Info-r16 |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 14.2.1.4.3-7: LPP ProvideLocation Information

| Derivation Path: TS 37.355 [49] clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | 1 |  |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideLocationInformation |  | Not present. |  |  |
| a-gnss-ProvideLocationInformation |  | Not present |  |  |
| otdoa-ProvideLocationInformation |  | Not present |  |  |
| ecid-ProvideLocationInformation |  | Not present |  |  |
| epdu-ProvideLocationInformation |  | Not present |  |  |
| sensor-ProvideLocationInformation-r13 |  | Not present |  |  |
| tbs-ProvideLocationInformation-r13 |  | Not present |  |  |
| wlan-ProvideLocationInformation-r13 |  | Not present |  |  |
| bt-ProvideLocationInformation-r13 |  | Not present |  |  |
| nr-ECID-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-Multi-RTT-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-DL-AoD-ProvideLocationInformation-r16 |  | Not present |  |  |
| nr-DL-TDOA-ProvideLocationInformation-r16 SEQUENCE { |  |  |  |  |
| nr-DL-TDOA-SignalMeasurementInformation-r16 SEQUENCE { |  |  |  |  |
| dl-PRS-ReferenceInfo-r16 |  |  |  |  |
| nr-DL-TDOA-MeasList-r16 SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16 { |  | 3 entries |  |  |
| NR-DL-TDOA-MeasElement-r16[1] SEQUENCE { |  |  | entry 1 |  |
| dl-PRS-ID-r16 |  |  |  |  |
| nr-PhysCellID-r16 |  | Cell 1 |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-RSTD-r16 |  | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |  |
| } |  |  |  |  |
| NR-DL-TDOA-MeasElement-r16[2] SEQUENCE { |  |  | entry 2 |  |
| dl-PRS-ID-r16 |  |  |  |  |
| nr-PhysCellID-r16 |  | Cell 2 |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-RSTD-r16 |  | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |  |
| } |  |  |  |  |
| NR-DL-TDOA-MeasElement-r16[3] SEQUENCE { |  |  | entry 3 |  |
| dl-PRS-ID-r16 |  |  |  |  |
| nr-PhysCellID-r16 |  | Cell 3 |  |  |
| nr-CellGlobalID-r16 |  |  |  |  |
| nr-ARFCN-r16 |  |  |  |  |
| nr-DL-PRS-ResourceID-r16 |  |  |  |  |
| nr-DL-PRS-ResourceSetID-r16 |  |  |  |  |
| nr-TimeStamp-r16 |  |  |  |  |
| nr-RSTD-r16 |  | Present |  |  |
| nr-AdditionalPathList-r16 |  |  |  |  |
| nr-TimingQuality-r16 |  |  |  |  |
| nr-DL-PRS-RSRP-Result-r16 |  |  |  |  |
| nr-DL-TDOA-AdditionalMeasurements-r16 |  |  |  |  |
| } |  |  |  |  |
| nr-dl-tdoa-LocationInformation-r16 |  |  |  |  |
| nr-DL-TDOA-Error-r16 |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 14.2.1.5	Test requirement

Table 14.2.1.5-1, Table 14.2.1.5-2 and Table 14.2.1.5-3 define the primary level settings including test tolerances for the test.

Table 14.2.1.5-1: General test parameters for RSTD measurement reporting delay

| Parameter |  |  | Unit |  | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference cell |  |  |  |  | Cell 1 |  | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [6] and TS 37.355 [49]. The reference cell is the PCell in this test case. |  |
| Neighbor cells |  |  |  |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the DL-TDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |  |
| SSB configuration |  | Config 1 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 2 |  |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.2 |  |  |  |
|  |  | Config 2 |  |  | SMTC.1 |  |  |  |
|  |  | Config 3 |  |  | SMTC.1 |  |  |  |
| PDSCH RMC configuration |  | Config 1 |  |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | Config 1 |  |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  |  | CCR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |  |  |
| Initial BWP configuration |  | Config 1,2,3 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Active DL BWP configuration |  | Config 1,2,3 |  |  | DLBWP.1.1 |  |  |  |
| Active UL BWP configuration |  | Config 1,2,3 |  |  | ULBWP.1.1 |  |  |  |
| PRS Configuration |  | Config 1 |  |  | PRS.1.1 FR1 |  |  |  |
|  |  | Config 2 |  |  | PRS.1.1 FR1 |  |  |  |
|  |  | Config 3 |  |  | PRS.2.1 FR1 |  |  |  |
| Physical cell ID PCI |  |  |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |  |
| CP length |  |  |  |  | Normal |  |  |  |
| DRX |  |  |  |  | OFF |  |  |  |
| Measurement gap |  |  |  |  | GP#24 or GP#0 |  | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  |  | s |  | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 |  | PRS are transmitted from synchronous cells |  |
| Expected RSTD |  |  | s |  | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355 [49] is the expectedRSTD indicator |  |
| Expected RSTD uncertainty for all neighbour cells |  |  | s |  | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 37.355 [49] is the expectedRSTD-Uncertainty index |  |
| Number of cells provided in OTDOA assistance data |  |  |  |  | 16 |  | Including the reference cell |  |
| PRS muting info |  |  |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ |  | Corresponds to prs-MutingInfo defined in TS 37.355 [49] Cell 1 and Cell 3 will be configured with different Comb patterns or resource offsets |  |
| PRS resource RE offset |  |  |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 |  | Cell 1 and Cell 3 are configured with different resource offsets |  |
| T1 |  |  | s |  | 3 |  | The length of the time interval from the beginning of each test |  |
| T2 |  |  | s |  | 1.28 |  | The length of the time interval that follows immediately after time interval T1 |  |

Table 14.2.1.5-2: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positioning frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PSS to SSS |  | dB | 0 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -59.63 | -59.63 | -59.63 |
|  | Config 2 | dBm/ 9.36MHz | -59.63 | -59.63 | -59.63 |
|  | Config 3 | dBm/ 38.16MHz | -53.54 | -53.54 | -53.54 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -85 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table 14.2.1.5-3: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5 | -11.20 | -11.20 |
|  | Config 2 | dB | -5 | -11.20 | -11.20 |
|  | Config 3 | dB | -5 | -11.20 | -11.20 |
| SSB | Config 1~3 | dB | 10 | 3 | 3 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.38 | -68.38 | -68.38 |
|  | Config 2 | dBm/ 9.36MHz | -68.38 | -68.38 | -68.38 |
|  | Config 3 | dBm/ 38.16MHz | -62.28 | -62.28 | -62.28 |
| SSB_RP Note4 | Config 1 | dBm/SCS | -88 | -95 | -95 |
|  | Config 2 | dBm/SCS | -88 | -95 | -95 |
|  | Config 3 | dBm/SCS | -85 | -92 | -92 |
| PRS |  | dB | -5.61 | -12.64 | -12.64 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the slots with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. The Io is calculated based only on the symbols where PRS is transmitted. |  |  |  |  |  |

The RSTD measurement time fulfils the requirements specified in clause 14.2.1.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in clause 14.2.1.3 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in TS 38.133[50] Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049

The response time is equal to the LPP time IE value plus the test tolerance. The LPP time IE value is derived from the RSTD measurement period plus T, where T = 50 ms. The RSTD measurement period follows the equation:

TRSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where CSSFPRS,i=1,NRxBeam,i = 1, NPRS,islot=16, Lavailable_PRS,i=1,Nsample= 4. N is the parameter durationOfPRS-ProcessingSysmbols from TS 37.355 [49], N’ is the parameter maxNumOfDL-PRS-ResProcessedPerSlot from TS 37.355 [49], Tlast,i = Ti + Tavailable_PRS,i and Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i.

Tavailable_PRS,i= LCMTPRS,i,MGRPi ; where Tprs = 160 ms, and MGRP is 80 (for GP#24) or 40 (for GP#0) depending on UE capabilities. Therefore, Tavailable_PRS,i=160 ms.

Ti depends on the UE parameter durationOfPRS-ProcessingSymbolsInEveryTms from TS 37.355 [49]

Finally, it results in the following equation:

16N'1N*4-1*Teffect+Tlast

Where the remaining parameters depend on the UE capabilities. The LPP time IE ranges between 0.698s and 655.57s. The value of the LPP time IE is rounded up to the next second (if the value is >128s, it should be rounded up to the next multiple of ten seconds). The result is transmitted in the response time IE in the LPP-RequestLocationInformation in Table 14.2.1.4.3-3. The LPP time IE ranges between 1s and 660s.

The test tolerance for the response time is 300ms. Therefore, the response time ranges between 1.3s and 660.3s.

The values of N’, N and Ti and the effect in the response time equation are defined in Table 14.2.1.5-4, Table 14.2.1.5-5 and Table 14.2.1.5-6 for reference.

Table 14.2.1.5-4: value of N’ based on maxNumOfDL-PRS-ResProcessedPerSlot

| maxNumOfDL-PRS-ResProcessedPerSlot | 16N' |
| --- | --- |
| n1 | 16 |
| n2 | 8 |
| n4 | 4 |
| n8 | 2 |
| >=n16 | 1 |

Table 14.2.1.5-5: value of N based on durationOfPRS-ProcessingSysmbols

| durationOfPRS-ProcessingSysmbols | 1N |
| --- | --- |
| nDot125 | 8 |
| nDot25 | 4 |
| nDot5 | 2 |
| >=n1 | 1 |

Table 14.2.1.5-6: value of Teffect and Tlast based on durationOfPRS-ProcessingSymbolsInEveryTms

| durationOfPRS-ProcessingSymbolsInEveryTms | Teffect | Tlast |
| --- | --- | --- |
| n8 | 160 | 168 |
| n16 | 160 | 176 |
| n20 | 160 | 180 |
| n30 | 160 | 190 |
| n40 | 160 | 200 |
| n80 | 160 | 240 |
| n160 | 160 | 320 |
| n320 | 320 | 480 |
| n640 | 640 | 800 |
| n1280 | 1280 | 1440 |

TRSTD,Total=i=1LTRSTD,i+ L-1*maxTeffect,i

The test tolerances are defined in clauses C.1.6, C 2.5 and C4.

The rate of successful tests during repeated tests shall be at least 90% with a confidence level of 95%.

< End of changes >