3GPP TSG-RAN WG5 Meeting #97                                                 	R5-228051

# Toulouse, France, 14 – 18 November 2022

Agenda Item:	5.3.27.10

Source: 	Ericsson, Nokia

Title: 	Discussion on RedCap UE testability in SUL test cases

Document for:	Discussion and Endorsement

# Introduction

During post #96-e meeting discussions, it was proposed to make SUL test cases optional for a RedCap UE. To further understand the feasibility of this approach, test case analysis with focus on reference sensitivity has been carried out and presented in this paper.

# Discussion

As per 38.101-1 [2]:

An interpretation of 7.1I in [2] could be that REFSENS from 7.3I may also apply to 7.3C, reference sensitivity for SUL. The content and analysis of this paper is based on this interpretation, to find out if this is testable.

In 7.3I.2 [2], the REFSENS values comes with side conditions, for example a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3. As per 7.1I, it is not specified whether the REFSENS side conditions in 7.3I applies along with the REFSENS values.

Observation 1: As per 7.1I, it is not specified whether the REFSENS side conditions in 7.3I applies along with the REFSENS values.

Proposal 1: Ask RAN4 to clarify if REFSENS side conditions in 7.3I applies when testing the requirements in section 7 for a RedCAp UE.

As per 7.3I.2 in [2] for a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2-1 for the applicable operating bands. Table 7.3I.2-1 specifies ΔR1R for TDD bands and FDD bands but not the combination of FDD/TDD and SUL band. In other words, the sensitivity allowance is unknow for a 1 Rx RedCap UE for any operating band combination for SUL.

Observation 2: Sensitivity allowance is unknow for 1 Rx RedCap UE for any operating band combination for SUL.

Proposal 2: Ask RAN4 for clarification of applicability of Table 7.3I.2-1 for SUL band combination.

As per snippet from 5.5C in 38.101-1[2]:

Table 5.5C-1: Supported channel bandwidths per SUL band combination

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n24A-n99A | n24 | 5, 10 | 0 |
|  | n99 | 5, 10 |  |

As per 5.2C in 38.101-1 [2]:

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n24-n992 | n24, n99 |
| SUL_n41-n802 | n41, n80 |
| SUL_n41-n812 | n41, n81 |
| SUL_n41-n832 | n41, n83 |
| SUL_n41-n952 | n41, n95 |
| SUL_n41-n972 | n41, n97 |
| SUL_n41-n982 | n41, n98 |
| SUL_n41-n992 | n41, n99 |
| SUL_n48-n992 | n48, n99 |
| SUL_n77-n802 | n77, n80 |
| SUL_n77-n842 | n77, n84 |
| SUL_n77-n992 | n77, n99 |
| SUL_n78-n802 | n78, n80 |
| SUL_n78-n812 | n78, n81 |
| SUL_n78-n822 | n78, n82 |
| SUL_n78-n832 | n78, n83 |
| SUL_n78-n842 | n78, n84 |
| SUL_n78-n862 | n78, n86 |
| SUL_n79-n802 | n79, n80 |
| SUL_n79-n812 | n79, n81 |
| SUL_n79-n832 | n79, n83 |
| SUL_n79-n842 | n79, n84 |
| SUL_n79-n952 | n79, n95 |
| SUL_n79-n972 | n79, n97 |
| SUL_n79-n982 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

Taking an example of band combination SUL_n24-n99. As per NOTE 2 in Table 5.2C-1 [2] above, simultaneous Rx/Tx capability is mandatory for this band combination. A RedCap UE could be of type HD-FDD [1], i.e. no support of simultaneous Rx/Tx.

Observation 3: No SUL band in combination with FDD band is applicable for a RedCap HD-FDD UE. A HD-FDD RedCap UE is untestable for SUL band in combination with FDD band.

Proposal 3: Ask RAN4 for clarification on the applicability of Table 5.2C-1 in [2] for RedCap UE.

As per 7.3I.2 in 38.101-1 [2]:

As can be seen in the table above, only REFSENS values for FDD and TDD operating bands are defined for RedCap UE with 1 Rx antenna port. REFSENS for any SUL band combination, for example SUL_n24_n99, is missing and the reference sensitivity requirement is unknown for RedCap UE in SUL band combination. The same applies for a RedCap UE with 2 Rx antenna port as per table 7.3I.2-2 in [2].

Observation 4: REFSENS is unspecified for a RedCap UE in SUL band combination.

Proposal 4: Ask RAN4 for clarification of REFSENS requirement for RedCap UE in SUL band combination.

# 3Conclusion

Observation 1: As per 7.1I, it is not specified whether the REFSENS side conditions in 7.3I applies along with the REFSENS values.

Proposal 1: Ask RAN4 to clarify if REFSENS side conditions in 7.3I in [2] applies when testing the requirements in section 7 for a RedCap UE.

Observation 2: Sensitivity allowance is unknow for 1 Rx RedCap UE for any operating band combination for SUL.

Proposal 2: Ask RAN4 for clarification of applicability of Table 7.3I.2-1 in [2] for SUL band combination.

Observation 3: No SUL band in combination with FDD band is applicable for a RedCap HD-FDD UE. A HD-FDD RedCap UE is untestable for SUL band in combination with FDD band.

Proposal 3: Ask RAN4 for clarification on the applicability of Table 5.2C-1 in [2] for RedCap UE.

Observation 4: REFSENS is unspecified for a RedCap UE in SUL band combination.

Proposal 4: Ask RAN4 for clarification of REFSENS requirement for RedCap UE in SUL band combination.

# 4References

[1]		R1-2110773	UE features for RedCap, Ericsson

[2]	     38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone    V17.6.0