3GPP TSG-RAN5 Meeting #97	R5-226351r1

# Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0387 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for R15 SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of new PICS for R15 SON_MDT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of CGI specific PICS pc_eutra_CGI_Reporting, pc_nr_CGI_Reporting, pc_nr_CGI_Reporting_ENDC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be correctly applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Modify the format of pc_nr_CGI_Reporting_NRDC and the item number |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| 2 | Support two independent measurement gap configurations for FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_independentGapConfig | No |  |  |
| 3 | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting | 38.306, 4.2.9 | Rel-15 | pc_intraAndInterF_MeasAndReport | Yes |  |  |
| 4 | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH | 38.306, 4.2.9 | Rel-15 | pc_csi_RSRP_AndRSRQ_MeasWithSSB | No |  |  |
| 5 | Support inter-RAT E-UTRA measurements and events B triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventB_MeasAndReport | Yes |  |  |
| 6 | Support SS-SINR measurents | 38.306, 4.2.9 | Rel-15 | pc_ss_SINR_Meas | No |  |  |
| 7 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting | Yes |  |  |
| 8 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting | Yes |  |  |
| 9 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_ENDC | Yes |  |  |
| 10 | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. | 36.331, 6.3.6 | Rel-15 | pc_gp2_gp3_en_dc | No |  |  |
| 11 | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp4_en_dc | No |  |  |
| <<< UNCHANGED TEXT SKIPPED >>> |  |  |  |  |  |  |  |
| 55 | Support configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_idleInactiveEUTRA_MeasReport | No |  |  |
| 56 | Support SRS-RSRP measurements between a NR Pcell and an interfering UE, upon network request as specified in 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_nr_CLI_Reporting_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 If the UE doesn’t support CLI SRS-RSRP measurement, it will not include this IE in its capability report. |
| XX | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NEDC | No |  |  |
| XY | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NRDC | No |  |  |
| XZ | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NEDC | Yes |  |  |
| YY | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NRDC | Yes |  |  |

<< END OF CHANGES >>